| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0436 |
| --- | --- |

| Title: | Report of non-3V high-level syntax BoG |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Contact(s): | Miska M. Hannuksela Nokia | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Non-3V high-level syntax BoG |  |  |

_____________________________

# Abstract

The document contains a report of the non-3V high-level syntax break-out group (BoG).

# Reviewed Documents

## SEI messages

JCTVC-M0044 Layers display info SEI message [J. Boyce, D. Hong, W. Jang (Vidyo)]

The contribution appears in section 6.4.12 in the JCT-VC meeting notes.

A layers display info SEI message is proposed which contains display resolution information about the scalable layers in the bitstream which is useful to a decoder, but which is not present in the VPS.  The coded resolution and cropping parameters already present in the SPS for each layer are sufficient to derive the display resolution, but if a layer is removed by a middle box, the decoder may not be able to access the SPS associated with the removed layer.  The parameters proposed for inclusion in this SEI message are similar to parameters included in SVC’s scalability info SEI message.

It was commented that the temporal persistence was not clearly described in the slide deck. It was responded that the persistence at maximum a coded video sequence, or when the next SEI message of the same type appears.

It was asked why the signalling of resolutions could not reside in the VPS. A change in VPS requires sending IDR (across layers). The proponent envisioned that an active layer SPS could change the spatial resolution, hence the spatial resolution information might not be persistent throughout the coded video sequent. However, in the current SHVC draft IDR (and BLA) pictures are required to be aligned across layers and a CRA picture does not activate a new layer SPS. There are related contributions suggesting either that IDR/BLA pictures are allowed to be non-aligned, or having specific stepwise layer access pictures (STLA), or starting the decoding process stepwise per layer. The BoG suggested revisiting this contribution after having decisions on the related contributions.

It was asked why not to derive the information from SPSs. The proponent responded that an SPS would be sent in-band in their application and could be pruned along with coded data of a layer.

## Signalling of cropped inter-layer reference

The BoG recommended having a discussion in the plenary or track meeting whether other scaling factors than 1.5 and 2 are to be supported in SHVC.

The BoG recommended having a discussion in the plenary or track meeting whether region-of-interest scalability is to be supported in SHVC.

The BoG agreed that the use case where different layers have different picture aspect ratios is relevant. It was noted that some of the following contributions can be used to respond to this need.

The following contributions appear in section 6.4.6 in the JCT-VC meeting notes.

JCTVC-M0219 Reference-layer cropping offsets signaling in SHVC [D.-K. Kwon, M. Budagavi (TI)]

Following the same approach in SVC, the current SHVC design allows to select enhancement layer (EL) region to use inter-layer prediction, but does not allow to select reference layer (RL) region to be re-sampled for inter-layer prediction. In this contribution, it is proposed to signal reference-layer cropping offsets in SPS and to select RL region to be resampled for inter-layer prediction based on these offsets.

It was asked if the approach of this contribution can be used for legacy-codec base layer. The proponent said that this approach can be used. It was clarified that the offsets are relative to the coded picture (not the output picture after conformance cropping). It was commented that for supporting legacy-codec base layer, the proposed offsets should be relative to the output picture. These aspects might need offline clarification.

It was clarified that the scaling factor for upsampling is derived on the basis of the indicated cropping offsets.

It was commented that this contribution could be used together with scaled reference offsets (which are e.g. proposed in JCTVC-M0309). The BoG recommended to include scaled reference offset in JCTVC-M0219 (to see how the proposal and scaled reference offsets are applied together) and then revisit the contribution.

JCTVC-M0309 Signaling of extended spatial scalability for SHVC [A. K. Ramasubramonian, X. Li, Y. Chen, V. Seregin (Qualcomm)]

This document proposes signaling of the regions of base layer and enhancement layer involved in spatial scalability. Similar to SVC, the offsets of the upsampled base layer picture boundary with respect to the enhancement layer picture boundary are signalled for each reference layer of the current enhancement layer.

It was asked why the signalling is proposed in VPS and not in SPS. VPS does not at the moment contain spatial resolution information. The proponent preferred to have all signalling related to multiple layers in the VPS. However, the proposed mechanism would work equally with the proposed syntax elements included in SPS.

It was clarified that different scaled reference offsets for a layer could be needed depending on which layer uses it as reference. Layer dependency signalling appears in VPS. Hence, if scaled reference offsets were included in SPS, there would be a parsing dependency. A way around this parsing dependency would be to signal the number of inter-layer references in SPS (but not indicate which layers exactly are these reference layers).

It was commented that the proposal is identical to the corresponding method in SVC except the syntax structure where the signalling appears.

It was asked whether it is proposed to apply the proposed method before or after conformance cropping. The proponent responded that it is an orthogonal issue and applying either before or after conformance cropping can be done with this contribution.

It was commented that the proposed specification text should be improved. It was requested to indicate clearly the differences compared to the test model (JCTVC-L1007).

The BoG recommended revisiting the contribution after the specification text is provided.

JCTVC-M0274 AHG9: On conformance-cropping-window-restricted inter-layer prediction [Y.-K. Wang, V. Seregin, J. Chen, X. Li, Y. Chen (Qualcomm)]

Currently, inter-layer prediction (ILP) in SHVC is restricted to come from within the conformance cropping window of an inter-layer reference picture (ILRP), regardless of whether the reference layer is HEVC or AVC conforming. This document advocates that this restriction should be removed such that ILP can come from any area in the entire decoded picture of an ILRP, regardless of whether the reference layer is HEVC or AVC conforming, at least for the cases wherein entire decoded pictures of the reference layer are accessible for ILP purposes.

Either of the following two options is proposed:

- To completely remove the restriction on ILP from within the conformance cropping window, such that ILP can come from any area in the entire decoded picture of an ILRP, regardless of whether the reference layer is HEVC or AVC conforming.

- To remove the restriction when the base layer is HEVC conforming, and to conditionally apply the restriction when the base layer is AVC conforming.

For option B, the syntax and semantics of video parameter set extension are proposed to be changed as follows:

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| if( avc_base_layer_flag ) |  |
| vps_ilp_within_conformance_cropping_window_flag | u(1) |
| ... | u(1) |
| } |  |

vps_ilp_within_conformance_cropping_window_flag equal to 1 specifies that, within the CVS, there is no inter-layer prediction, for any syntax elements, variables or sample values used in the decoding process, from information outside the conformance cropping window of a reference picture with nuh_layer_id equal to 0. vps_ilp_within_conformance_cropping_window_flag equal to 0 specifies that such a limit is not applied. When not present, the value of vps_ilp_within_conformance_cropping_window_flag is inferred to be equal to 0.

It was requested to provide specification text indicating the differences compared to the test model (JCTVC-L1007), particularly when it comes to the decoding process.

It was commented that with SNR scalability in the current SHVC draft, padding is required (when a base layer picture is used as reference for enhancement layer) if the conformance cropping window is smaller than the decoded picture size. It was commented that in textureRL the complexity could be smaller, if it the encoder is disallowed to apply inter-layer prediction from blocks where padding would be required.

Some implementations pad pixels outside pixel boundaries for motion compensation purposes. Hence, if padding would not be required for conformance cropping window for upsampling purposes, then no additional padding (compared to padding done anyways for motion compensation) would be required in some implementations.

It was remarked that JCTVC-M0180 is similar in some aspects.

The BoG recommended revisiting the contribution after the specification text is provided.

## Parameter sets

The following contributions appear in section 6.4.3 in the JCT-VC meeting notes.

The contributions were not reviewed yet in the BoG.

JCTVC-M0153 Extension parameter set [Y. Cho, B. Choi, M. W. Park, J. Y. Lee, J. Yoon, H. Wey, C. Kim (Samsung)]

JCTVC-M0179 AHG9: APS for inter-layer processing signaling [Y He, J Dong, Y Ye (??)]

JCTVC-M0310 Cross-check of JCTVC-M0179 on APS for inter-layer processing signaling [H. Yang (Huawei)] [miss]