| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0438 |
| --- | --- |

| Title: | RCE2: Cross-check of JCTVC-M0052 (Table 6) and JCTVC-M0193 (Table 1) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Haoping Yu Jing Ye Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95050 USA | Email: Tel: | haoping.yu@huawei.com +1 408 330 4904 |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

This contribution reports the results of a cross-check performed by Huawei on part of JCTVC-M0052 (Table 6) and part of JCTVC-M0193 (Table 1). The verification tests were carried out using the test sequences and conditions specified in JCTVC-L1122. Our test results have a perfect match, in terms of bitrates, with those presented in the contribution documents.

# Description

The basic idea of JCTVC-M0052 and JCTVC-M0193 is the same, which is to generate pixel-wise prediction by using a template based sample-adaptive weighted prediction (SWP).

The difference between the two methods under test is that Table 6 of JCTVC-M0052 represents the results when SWP only replace the PLANAR mode, and on the other hand, Table 1 of JCTVC-M0193 shows the results when SWP replace both PLANAR and DC modes.

# Results

The test conditions for RCE2 as specified in JCTVC-L1122 were used in the verification tests. The simulations were run on a 64-bit Windows cluster. Our results have a perfect match with those reported by the proponents. The complete cross-check results are provided in the accompanying spread-sheets. Below shows the summary of the test results.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.2 | -9.1% |
| Class B | 2.1 | 2.3 | -6.8% |
| SC(GBR) | 7.5 | 8.8 | -12.1% |
| RangeExt | 2.2 | 2.3 | -4.8% |
| Overall (w/o SC) | 3.2 | 3.5 | -6.9% |
| Overall (w/ SC) | 5.3 | 6.2 | -9.5% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 31.3 | -4.5% |
| Class B | 2.6 | 2.6 | -1.8% |
| SC(GBR) | 88.8 | 103.0 | -8.9% |
| RangeExt | 2.5 | 2.6 | -1.2% |
| Overall (w/o SC) | 13.3 | 14.1 | -2.7% |
| Overall (w/ SC) | 51.0 | 58.5 | -5.8% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 49.2 | -3.3% |
| Class B | 2.6 | 2.6 | -1.5% |
| SC(GBR) | 301.9 | 349.5 | -8.3% |
| RangeExt | 2.5 | 2.6 | -1.1% |
| Overall (w/o SC) | 20.5 | 21.2 | -2.1% |
| Overall (w/ SC) | 161.2 | 185.4 | -5.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 1: Bit-savings by only replacing PLANAR with respect to lossless coding anchor

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 4.6 | 5.3 | -11.2% |
| Class B | 2.1 | 2.3 | -6.8% |
| SC(GBR) | 7.5 | 9.0 | -14.3% |
| RangeExt | 2.2 | 2.3 | -4.4% |
| Overall (w/o SC) | 3.2 | 3.5 | -7.6% |
| Overall (w/ SC) | 5.3 | 6.3 | -11.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 29.3 | 31.9 | -6.3% |
| Class B | 2.6 | 2.6 | -1.8% |
| SC(GBR) | 88.8 | 104.7 | -10.4% |
| RangeExt | 2.5 | 2.6 | -1.1% |
| Overall (w/o SC) | 13.3 | 14.3 | -3.3% |
| Overall (w/ SC) | 51.0 | 59.5 | -6.9% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 47.4 | 50.0 | -5.0% |
| Class B | 2.6 | 2.6 | -1.4% |
| SC(GBR) | 301.9 | 354.5 | -9.6% |
| RangeExt | 2.5 | 2.6 | -1.0% |
| Overall (w/o SC) | 20.5 | 21.5 | -2.7% |
| Overall (w/ SC) | 161.2 | 188.0 | -6.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2: Bit-savings by replacing both PLANAR and DC with respect to lossless coding anchor

# Conclusions

Our verification test results confirmed the reported coding efficiency gain shown in Table 6 of JCTVC-M0052 and in Table 1 of JCTVC-M0193.