| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0439_r1 |
| --- | --- |

| Title: | Complexity reduction for residual DPCM in HEVC lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Andrea Gabriellini Centre House, Block D, 56 Wood Lane, W12 7SB, London (UK) |  | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

The residual DPCM for HEVC lossless coding in intra predicted blocks has been proposed in documents JCTVC-L0117 and JCTVC-M0079. Residual DPCM exploits the spatial correlation among residuals belonging to the same block and is applied only along the horizontal and vertical direction when the selected intra prediction direction is one of these two. Since residual DPCM is applied independently over each row or column, it allows parallelisation of inverse DPCM at the decoder. This contribution proposes a complexity reduction for residual DPCM. The main idea is to limit the dependency among the samples involved during the DPCM prediction. The complexity reduction is achieved by performing the residual DPCM within chunks of samples of a given length which is proposed to be 8. It is reported that experimental results over the RCE2 test set show that the compression ratios of the proposed method are comparable to the ones achieved by the original residual DPCM. In particular, it is reported that average compression ratio increase of 9.1% and 11.0% are achieved for Class F and Screen Content (RGB), respectively, for all intra configuration.

# Residual DPCM in HEVC lossless coding

The residual DPCM process is applied when the cu_transquant_bypass_flag is equal to 1, the block prediction mode is intra and the prediction direction is either horizontal or vertical. The residual DPCM exploits the spatial correlation among the residuals inside the block. Schematically, it can be represented as in Figure 1 for the horizontal and vertical predictions.

| Horizontal residual DPCM | Vertical residual DPCM |
| --- | --- |

Figure 1: DPCM options for blocks of residuals.

As may be noted from the previous figure, both at the encoder and decoder the residual DPCM can be parallelised by running a different residual DPCM for each row or column.

The application of the residual DPCM leads to the new block of residuals where samples are denoted as r’ and is obtained from the block of residuals r as follows:

r'hor(i,j)=r(i,j)if j=0r(i,j)−r(i,j−1)otherwise,

r'ver(i,j)=r(i,j)if i=0r(i,j)−r(i−1,j)otherwise,

where the block consists of M rows and N columns, i = 0,…, M - 1, j = 0,..., N – 1, and r’hor and r’ver denote the residual DPCM obtained when the prediction direction of the block is horizontal and vertical, respectively. At the decoder side the reconstruction is simply obtained as follows:

rhor(i,j)=k=0jr'hor(i,k)   ,

rver(i,j)=k=0ir'ver(k,j)   .

From the previous formulas it can be seen that for inverse DPCM, each sample reconstruction depends on the previous samples in that row or column. As an example, if the residual DPCM is applied along the horizontal direction, to obtain the sample a position (0, N - 1), the samples at position (0, N - 1), (0, N - 2), ..., (0, 0) have to be added up. For large block sizes this dependency may be undesirable.

# Proposed residual DPCM with chunking

In order to limit the prediction dependencies at the decoder side, the residual samples are subdivided in chunks of a predetermined length R in the direction of prediction, so that no residual to be reconstructed requires additions of more than R values. After the prediction of (R - 1) consecutive samples in a row of column, the DPCM prediction chain is halted: the Rth sample is not predicted from the previous samples but instead is directly signalled in the bitstream. The Rth sample is the start of the second chunk of residual samples. This subdivision of the residual block is introduced to limit the number of samples required to fully reconstruct a sample at the decoder side. In this proposal, the chunk length is set to 8. DPCM is now defined as:

r'hor(i,j)=r(i,j)if i=0,R,2⋅R,...r(i,j)−r(i,j−1)otherwise,

r'ver(i,j)=r(i,j)if j=0,R,2⋅R,...r(i,j)−r(i−1,j)otherwise.

At the decoder side, the reconstruction is done according to the following:

rhor(i,j)=k=jR⋅Rjr'hor(i,k)   ,

rver(i,j)=k=iR⋅Rir'ver(k,j).

where  denotes the floor operator which returns the integer less than or equal to its argument. An example of how the proposed residual DPCM chunking is applied is represented in Figure 2.

Figure 2: DPCM with reduced dependencies for lower complexity decoding.

# Test results

The proposed residual DPCM with chunking is implemented on top of HEVC range extension software HM10.0_RExt2.0, and its coding gain is evaluated under the common test condition for RCE2 specified in [1]. Table 1 show the bitrate savings for the residual DPCM as proposed in JCTVC-M0079 [3] while Table 2 show the bitrate savings for the proposed chunking on residual DPCM. Compared to the data in Table 1, it may be noted that the overall bitrate savings are not influenced by the proposed chunking.

Table 1: Bitrate savings for original residual DPCM as proposed in JCTVC-M0079 [3].

| Config. Class | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class F | -9.3% | -5.7% | -4.6% |
| Class B | -4.5% | -0.9% | -0.6% |
| SC (GBR) | -11.1% | -7.7% | -7.1% |
| RangeExt | -2.8% | -0.7% | -0.6% |
| Overall (w/o SC) | -5.7% | -2.7% | -2.2% |
| Overall (w/ SC) | -8.4% | -5.2% | -4.6% |
| Enc Time[%] | 101% | 100% | 100% |
| Dec Time[%] | 98% | 102% | 98% |

Table 2: Bitrate savings for the proposed chunking on residual DPCM.

| Config. Class | All Intra | Random Access | Low Delay B |
| --- | --- | --- | --- |
|  | Bit-rate saving | Bit-rate saving | Bit-rate saving |
| Class F | -9.1% | -5.6% | -4.5% |
| Class B | -4.5% | -0.9% | -0.6% |
| SC (GBR) | -11.0% | -7.6% | -7.3% |
| RangeExt | -2.8% | -0.7% | -0.6% |
| Overall (w/o SC) | -5.7% | -2.7% | -2.2% |
| Overall (w/ SC) | -8.4% | -5.2% | -4.7% |
| Enc Time[%] | 102% | 104% | 103% |
| Dec Time[%] | 99% | 106% | 106% |

# Draft text changes

This section presents the proposed text changes to the current working draft for range extensions [2]. The changes are highlighted in yellow and are relative to the text changes required by the residual DPCM as proposed in [3].

8.6.6 Intra residual transform-bypass decoding process

This process is invoked when cu_transquant_bypass_flag is equal to 1, pred_mode_flag is equal to 1, and the applicable intra prediction mode is equal to either vertical or horizontal mode. The process for the Cb and Cr components is applied in the same way as for the luma component.

Inputs to this process are:

- a variable nT specifying the transform block size

- a variable predModeIntra specifying the intra prediction mode

- an (nT)x(nT) array r with elements r[ x ][ y ] which is either an array relating to a residual transform-bypass block of the luma component or an array relating to a residual transform-bypass block of the Cb and Cr component.

Output of this process is a modified version of the (nT)x(nT) array r with elements r[ x ][ y ] containing the result of the intra residual transform-bypass decoding process.

Let f be a temporary (nT)x(nT) array with element f[ x ][ y ], which are derived by;

f[ x ][ y ] = r[ x ][ y ]   with x=0..nT-1 and y=0..nT-1

Depending on predModeIntra, the values of the residual samples r[x][y], with x=0..nT-1 and y=0..nT-1, are derived as follows:

- If predModeIntra is equal to 26 (Intra_Vertical),

- residual sample r[x][y], is derived as follows:

r[x][y]=k=0y%8f[x][y-k]

- Otherwise (predModeIntra is equal to 10 (Intra_Horizontal)),

- residual sample r[x][y], is derived as follows:

r[x][y]=k=0x%8f[x-k][y]

# References

# W. Gao, M. Zhou, P. Amon, and S. Lee, “HEVC Range Extensions Core Experiment 2 (RCE2): Intra Prediction for Lossless Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L1122, Jan. 2013.

# D. Flynn, J. Sole and T. Suzuki, “HEVC range extension draft 2”, Document of Joint Collaborative Team on Video Coding, JCTVC-L1005, Jan. 2013.

# S. Lee, I.-K. Kim and C. Kim, “Residual DPCM for HEVC lossless coding”, Document of Joint Collaborative Team on Video Coding, JCTVC-M0079, April 2013.

# Patent rights declaration

The BBC R&D may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).