| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC- M0443 |
| --- | --- |

| Title: | Cross-check of JCTVC-M0255 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Pieter Kapsenberg | Tel: Email: | +1 480 552 5481 pieter.kapsenberg@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This contribution presents a cross-check for proposal JCTVC-M0255: Decoding a 10-bit HEVC sequence using an 8-bit decoder. The provided source code has been inspected and found to match the proposal, and the efficacy of the proposed method is confirmed. Questions are raised about conformance.

# Introduction

In [1], two methods of decoding a 10-bit sequence using 8-bit logic are discussed. Both methods aim to decode a visually acceptable 8-bit version of the coded 10-bit picture. With the reduced precision, error drift is expected. Method 1 simply adjusts the coefficient scaling and scales the SAO offsets, and the error drift results in poor quality pictures. Method 2 adds a rounding modification technique to inter- and intra-prediction, as well as the reconstruction process. This results in significantly increased picture quality.

# Results

The proposed method, Method 2, would be simple to implement in an 8-bit architecture, adding a way to obtain visually acceptable pictures as opposed to not being able to decode 10-bit sequences at all.

The source code modifications appear to match the proposal, and are not too invasive.

[1] suggests that the proposed technique be included in an Annex that would form an integral part of the HEVC standard. While the technique is certainly interesting and is a real solution to a real issue, it raises a few concerns:

- The drift-avoidance technique gets set in stone, not leaving room for improvements. Since the decoded picture is not expected to match the intended reconstruction anyway, this restriction seems undesirable.

- If the proposed text is amended to allow for other techniques, then it becomes difficult to say what is and isn’t a conforming output.

- The Picture Digest SEI message will obviously no longer apply. If the proposed technique were to be part of the standard, it seems that a matching digest SEI message would be desired. [1] does not address this.

# References

[1] D. Flynn, G. Martin-Cocher, D. He , “Decoding a 10-bit HEVC sequence using an 8-bit decoder”, JCTVC-M0255, Incheon, April 2013