| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0445 |
| --- | --- |

| Title: | Uni-prediction for combined inter mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Marta Karczewicz Do-Kyoung Kwon Madhukar Budagavi | Email: | vseregin@qti.qualcomm.com d-kwon@ti.com |
| Source: | Qualcomm Incorporated, Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution provides an analysis of two simplified solutions for combined prediction mode tested in SCE3.

# Introduction

It was noticed that combined mode from SCE3 [1] introduces bandwidth and computational complexity increase comparing to the HEVC worst case. Several contributions addressed this problem and reduced the complexity of the combined mode. Among those contributions, two approaches were identified the most simplest and further analysis of these two methods is provided in this contribution.

# Simplified combined prediction

Both methods [2, 3] provide complexity reduction for combined mode, achieving memory access and calculation complexity to be within HEVC worst case level.

Basic idea of this simplification is to restrict inter prediction, being part of the combined mode, to uni-directional inter prediction. The following table summarizes the details of each approach.

Table 1. Uni-prediction with combined mode.

|  | 8x8 CU 2NxN, Nx2N | Uni-prediction | Bi-prediction Identical motion | Bi-prediction Non identical motion | Chroma |
| --- | --- | --- | --- | --- | --- |
| JCTVC-M0220 | Not used | Combined prediction | Combined prediction | Bi-prediction | Combined prediction |
| JCTVC-M0297 | Not used | Combined prediction | For Merge mode, bi-directional MV is converted to uni-L0 MV. For AMVP mode, inter direction signaling is restricted to be uni-direction. |  | Not used |

In the next table, average BD rate reduction for luma and both chroma components among RA, LB, and LP configuration are summarized along with the average encoding and decoding running time.

Table 2. Experimental results against SHM1.0 IntraBL anchor.

|  | Luma BD rate | Chroma BD rate | Enc | Dec |
| --- | --- | --- | --- | --- |
| SCE3.2 | -0.92 | -4.35 | 113% | 100% |
| M0220 | -0.70 | 0.45 | 114% | 102% |
| M0297 | -0.79 | -3.45 | 106% | 99% |

Average memory accessed and calculation complexity for RA, LB, and LP tests cases are summarized in the next table.

Table 3. Average memory access and computational complexity

|  | Pure | DDR2 | DDR3 | Mults | Adds |
| --- | --- | --- | --- | --- | --- |
| SCE3.2 | 106% | 105% | 105% | 106% | 107% |
| M0220 | 104% | 104% | 104% | 104% | 105% |
| M0297 | 103% | 102% | 102% | 104% | 105% |

# References

[1] V. Seregin, J. Chen, and M. Karczewicz “SCE3: Combined inter mode (test 3.2)”, JCTVC-M0294, Incheon, KR, Apr. 2013.

[2] D.-K. Kwon, and M. Budagavi, “Non-SCE3: Uni-directional combined prediction (UCP) in Inter slice”, JCTVC-M0220, Incheon, KR, Apr. 2013.

[3] V. Seregin, M. Karczewicz “Non-SCE3: Bandwidth reduction for combined inter mode”, JCTVC-M0297, Incheon, KR, Apr. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).