| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0445 |
| --- | --- |

| Title: | Uni-prediction for combined inter mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Marta Karczewicz Do-Kyoung Kwon Madhukar Budagavi | Email: | vseregin@qti.qualcomm.com d-kwon@ti.com |
| Source: | Qualcomm Incorporated, Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution provides an analysis of two simplified solutions for combined prediction mode tested in SCE3.

# Introduction

It was noticed that combined mode from SCE3 [1] introduces bandwidth and computational complexity increase comparing to the HEVC worst case. Several contributions addressed this problem and reduced the complexity of the combined mode. Among those contributions, two approaches were identified the most simplest and further analysis of these two methods is provided in this contribution.

# Simplified combined prediction

Both methods [2, 3] provide complexity reduction for combined mode, achieving memory access and calculation complexity to be within HEVC worst case level.

Basic idea of this simplification is to restrict inter prediction, being part of the combined mode, to uni-directional inter prediction. The following table summarizes the details of each approach.

Table 1. Uni-prediction with combined mode.

|  | 8x8 CU 2NxN, Nx2N | Uni-prediction | Bi-prediction Identical motion | Bi-prediction Non identical motion | Chroma |
| --- | --- | --- | --- | --- | --- |
| JCTVC-M0220 | Not used | Combined prediction | Combined prediction | Bi-prediction | Combined prediction |
| JCTVC-M0297 | Not used | Combined prediction | For Merge mode, bi-directional MV is converted to uni-L0 MV. For AMVP mode, inter direction signaling is restricted to be uni-direction. |  | Not used |

In the next table, average BD rate reduction for luma and both chroma components among RA, LB, and LP configuration are summarized along with the average encoding and decoding running time.

Table 2. Experimental results against SHM1.0 IntraBL anchor.

|  | Luma BD rate | Chroma BD rate | Enc | Dec |
| --- | --- | --- | --- | --- |
| SCE3.2 | -0.92 | -4.35 | 113% | 100% |
| M0220 | -0.70 | 0.45 | 114% | 102% |
| M0297 | -0.79 | -3.45 | 106% | 99% |

Average memory accessed and calculation complexity for RA, LB, and LP tests cases are summarized in the next table.

Table 3. Average memory access and computational complexity

|  | Pure | DDR2 | DDR3 | Mults | Adds |
| --- | --- | --- | --- | --- | --- |
| SCE3.2 | 106% | 105% | 105% | 106% | 107% |
| M0220 | 104% | 104% | 104% | 104% | 105% |
| M0297 | 103% | 102% | 102% | 104% | 105% |

# References

[1] V. Seregin, J. Chen, and M. Karczewicz “SCE3: Combined inter mode (test 3.2)”, JCTVC-M0294, Incheon, KR, Apr. 2013.

[2] D.-K. Kwon, and M. Budagavi, “Non-SCE3: Uni-directional combined prediction (UCP) in Inter slice”, JCTVC-M0220, Incheon, KR, Apr. 2013.

[3] V. Seregin, M. Karczewicz “Non-SCE3: Bandwidth reduction for combined inter mode”, JCTVC-M0297, Incheon, KR, Apr. 2013.

# Specification text

## Specification text for JCTVC-M0297

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( nuh_layer_id > 0 ) |  |
| combined_mode_flag | ae(v) |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| } |  |
| else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( nuh_layer_id > 0 && !(log2CbSize == 3 && PartMode != PART_2Nx2N) ) |  |
| combined_mode_flag | ae(v) |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

combined_mode_flag equal to 1 indicates that the current coding unit is coded with combined prediction mode. combined_mode_flag equal to 0 indicates that the current coding unit is not coded with combined prediction mode.

When combined_mode_flag is not present, it is inferred to be equal to 0.

##### 8.5.3.2.1 Derivation process for luma motion vectors for merge mode

…

10. When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12, or combined_mode_flag[ xOrigP ][ yOrigP ] equal to 1, the following applies:

refIdxL1 = −1		(8-1)

predFlagL1 = 0		(8-2)

##### Table 7-1 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( ( nPbW + nPbH ) != 12 && combined_mode_flag == 0 ) | ( nPbW + nPbH ) = = 12 ¦¦ (combined_mode_flag == 1) |
| 0 | PRED_L0 | PRED_L0 |
| 1 | PRED_L1 | PRED_L1 |
| 2 | PRED_BI | na |

##### H.X.X.X Decoding process for combined inter prediction samples

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCbS specifying the size of the current luma coding block,

–	two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

–	the luma motion vectors mvL0 and mvL1,

–	the chroma motion vectors mvCL0 and mvCL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags, predFlagL0, and predFlagL1.

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

–	an (nCbSC)x(nCbSC) array preSamplesCb of chroma prediction samples for the component Cb, where nCbSC is derived as specified below,

–	an (nCbSC)x(nCbSC) array predSamplesCr of chroma residual samples for the component Cr, where nCbSC is derived as specified below.

The variable nCbSL is set equal to nCbS and the variable nCbSC is set equal to nCbS  >>  1.

This process is invoked for combined_mode_flag[xCb][ yCb] equal to 1.

The arrays predSamplesL, preSamplesCb, and predSamplesCr are derived by invoking 8.5.3.3 process with xCb, yCb, xBl, yBl, nCbS, nPbW, nPbH, mvL0,  mvL1,  mvCL0, mvCL1, predFlagL0, and predFlagL1 as an input. And the array predRLSamplesL is derived by invoking H.8.8.2 process with xCb, yCb, and log2CbSize ( where nCbS = 1 << log2CbSize ) as an input.

Only luma array is modified for combined mode as follows, with x = 0.. nCbS – 1, y = 0.. nCbS – 1:

predSamplesL[x][y] = ( predSamplesL [x][y] + predRLSamplesL [x][y] + 1 ) >> 1

## Specification text for JCTVC-M0220

coding_unit () has the same changes as in JCTVC-M0297.

##### H.X.X.X Decoding process for combined inter prediction samples

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xBl, yBl ) specifying the top-left sample of the current luma prediction block relative to the top-left sample of the current luma coding block,

–	a variable nCbS specifying the size of the current luma coding block,

–	two variables nPbW and nPbH specifying the width and the height of the luma prediction block,

–	the luma motion vectors mvL0 and mvL1,

–	the chroma motion vectors mvCL0 and mvCL1,

–	the reference indices refIdxL0 and refIdxL1,

–	the prediction list utilization flags, predFlagL0, and predFlagL1.

Outputs of this process are:

–	an (nCbSL)x(nCbSL) array predSamplesL of luma prediction samples, where nCbSL is derived as specified below,

–	an (nCbSC)x(nCbSC) array preSamplesCb of chroma prediction samples for the component Cb, where nCbSC is derived as specified below,

–	an (nCbSC)x(nCbSC) array predSamplesCr of chroma residual samples for the component Cr, where nCbSC is derived as specified below.

The variable nCbSL is set equal to nCbS and the variable nCbSC is set equal to nCbS  >>  1.

This process is invoked for combined_mode_flag[xCb][ yCb] equal to 1

The arrays predSamplesL, preSamplesCb, and predSamplesCr are derived by invoking 8.5.3.3 process with xCb, yCb, xBl, yBl, nCbS, nPbW, nPbH, mvL0,  mvL1,  mvCL0, mvCL1, predFlagL0, and predFlagL1 as an input.

The following applies if one of the following are true:

–	predFlagL0 is equal to 0 or predFlagL1 is equal to 0

–	mvL0 is equal to mvL1 and refIdxL0 is equal to refIdxL1

The array predRLSamplesL, predRLSamplesCb and  predRLSamplesCr are derived by invoking H.8.8.2 process with xCb, yCb, and log2CbSize ( where nCbS = 1 << log2CbSize ) as an input.

Luma array is modified for combined mode as follows, with x = 0.. nCbSL – 1, y = 0.. nCbSL – 1:

predSamplesL[x][y] = ( predSamplesL [x][y] + predRLSamplesL [x][y] + 1 ) >> 1

Chroma array is modified for combined mode as follows, with x = 0.. nCbSC – 1, y = 0.. nCbSC – 1:

predSamplesCb[x][y] = ( predSamplesCb [x][y] + predRLSamplesCb [x][y] + 1 ) >> 1

predSamplesCr[x][y] = ( predSamplesCr [x][y] + predRLSamplesCr [x][y] + 1 ) >> 1

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).