- H.7.3.8.5  Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag) |  |
| texture_rl_flag[ x0 ][ y0 ] | ae(v) |
| if( ! texture_rl_flag[ x0 ][ y0 ] ) { |  |
| … |  |
| if(( PredMode[ x0 ][ y0 ] != MODE_INTRA && !(PartMode = = PART_2Nx2N && merge_flag[x0][y0])) ¦¦ PredMode[ x0 ][ y0 ] = = MODE_TEXTURE_RL) ) |  |
| no_residual_syntax_flag | ae(v) |
| if( !no_residual_syntax_flag ) { |  |
| MaxTrafoDepth = ( PredMode[ x0 ][ y0 ] = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if ( PredMode[ x0 ][ y0 ] != MODE_INTRA && PredMode[ x0 ][ y0 ] != MODE_INTRA_BL && PartMode = = PART_2Nx2N) |  |
| APC_enable_flag[ x0 ][ y0 ] | ae(v) |
| transform_tree( x0, y0 x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |

- H.7.4.9.5 Coding unit semantics

The specifications in subclause F.7.4.9.5 apply with following modifications and additions.

texture_rl_flag[ x0 ][ y0 ] specifies the prediction mode for the current coding unit. texture_rl_flag[ x0 ][ y0 ] equal to 1 specifies that the prediction samples are derived using reconstructed samples of the reference layer representation as specified in subclause Error: Reference source not found; texture_rl_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded in either intra prediction or inter prediction mode.

When texture_rl_flag is equal to 1, the variable PredMode[ x ][ y ] and PartMode[ x ][ y ] are derived as follows.

for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS – 1,

PredMode[ x ][ y ] = MODE_TEXTURE_RL		(H-1)

PartMode [ x ][ y ] = PART_2Nx2N		(H-2)

When texture_rl_flag[ x0 ][ y0 ] not present, it is inferred to be equal to 0.

When texture_rl_flag[ x0 ][ y0 ] is equal to 1, the current coding unit is regarded as being coded with intra prediction mode in the in-loop filter process specified in subclause H.8.7.

APC_enable_flag[ x0 ][ y0 ] specifies whether the adaptive predictor compensatoin is enabled for the current coding unit. APC_enable_flag [ x0 ][ y0 ] equal to 1 specifies that the prediction samples are derived by averaging the temporal predicted samples and the reconstructed samples of the reference layer representation as specified in subclause Error: Reference source not found.5. When APC_enable_flag [ x0 ][ y0 ] not present, it is inferred to be equal to 0.

- H.8.8.5  Decoding process for adaptive predictor compensation

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	two variables nPbW and nPbH specifying the width and the height of the current luma prediction block,

–	a (PicWRL)x(PicHRL) array rlPicSampleL,

–	a (PicWRL)x(PicHRL) array rlPicSampleL,

–	an (nPbW)x(nPbH) array interPredSamplesL of luma inter prediction samples, where the interPredSamplesL [ x ][ y ] with ( x = 0 .. nPbW – 1, y = 0 .. nPbH – 1 ) are derived by invoking the luma sample interpolation process specified in subclause 8.5.3.3.1 with predSamplesL and luma sample location ( x ,  y ) given as inputs and predSamplesL[ x ][ y ] as output.

–	the reference indices refIdxL0 and refIdxL1 of the current luma prediction block,

Output of this process is a (nPbW)x(nPbH) array predredSamplesL of luma prediction samples.

For each luma sample location ( xL = 0..nPbW − 1, yL = 0..nPbH − 1 ) inside the prediction luma sample array predSampleL, the corresponding prediction luma sample value predSampleL[ xL][ yL ] is derived as follows:

- If the refIdxL0 is equal to or larger than 0, and the refIdxL1 is equal to or larger than 0, the following applies:

–	predSampleL[ xL][ yL ] = (interPredSamplesL [ xL][ yL ] + rlPicSampleL [ xC + xL][ yC + yL ]) >> 1

- Otherwise, the following applies:

–	predSampleL[ xL][ yL ] = interPredSamplesL [ xL][ yL ]