| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0449 |
| --- | --- |

| Title: | BoG Report on reference layer sample location derivation in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Jianle Chen | Email: | cjianle@qti.qualcomm.com |
| Source: | BoG chair |  |  |

_____________________________

# Abstract

The document contains a report of the break-out group (BoG) on reference layer sample location derivation. The following two topics are discussed during the meeting and the recommendations are made.

- Reference layer sample location derivation

- Reference layer sample location derviation used in resampling process

# Reference layer sample location derivation

Output of this process is a sample location indicates the reference layer sample location relative to the top-left sample of the reference layer picture.

## Current SHM (both software and text)

Mathematic equations are used to derive reference layer sample location in the current test model. The devision operation is used.

xRef = ( xP × PicWRL + scaledW / 2 ) / scaledW 
yRef = ( yP × PicHRL + scaledH / 2 ) / scaledH

## Method in JCTVC-M0133

Scaling factors are firstly derived at sequence level as follows

shiftX = 14
shiftY = 14

scaleFactorX = ( ( PicWRL << shiftX ) + ( scaledW >> 1 ) ) / scaledW
scaleFactorY = ( ( PicHRL << shiftY ) + ( scaledH >> 1 ) ) / scaledH

The sample position of the corresponding sample at reference layer is derived as follows.

xRef = ( ( xP ) * scaleFactorX + ( 1 << ( shiftX − 1 ) ) ) >> shiftX
yRef = ( ( yP ) * scaleFactorY + ( 1 << ( shiftY − 1 ) ) ) >> shiftY

## H.264/SVC method

Scaling factors are firstly derived at sequence level as follows

shiftX = ( ( levelIdc <= 30 )  ?  16  :  ( 31 − Ceil( Log2( PicWRL ) ) ) ) 
shiftY = ( ( levelIdc <= 30 )  ?  16  :  ( 31 − Ceil( Log2( PicHRL  ) ) ) )

## Comments and Recommendation

Comments:

One expert express that setting shift bit as 14 results in half pel rounding error in 8K picture size.

Setting shift bit as 16 results in 1/8 pel rounding error in 8K picture size.

Setting shift bit as 16 is safe to keep the dynamic range within 32 bits.

Fixed shift bits is preferred if the accuracy is enough.

Scaled picture offset need be considered. There are a few proposals in high level syntax section to address this issue which is orthogonal to the BoG topic.

Alignment with sample location derivation used in resampling process need to be considered

Recommendation:

- Adopt the method from JCTVC-M0133 with shift bit equal to 16, to align with the method recommended in section2.

# Reference layer sample location derivation used in resampling process

Output of this process is a sample location indicates the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

## Method in SHM text

Mathematic equations are used to derive the reference layer sample location as follows,

The variable xRef16 is derived as follow:

xRef16 = ( xP * PicWRL * 16 + ScaledW / 2) / ScaledW

The variable yRef16 is derived as follows:

If cIdx is equal to 0, the variable yRef16 is derived as follow:

yRef16 = (yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH

Otherwise, the variable yRef16 is derived as follow:

yRef16 = ( yP * PicHRL * 16 + ScaledH / 2 ) / ScaledH – offset

where the value of offset is derived as follows:

if (ScaledH  is equal to PicHRL)
		offset = 0
	otherwise if (ScaledH  is equal to 1.5 * PicHRL )
		offset = 1
	otherwise if (ScaledH  is equal to 2.0 * PicHRL )
		offset = 2

[Note: only SNR, 1.5x and 2.0x scalalabilies are supported]

## Method in SHM software

Similarly, scaling factors are firstly derived at sequence level as follows:

shiftX = 16
shiftY = 16

scaleFactorX = ( ( PicWRL << shiftX ) + ( ScaledW >> 1 ) ) / ScaledW
scaleFactorY = ( ( PicHRL << shiftY ) + ( ScaledH >> 1 ) ) / ScaledH

The variables of PhaseX, PhaseY are derived as follows:

phaseX = 0
phaseY = (cIdx = = 0) ? 0 : 1

[Note: phaseY = 1 for cIdx ! = 0 reflects accurate chroma phase alignment]

The variables of addX, addY are derived as follows:

addX = ( ( ( PicWRL * phaseX) << ( shiftX – 2 ) ) + ( ScaledW >> 1 ) ) /ScaledW + ( 1 << ( shiftX – 5 ) )

addY = ( ( ( PicHRL *  phaseY) << ( shiftY – 2 ) ) + (ScaledH >> 1 ) ) / ScaledH + ( 1 << ( shiftY – 5 ) )

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( xP * scaleFactorX + addX ) >> ( shiftX – 4 ) ) – 4 * phaseX
yRef16 = ( ( yP * scaleFactorY + addY ) >> ( shiftY – 4 ) ) – 4 * phaseY

[Note: this is a simplified version of H.264/SVC method, JCTVC-M0188, JCTVC-M0322 and JCTVC-M0425 propose to adopt this method in SHVC]

## H.264/SVC method

Comparing to the method mentioned in section 4.2, the following additions are used in H.264/SVC.

- The variables shiftX and shiftY is derived as follows,

shiftX = ( ( levelIdc <= 30 )  ?  16  :  ( 31 − Ceil( Log2( PicWRL ) ) ) )
shiftY = ( ( levelIdc <= 30 )  ?  16  :  ( 31 − Ceil( Log2( PicHRL  ) ) ) )

- Chroma phase offset is signaled to indicate the chroma sample displacement relative to luma sample in the unit of ½ luma sample

phaseX = (cIdx = = 0) ? 0 : chroma_phase_x
phaseY = (cIdx = = 0) ? 0 : chroma_phase_y

## Comments and Recommendations

Comments:

Multiple experts express that for 1.5x and 2.x scalabilities, the current software implementation does not have accuracy problem for any pict2ure size.

According to one expert’s analysis, up to 4k size, the grid accuracy is 1/32 for arbitrary spatial ratio scalability when shift bit is equal to 16,

It’s possible to simplify the position mapping when only SNR, 1.5x and 2.x spatial scalabilities are supported. If accurate chroma phase shift is not needed, the process can be further simplified.

Recommendation:

- Adopt the method mentioned in section2.2 into the text, which is the method used in the current software, as proposed in JCTVC-M0188, JCTVC-M0322 and JCTVC-M0425.

- Further study on whether higher accuracy, larger shift bit, is needed.

- Suggest cleaning the related codes containing the division operation which is disabled by default in the current SHM software.