| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0451 |
| --- | --- |

| Title: | AHG5: Cross-check report of CU based chroma intra prediction with reduced reference (JCTVC-M0412) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shohei Matsuo Masaaki Matsumura Seishi Takamura Hiroshi Fujii Atsushi Shimizu 1-1, Hikari-no-oka, Yokosuka-shi, Kanagawa 239-0847, JAPAN | Tel: Email: | +81-46-859-2070 matsuo.shohei@lab.ntt.co.jp +81-46-859-3521 matsumura.masaaki@lab.ntt.co.jp +81-46-859-2371 takamura.seishi@lab.ntt.co.jp +81-46-859-8452 fujii.hiroshi@lab.ntt.co.jp +81-46-859-2448 shimizu.atsushi@lab.ntt.co.jp |
| Source: | NTT (Nippon Telegraph and Telephone) Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for JCTVC-M0412.

# Introduction

To support non-4:2:0 formats, range extended software HM10.0 RExt2.0 was released in the 12th JCT-VC meeting. In order to improve the coding efficiency for non-4:2:0 sequences, JCTVC-M0412 proposed a CU-based chroma prediction method with reduced reference, which is a simplification method of LM chroma mode proposed in JCTVC-M0097. A load of the parameter derivation process for each coding unit is reduced by using the limited reference samples with sub-sampling. The detail of the proposal is described in JCTVC-M0412 [1].

# Cross-check results

The software version was HM10.0 RExt2.0. The reference was generated by using the common test conditions [2]. The provided source code was compiled by using Microsoft Visual Studio 2010 Professional edition, and the experiments were performed on the cluster of PCs with 64-bit Windows 7 professional, Intel Core i5-2500K (4.20GHz) and 16GB RAM. The obtained results are summarized in Table 1.

Table 1: Cross-check results of M0412 (CU-based chroma intra prediction with reduced reference).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -17.7% | -16.2% | -16.7% | -13.4% | -12.8% | -13.1% | -9.9% | -9.6% | -9.7% |
| YCbCr 4:4:4 | -1.8% | -6.3% | -5.9% | -1.9% | -4.0% | -5.2% | -1.8% | -2.6% | -3.7% |
| YCbCr 4:2:2 | -0.1% | -1.2% | -0.9% | -0.1% | -0.7% | -0.6% | -0.1% | -0.4% | -0.4% |
| Overall | -7.0% | -8.3% | -8.2% | -5.5% | -6.1% | -6.6% | -4.2% | -4.4% | -4.8% |
|  | -7.0% | -8.3% | -8.3% | -5.5% | -6.2% | -6.7% | -4.2% | -4.5% | -4.7% |
| Enc Time[%] | 107% |  |  | 107% |  |  | 108% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |

# Conclusion

This contribution reports the test results for a CU-based chroma intra prediction proposed in JCTVC-M0412. The results of coding efficiency completely matched with the proponent’s results. No mismatch between encoder’s local decoded picture and decoder’s output is observed. Relative encoding and decoding time are also confirmed. Source code was reviewed.

# References

- K. Kawamura, T. Yoshino, S. Naito, “AHG5: CU based chroma intra prediction with reduced reference,” JCTVC-M0412, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013.

- D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions,” JCTVC-L1006, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

# Patent rights declaration(s)

NTT Corporation does not have any current or pending patent rights relating to the technology described in this contribution.