| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0453 |
| --- | --- |

| Title: | Non-RCE2: Cross-check of JCTVC-M0439 on ‘Complexity Reduction for residual DPCM in HEVC lossless coding’ |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution reports the results of a cross-check of contribution JCTVC-M0439 on “Complexity Reduction for residual DPCM in HEVC lossless coding”. This cross-check confirms the results reported by the proponent of the test.

# Introduction

JCTVC-M0439 proposes a simplification of a residual DPCM scheme that was tested in RCE2 (JCTVC-M0079). In the original scheme, residual DPCM is applied only for the horizontal and vertical prediction directions, where each residual sample is predicted using the previous residual sample in the prediction direction, and the different subsequently coded. As the authors noted, each row (column) for the horizontal (vertical) prediction directional can be processed in parallel due to no dependencies existing between rows (columns). In JCTVC-M0439, the authors propose to further increase the degree of parallelization possible by breaking up the DPCM chain into chunks of a pre-defined size; the experiments used a chunk size of 8 samples. By signaling the residual sample values at the start of each DPCM chunk, each DPCM chunk can be reconstructed in parallel.

We have studied the source code provided by the proponents, and verified that it implements the technique described in JCTVC-M0439.

# Experimental results

The test sequences used are all four Class F sequences, Class B sequence 1080p Kimono1 and ParkScene, range extension 1080p 4:2:2 sequence EBUHorse and EBUWaterRocksClose (10-bit, 500 frames), range extension 1080p 4:4:4 sequence EBURainFruits and BirdsInCage (10-bit, 600 frames), and ten screen content 4:4:4 RGB sequences.

Results of the cross-check can be found in the accompanying excel spread sheets.

# Conclusion

This cross-check confirms the results reported by the proponent of the test.