| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0455 |
| --- | --- |

| Title: | BoG Report on SHVC complexity assessment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Elena Alshina Edouard François | Email: | edouard.francois@crf.canon.fr elena_a.alshina@samsung.com |
| Source: | BoG chair |  |  |

_____________________________

# Abstract

The document contains a report of the break-out group (BoG) on SHEV complexity assessment.

# Open issues related to complexity assessment

During the development of the complexity assessment module, some issues were extensively discussed. These are highlighted below.

- Should up-sampling and MC operations/memory bandwidth be reported separately or should they be added and reported as a single number?

It is recommended to include both up-sampling and MC operations/memory numbers in worst case analysis and intermediate data for reference. But to report the sum.

- What would be the scaler hardware architecture for SNR scalability in the current SHVC test model? (1) Would the scaler block be reused assuming scaling factor of 1 (i.e. integer pel positions in SNR case would be filtered with [ ... 0 0 1 0 0 ...] filter) or (2) Would BL picture pointer is used?

There is design flexibility. We can bypass re-sampling process in case when EL and BL picture sizes are identical. Or we can use trivial filter. The first option is less complex (optimal in some sense). It is suggested to assume the optimal implementation when we evaluate the worst case complexity.

But we don’t have this flexibility when scaling factor is 1, but BL and EL picture sizes are different.

MPEG, VCEG requirements say that this case shall be supported, but not under development yet.

This is important for complexity evaluation of RefdxPic only.

It was suggested to assume “no- resampling complexity” for SNR test (InterLayerRefPic created by redirecting the pointer to RecBL). After supporting of scaling factor is 1, but different BL and EL picture sizes will be clarified (HLS BoG ?) then complexity assessment for SNR case should be re-discussed.

- What is the implication of JCTVC-L0178 cropping and padding adoption at last meeting on memory bandwidth for SNR scalability for RefIdx picture based processing? Can BL picture pointer be directly reused by EL decoder or will picture data copy of BL picture be needed to create reference picture for EL?

The definition of picture size need clarification (cropped, decoded…). This is open issue discussed in SHVC HLS BoG. The complexity assessment for SNR case should be re-discussed based on decision.

- Memory bandwidth for RefIdx with picture based processing is computed in the complexity assessment module assuming LCU by LCU up-sampling with LCU size = 64. What would be the optimal block size to use? Would it need to be matched with min LCU size? Currently, average complexity for RefIdx picture based approach is calculated using 64x64 block, whereas the worst case is calculated using 16x16 block.

It was decided to consider both reading and writing for RefIdxPic re-sampling process. It was suggested to use block-size 32x32 (at least one company uses namely this block size in implementation). For reading process block extension in only vertical direction is needed. For writing process no block extension is needed.

Fig is needed for explanation.

- What scaling ratio do we consider for the worst spatial scalability case? Currently only ratios 2x and 1.5x are supported in SHM-1.0. But in the template, ratio 1x is used to compute the worst case numbers of spatial scalability (based on the hypothesis that in the future, SHVC will support any spatial ratio).

It was suggested to use ×1,5 as the worst case in case of spatial scalability.

- Memory band-width for motion vector mapping in RefIdx is not counted so far?

Memory band-width calculation for MV prediction from BL was added to both RefIdx and TextureRL. And impact of this process in very small (200% -> 200,4%) under assumption that compressed MV field is used.

- Do we consider BL reference pictures as additional pictures to be stored in the EL DPB?

Collocated BL is additional reference frame compared to single layer HEVC decoder.

Number of additional reference for GRP-like approach depends on AU definition, which is not mandate for this BoG.

- Should we count writing memory access (for reconstructed Pic)?

It was recommended to add writing memory calculation for the worst and average case complexity assessment.

- The concern about average complexity assessment module has been raised. We do need to find a good way to properly interpret those numbers.

For tools in SCE comparison average complexity is useful. Within one SCE category it helps to make decision. Interpretation of average complexity data (relation with power consumption) shall be studied in AhG.

- Memory access for “parsed data” is not included. Shall we count it? How we can measure it?

After discussion it was recommended not to count coefficients band-width.

- Normalization is not the same for the worst and average case complexity evaluation. Should we synchronize them?

[Lena]: s/w module out-puts memory access normalized per picture and number of computations normalized per number of samples predicted using MC or ILPred. The worst case memory access and computations were reported per sample. It is better to synchronize normalization. Which variant is preferable?

- How complex is SHVC compare to 2 single-layers HEVC (simulcast) and 1 (EL) layer HEVC decoders?

Excel spreadsheet attached to this report contains simple memory access calculation. Cross-verification by volunteers is needed. Preliminary conclusions are as follows.

It is suggested for SCEs to use single layer EL as reference, in order to assess the additional complexity due to scalability feature.

10.1. In the worst case 2 layers HEVC–simulcast is twice more complex (200%) compared to single layer HEVC.

10.2. In the worst case 2 layers SHVC has very small extra memory access compared to 2 layers HEVC–simulcast due to MV mapping. But this extra memory access is negligible (200,4%) since compressed MV filed is used.

10.3. Assuming block-based up-sampling on a fly for TextureRL and RefIdx frameworks the worst case complexity is 8x8 bi-direction MC prediction. Inter-layer texture prediction complexity is only 22% (TextureRL) / 31% (RefIdx) compared to 8x8 bi-pred.

10.4. Assuming block-based up-sampling on a fly for SNR scalability w/o inter-layer filtering the cost of inter-layer texture prediction is 16% compare to the worst case. With de-noising filter these numbers goes to 30% (TextureRL) / 37% (RefIdx) (but still do not exceed the worst case).

10.5. The memory access in RefIdx with picture processing in 152% (of single layer HEVC decoder) in ×1,5 case. Which is 8% higher than RefIDx with PU based processing.

| Process | ×1,5 | SNR |
| --- | --- | --- |
| 2 layers SHVC IntraBL | 145% | 200% |
| 2 layers SHVC RefIdx PU | 145% | 200% |
| 2 layers SHVC Picture Base IL processing (optional) | 166% | 200% (*) |
| 2 layers HEVC | 144% | 200% |
| 1 layer (EL) HEVC | 100% | 100% |

(*) Based on decision on #2 and #3 this number can be up-dated

# To Do List

Simplest version of the excel template for worst case memory access analysis is attached was created. After this one is agreed this need to be modified to include different memory architectures, to count computations (not only memory access). Complexity assessment for example tools planned to be studied in next SCE need to be discussed during this meeting.

# Recommendation

- Continue AhG17 activity, including the worst case and average complexity evaluation. Need to continue measurement for both memory band-width and computations.

- It is recommended to report data relatively to single layer decoder (on EL resolution).

- In case of several possible designs it is suggested to assume the optimal (lowest complexity) implementation for the worst case evaluation.

- It is recommended to include memory band-width measurement for reconstructed signal writing, MV mapping process and reading-writing during Picture Based re-sampling.

- Use spatial scalability ratio 1.5 for the worst case complexity evaluation (unless it will be decided that SHVC shall support arbitrary scalability ratio).

- Complexity evaluation spreadsheet and s/w module will be released in several versions. Anchor is the same for both but “test” part will contain example of tool in each SCE category. AhG will prepare “anchor” part. SCE coordinators are mandated to take responsibility for “examples” preparation.

- Issues #2 and #3 needs clarification before complexity evaluation methodology can be finalized (for “SNR” case).

- Consider Collocated BL Picture as additional reference frame (compare to single layer decoder).

- Interpretation of average complexity data (relation with power consumption) shall be studied in AhG.

# Explanation about The Worst Case complexity evaluation template

The evaluation describes here relates to the memory access.

| Process | EL_MC | MVmap | BL_MC | Resampling | Resampling | Rec EL | Rec BL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Access Type | Read | Read | Read | Read | Write | Write | Write |
| 2 layers SHVC IntraBL | 1 | 1 | 1 |  |  | 1 | 1 |
| 2 layers SHVC RefIdx PU | 1 | 1 | 1 |  |  | 1 | 1 |
| 2 layers SHVC RefIdx Pic | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 layers HEVC | 1 |  | 1 |  |  | 1 | 1 |
| 1 layer (EL) HEVC | 1 |  |  |  |  | 1 |  |

- Reading for MC: extended block (W+7)*(H+7)+(W+3)*(H/2+3), assuming 8t Luma and 4t Chroma filters

- Reading of MV map: 10 bytes per 16x16 region (compressed MV: 16 bits per component plus 4 bits for ref_idx, equals to 36 bits, rounded to 40 bits, doubled in case of bi-pred)

- Resampling in RefIdxPic:

  - Picture is processed 32x32 blocks (size suggested by TI)

    - Reading from RecBL

      - only vertical extension is needed

    - Writing to InterLayerRefPic

      - Not extended block

- Writing reconstructed signal: not extended block (W)*(H)+(W)*(H/2)