| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0457_v2 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 4th Meeting: Incheon, KR, 20–26 Apr. 2013 | Document: JCT3V-D0313_v2 |

| Title: | MV-HEVC/SHVC HLS: On inter-layer sample and syntax prediction indications (combining aspects of JCTVC-M0205 and JCTVC-M0046) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela (Nokia) Jill Boyce (Vidyo) | Email: | miska.hannuksela@nokia.com jill@vidyo.com |
| Source: | Nokia Corporation, Vidyo |  |  |

_____________________________

# Abstract

This contribution integrates JCTVC-M0205 and JCTVC-M0046 into one proposal.

The v2 revision better enables future harmonization with other contributions.

# Proposed Specification Text

## Proposed Changes in MV-HEVC and SHVC WD

### VPS Extension Syntax and Semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| direct_dep_type_len_minus1 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| } |  |

direct_dep_type_len_minus2 + 2 specifies the number of bits of the direct_dependency_type[ i ][ j ] syntax element. In bitstreams conforming to this version of this Specification [Ed.(MH): this Specification meaning MV-HEVC and refIdx based SHVC], the value of direct_dep_type_len_minus2 shall be equal 0. Although the value of direct_dep_type_len_minus2 shall be equal to 0 in this version of this Specification, decoders shall allow other values of direct_dep_type_len_minus1 in the range of 0 to 31, inclusive, to appear in the syntax.

[Ed.(MH): The derivation of NumTRLSamplePredRefLayers[ i ], NumTRLMotionPredRefLayers[ i ], TRLSamplePredEnabledFlag[ i ][ j ], TRLMotionPredEnabledFlag[ i ][ j ] will be included in the specification text for TextureRL based scalability.]

direct_dependency_type[ i ][ j ] is used to derive the variables NumRISamplePredRefLayers[ i ], NumRIMotionPredRefLayers[ i ], RISamplePredEnabledFlag[ i ][ j ], RIMotionPredEnabledFlag[ i ][ j ], NumTRLSamplePredRefLayers[ i ], NumTRLMotionPredRefLayers[ i ], TRLSamplePredEnabledFlag[ i ][ j ], TRLMotionPredEnabledFlag[ i ][ j ] as follows.

for( i = 1; i <= vps_max_layers_minus1; i++ )
	NumRISamplePredRefLayers[ i ] = 0
	NumRIMotionPredRefLayers[ i ] = 0
	NumTRLSamplePredRefLayers[ i ] = 0
	NumTRLMotionPredRefLayers[ i ] = 0
	for( j = 0; j < i; j++ )
		if( direct_dependency_flag[ i ][ j ]  = =  1 ) {
			RISamplePredEnabledFlag[ i ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1) & 1 )
			NumRISamplePredRefLayers[ i ] += RISamplePredEnabledFlag[ i ][ j ]
			RIMotionPredEnabledFlag[ i ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1) & 2 ) >> 1 )
			NumRIMotionPredRefLayers[ i ] += RIMotionPredEnabledFlag[ i ][ j ]
			TRLSamplePredEnabledFlag[ i ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1) & 4 ) >> 2 )
			NumTRLSamplePredRefLayers[ i ] += TRLSamplePredEnabledFlag[ i ][ j ]
			TRLMotionPredEnabledFlag[ i ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1) & 8 ) >> 3 )
			NumTRLMotionPredRefLayers[ i ] += TRLMotionPredEnabledFlag[ i ][ j ]
		}

It is a requirement of bitstream conformance that when NumRISamplePredRefLayers[ i ] or NumRIMotionPredRefLayers[ i ] is not equal to 0, NumTRLSamplePredRefLayers[ i ] and NumTRLMotionPredRefLayers[ i ] shall be equal to 0. It is a requirement of bitstream conformance that when NumTRLSamplePredRefLayers[ i ] or NumTRLMotionPredRefLayers[ i ] is not equal to 0, NumRISamplePredRefLayers[ i ] and NumRIMotionPredRefLayers[ i ] shall be equal to 0.

The variable InterLayerRefIdxEnableFlag for decoding a slice having nuh_layer_id equal to nuhLayerId is set equal to ( NumRISamplePredRefLayers[ LayerIdInVps[ nuhLayerId ] ] > 0 ).

### Slice segment header syntax and semantics

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| AltCollocatedIndicationFlag = 0 |  |
| if( nuh_layer_id > 0 && NumRIMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] ) { |  |
| alt_collocated_indication_flag | u(1) |
| AltCollocatedIndicationFlag = alt_collocated_indication_flag |  |
| } |  |
| if( AltCollocatedIndicationFlag ) |  |
| if( NumRIMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| … |  |
| if (nuh_layer_id > 0) { |  |
| … |  |
| if( NumTRLSamplePredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if (inter_layer_pred_enabled_flag |  |
| inter_layer_pred_only_flag | u(1) |
| } |  |
| if( NumTRLMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] > 0 ) |  |
| enable_inter_layer_motion_flag | u(1) |
| } |  |
| … |  |

...

alt_collocated_indication_flag equal to 0 specifies that a collocated picture for temporal motion vector prediction is indicated using collocated_from_l0_flag, if present, and collocated_ref_idx. alt_collocated_indication_flag equal to 1 specifies that a collocated picture for temporal motion vector prediction is indicated using collocated_ref_layer_idx. When alt_collocated_indication_flag is not present, it is inferred to be equal to 0.

The variable InterLayerMfmEnableFlag is derived as follows:

- If alt_collocated_indication_flag is equal to 0, InterLayerMfmEnableFlag is set equal to 0.

- Otherwise, if alt_collocated_indication_flag is equal to 1 and either the width or the height of the collocated picture is different from the width or the height, respectively, of the current picture, InterLayerMfmEnableFlag is set equal to 1.

- Otherwise, InterLayerMfmEnableFlag is set equal to 0.

collocated_ref_layer_idx specifies the collocated picture for temporal motion vector prediction as specified in subclause 8.5.3.2.7. When alt_collocated_indication_flag is equal to 1 and NumRIMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] is equal to 1, collocated_ref_layer_idx is inferred to be equal to 0. collocated_ref_layer_idx shall be in the range of 0 to NumRIMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1, inclusive.

…

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture. When not present, the value of inter_layer_pred_enabled_flag is inferred to be equal to 0.

inter_layer_pred_only_flag equal to 1 specifies that inter-layer prediction is used in decoding of the current picture. inter_layer_pred_only_flag equal to 0 specifies that inter-layer prediction may be used in decoding of the current picture. When not present, the value of inter_layer_pred_only_flag is inferred to be equal to 0.

The variable InterLayerTextureRlEnableFlag is derived as follows.

InterLayerTextureRlEnableFlag =  ( inter_layer_pred_enabled_flag ) && ( NumTRLSamplePredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] > 0 )

enable_inter_layer_motion_flag equal to 1 specifies that inter-layer motion prediction may be performed for the current slice.  enable_inter_layer_motion_flag equal to 0 specifies that inter-layer motion prediction is not performed for the current slice.

The variable InterLayerMotionPredictionEnableFlag is derived as follows.

InterLayerMotionPredictionEnableFlag  =  
	enable_inter_layer_motion_flag && ( NumTRLMotionPredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] > 0 )

The variables InterRefEnabledInRPLFlag and InterLayerRefEnabledInRPLFlag are derived as follows.

- If NumTRLSamplePredRefLayers[ LayerIdInVps[ nuh_layer_id ] ] is greater than 0, InterRefEnabledInRPLFlag is set equal to !inter_layer_pred_only_ flag and InterLayerRefEnabledInRPLFlag is set equal to inter_layer_pred_enabled_flag

- Otherwise, InterRefEnabledInRPLFlag is set equal to 1 and InterLayerRefEnabledInRPLFlag is set equal to 1.

### Temporal motion vector derivation process

- 8.5.3.2.7	Derivation process for temporal luma motion vector prediction

…

- Depending on the values of alt_collocated_indication_flag, collocated_ref_layer_idx, slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the collocated picture, is derived as follows:

- If alt_collocated_indication_flag is equal to 1, colPic is derived as follows

for( currIdx = 0, i = NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] – 1; i >= 0; i-- )
	if( RIMotionPredEnabledFlag[ LayerIdInVps[ nuh_layer_id ] ][ i ] ) {
		if( currIdx  = =  collocated_ref_layer_idx )
			colPic = the picture in the current access unit with
				nuh_layer_id equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ]
		currIdx++
	}

- Otherwise, if slice_type is equal to B and collocated_from_l0_flag is equal to 0, colPic is set equal to RefPicList1[ collocated_ref_idx ].

- Otherwise (slice_type is equal to B and collocated_from_l0_flag is equal to 1 or slice_type is equal to P), colPic is set equal to RefPicList0[ collocated_ref_idx ].

### Reference picture set and list related changes in Annex G (MV-HEVC and SHVC WD)

- G.7.4.7.2	Reference picture list modification semantics

The specifications in subclause Error: Reference source not found apply with following modifications.

–	Equation (7-43) specifying the derivation of NumPocTotalCurr is replaced by:

NumPocTotalCurr = 0
for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++)
	if(UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
		NumPocTotalCurr++
for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++)	(G-1)
	if(UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		NumPocTotalCurr++
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
	if( UsedByCurrPicLt[ i ] )
		NumPocTotalCurr++
NumPocTotalCurr += NumRISamplePredRefLayersNumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]

…

- G.8.1.1	Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer pictures RefPicSetInterLayer.

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0, j = 0; i < NumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) {
	if( RISamplePredEnabledFlag[ LayerIdInVps[ nuh_layer_id ] ][ i ] ) {
		RefPicSetInterLayer[ ij ]  = the picture with picture order count equal to PicOrderCnt and
			nuh_layer_id equal to RefLayerId[ LayerIdInVps[ nuh_layer_id ][ i ] ]
		RefPicSetInterLayer[ ij++ ] is marked as "used for long-term reference"
	}
}

- G.8.1.2	Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies.

for( i = 0; i < NumRISamplePredRefLayersNumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; i++ ) 
	RefPicSetInterLayer[ i ] is marked as "used for short-term reference"

…

- G.8.3.4	Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause 8.5.3.3.2. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture lists RefPicList0 and, for B slices, RefPicList1 are derived as follows:

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	if ( InterRefEnabledInRPLFlag ) {
		for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(G-2)
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	} 
	if ( InterLayerRefEnabledInRPLFlag ) {
		for( i = 0; i < NumRISamplePredRefLayersNumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ]; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ] 
	} 
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++)			(G-3)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	if ( InterRefEnabledInRPLFlag ) {
		for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(G-4)
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	} 
	if ( InterLayerRefEnabledInRPLFlag ) {
		for( i = 0; i< NumRISamplePredRefLayersNumDirectRefLayers[ LayerIdInVps[ nuh_layer_id ] ] ; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
	} 
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(G-5)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

## Proposed Changes in SHVC Test Model Text

### SPS extension syntax changes

- G.7.3.2.2.1	Sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_inter_layer_mfm_enable_flag | u(1) |
| } |  |

### Coding unit syntax changes for TextureRL

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enable_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if (nuh_layer_id > 0 && InterLayerTextureRlEnableFlag ) |  |
| texture_rl_flag[ x0 ][ y0 ] | ae(v) |
| if( !texture_rl_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| … |  |

texture_rl_flag[ x0 ][ y0 ] specifies the prediction mode for the current coding unit. texture_rl_flag[ x0 ][ y0 ] equal to 1 specifies that the prediction samples are derived using reconstructed samples of the reference layer representation as specified in subclause Error: Reference source not found; texture_rl_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is coded in either intra prediction or inter prediction mode.

When texture_rl_flag[ x0 ][ y0 ] not present, it is inferred to be equal to inter_layer_pred_only_flag.

When texture_rl_flag is equal to 1, the variable PredMode[ x ][ y ] and PartMode[ x ][ y ] are derived as follows.

for x = x0..x0 + nCbS − 1 and y = y0..y0 + nCbS – 1,

PredMode[ x ][ y ] = MODE_TEXTURE_RL		(H-xx)

PartMode [ x ][ y ] = PART_2Nx2N		(H-xx)

When texture_rl_flag[ x0 ][ y0 ] not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Vidyo may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).