| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0459 |
| --- | --- |

| Title: | non-SCE3: Cross-check for uni-prediction for combined inter mode (JCTVC-M0445) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Junghye Min Elena Alshina | Email: | jh643.min@samsung.com elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This report verifies performance of combined prediction mode which allows combination with only uni-directional MC.

# Introduction

S/w provided by proponents contains implementation which matches description in JCTVC-M00445: combined mode is not applies if EL prediction is bi-directional (regular bi-pred is performed in this case). Additionally in 8x8 CUs only 2Nx2N partitioning is allowed. With these restrictions the worst case memory band-width doesn’t exceed HEVC the worst case.

# Test results

Test results we obtained perfectly match with proponents data (shown in Table 1). Tool was tested vs SHM1.0 (IntraBL framework).

Table 1. Performance test summary.

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,2% | -1,5% | -1,6% |  |  |  | -0,1% | -1,0% | -1,2% |
| Class B | -0,2% | -1,0% | -1,3% | -0,2% | -1,3% | -1,7% | -0,3% | -1,8% | -2,2% |
| Overall (Test vs Ref) | -0,2% | -1,2% | -1,4% | -0,2% | -1,3% | -1,7% | -0,3% | -1,5% | -1,9% |
| Overall (Test vs single layer) | 19,1% | 30,4% | 31,2% | 16,2% | 26,7% | 28,6% | 14,5% | 26,8% | 30,0% |
| EL only (Test vs Ref) | -0,2% | -1,3% | -1,5% | -0,3% | -1,6% | -2,0% | -0,3% | -1,7% | -2,1% |
| Enc Time[%] | 112,1% |  |  | 109,8% |  |  | 108,2% |  |  |
| Dec Time[%] | 98,8% |  |  | 97,3% |  |  | 96,5% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1,1% | -4,2% | -4,3% |  |  |  | -1,5% | -4,5% | -5,3% |
| Class B | -1,4% | -3,6% | -4,3% | -1,4% | -5,5% | -6,2% | -3,0% | -8,6% | -10,2% |
| Overall (Test vs Ref) | -1,3% | -3,8% | -4,3% | -1,4% | -5,5% | -6,2% | -2,6% | -7,5% | -8,8% |
| Overall (Test vs single layer) | 24,4% | 31,1% | 32,0% | 20,8% | 25,1% | 26,9% | 19,3% | 23,4% | 25,4% |
| EL only (Test vs Ref) | -1,7% | -4,3% | -4,8% | -2,1% | -6,6% | -7,4% | -3,2% | -8,4% | -9,8% |
| Enc Time[%] | 118,3% |  |  | 113,8% |  |  | 112,9% |  |  |
| Dec Time[%] | 98,8% |  |  | 95,7% |  |  | 95,7% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,2% | -1,7% | -1,8% |  |  |  | -0,3% | -1,5% | -1,8% |
| Class B | -0,3% | -1,3% | -1,5% | -0,2% | -1,8% | -2,0% | -0,4% | -2,2% | -2,6% |
| Overall (Test vs Ref) | -0,3% | -1,4% | -1,6% | -0,2% | -1,8% | -2,0% | -0,4% | -2,0% | -2,4% |
| Overall (Test vs single layer) | 27,9% | 35,3% | 36,3% | 24,4% | 30,4% | 32,4% | 23,3% | 30,7% | 33,7% |
| EL only (Test vs Ref) | -0,4% | -1,6% | -1,8% | -0,5% | -2,3% | -2,5% | -0,5% | -2,3% | -2,7% |
| Enc Time[%] | 110,2% |  |  | 108,6% |  |  | 106,8% |  |  |
| Dec Time[%] | 96,8% |  |  | 96,5% |  |  | 97,2% |  |  |
| Enc Mem[%] | 100,3% |  |  | 100,3% |  |  | 100,3% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |