| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0460 |
| --- | --- |

| Title: | BoG Report on Performance analysis and non-normative encoder enhancements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Alberto Duenas | Email: | Alberto.duenas@ngcodec.com |
| Source: | BoG chair |  |  |

_____________________________

# Abstract

The document contains a report of the break-out group (BoG) on Performance analysis and non-normative encoder enhancements.

The following contributions were presented on this BoG.

- JCTVC-M0036 Adaptive bit allocation for R-lambda model rate control in HM [B. Li, H. Li, L. Li (USTC)]

This document presents an adaptive bit allocation algorithm to improve the R-lambda model rate control algorithm in HM. With the proposed adaptive bit allocation algorithm, when targeting the HM-10.0 anchor bitrate, the coding efficiency is improved by about 2.9%, 2.9%, 1.9%, and 2.0% for RA-Main, RA-Main10, LB-Main, and LB-Main10 respectively (Class F sequences excluded). The average performance gap between enabling rate control algorithm and HM-10.0 anchor is about 2.4%, 2.3%, 1.9%, and 1.9% for RA-Main, RA-Main10, LB-Main, and LB-Main10 respectively (Class F sequences excluded). The bitrate accuracy is also improved a little.

This document also presents several modifications to support intra picture rate control better. This document applies a unified model and parameter updating method for both intra and inter pictures. With the modifications for intra coded pictures, the average Y BD-Rate gap between the R-lambda model rate control algorithm and HM-10.0 anchor are 0.8%, 0.7%, 2.5%, 2.4%, 1.8%, 1.7%, 1.1%, and 0.9% for AI-Main, AI-Main10, RA-Main, RA-Main10, LB-Main, LB-Main10, LP-Main, and LP-Main10 respectively. The average sequence level bitrate error are 0.00%, 0.00%, 0.18%, 0.19%, 0.07%, 0.11%, 0.09%, and 0.10% for AI-Main, AI-Main10, RA-Main, RA-Main10, LB-Main, LB-Main10, LP-Main, and LP-Main10 respectively.

The BoG recommends adoption on future versions of the reference software model for HEVC of the enhancements that are non related to intra picture that are presented on this contribution and thanks USTC for their work on this contribution and the improvement of the reference software model for HEVC.

- JCTVC-M0037 Rate control by R-lambda model for SHVC [L. Li, B. Li, H. Li (USTC)]

This contribution extends the R-λ model based rate control algorithm in HEVC to SHVC. The proposed algorithm was implemented in SHM-1.0 for both the ref_idx framework and Intra_BL framework. When targeting the bitrate of enhancement layer of SHM-1.0 anchor, the average bitrate errors (the difference of the target bitrate and the actual bitrate) of the proposed algorithm are about 0.01%~0.14% for different cases, the average luma BD-rate compared to the anchor is about 2.5%~4.4%.

It was noted that they are potential improvement to be done to the current R-λ model based rate control algorithm in SHVC and this proposal address some of them.

The BoG recommends adoption of this contribution on future versions of the reference software model for SHVC and thanks USTC for their work on this contribution the improvement of the reference software model for SHVC.

- JCTVC-M0257 Intra Frame Rate Control Based on SATD [M. Karczewicz, X. Wang (Qualcomm)]

In this contribution, an R-λ model based rate control scheme for intra frame/slice is proposed for HEVC. The proposed rate control algorithm is implemented on HM-10.0. Compared with the existing rate control algorithm in HM-10.0, it is asserted that the proposed algorithm provides more accurate matching of target rate while maintaining similar coding efficiency.

No presentation uploaded.

It was mentioned that several of the proposed ideas have been also presented on JCTVC-M0036.

The differences between this proposal and JCTVC-M0036 were discussed, and it was mentioned that they do actually have many similarities and that JCTVC-M0257 has some enhancements in addition to what is proposed on JCTVC-M0036

The BoG recommends adoption of this contribution on future versions of the reference software model for HEVC and thanks Qualcomm for their work on this contribution.

- JCTVC-M0259 Encoder Optimization for SHVC: Enhancement Layer Lambda Refinement in RA Configurations [X. Li, J. Chen, M. Karczewicz (Qualcomm)]

In this proposal, an enhancement layer encoder optimization method is proposed for SHVC RA configurations. Lagrange multiplier in rate-distortion module of enhancement layer is refined based on temporal level, resolution ratio and QP gap between base and enhancement layer. It is reported that on average 0.8%, 1.1% and 1.2% BD-rate reduction was obtained for Y, U, and V components in both IBL and RefIdx frameworks.

No presentation uploaded.

The BoG recommends adoption of this contribution on future versions of the reference software model for SHVC and thanks Qualcomm for their work on this contribution and thanks Qualcomm for their work on this contribution the improvement of the reference software model for SHVC.

- JCTVC-M0041 Updated results on HEVC still picture coding performance [K. Ugur, J. Lainema (Nokia)]

It was indicated that this contribution was planned to be presented on a Joint meeting with JPEG and it decided that is presented during that joint meeting.

- JCTVC-M0166 Comparison of Compression Performance of HEVC Draft 10 with AVC for UHD-1 material [P. Andrivon, P. Bordes (Technicolor), M. Arena, P. Sunna (RAI)]

This contribution relates performances comparison between H.264/AVC (JM18.4) and HEVC Draft 10 (HM10.0) considering 8-bit and 10-bit bit-depth UHD 3840x2160 resolution (a.k.a. 2160p or UHD-1 or Quad Full HD) material. Test material coding with different nature is assessed and both the Main Profile and the consumer-oriented Main 10 Profile. Concerning UHD-1 8 and 10 bits test material, this contribution states that HEVC Main and Main 10 Profiles (HM10.0) outperforms AVC High and equivalent High 10 Profiles (JM18.4 with HM-like configuration files) in objective measure (BD-rate) of around 25% for All Intra, 45 % for Random Access, 45 % for Low Delay B on average and it shows result of comparing HEVC and HEVC Main 10.

### The results for HEVC Main Profile vs AVC High Profile (8-bit DPB) for 2160p (8-bit) sequences are as follows:

|  | Y | U | V |
| --- | --- | --- | --- |
| AI | -26.6% | -22.8% | -30.6% |
| RA | -47.3% | -34.7% | -41.1% |
| LB | -48.7% | -33.0% | -41.3% |

AI gains of HEVC Main HM10.0 over JM18.4 are around 25 % for AI, 45% for both RA and LB configurations.

### The results for HEVC Main 10 Profile vs AVC High Profile (10-bit DPB) for 2160p (10-bit) sequences are as follows:

|  | Y | U | V |
| --- | --- | --- | --- |
| AI | -25.5% | -20.8% | -26.9% |
| RA | -46.8% | -23.6% | -31.1% |
| LB | -50.8% | -28.5% | -37.3% |

Results presented here for Main 10 are consistent with those presented for HEVC Main.

### The results for comparing the HEVC Main and Main 10 Profiles comparison considerations for 2160p 8-bit and 10-bit sequences respectively are as follows:

|  | Y | U | V |
| --- | --- | --- | --- |
| AI | -5.3% | -17.0% | -24.0% |
| RA | -6.9% | -24.1% | -32.7% |
| LB | -7.3% | -25.7% | -33.9% |

The following numbers are to be interpreted very carefully as the table stands for a summary of different nature PSNR-based metric computation. It is remarked that 10-bit content here contains non-null two LSB.

Considering HEVC, here above presented results as well as attached detailed results tend to show that for the considered versatile test material and test conditions, 10-bit coding does objectively not add supplementary cost than 8-bit coding in terms of objective fidelity to the source content.

The group acknowledge the very positive results of the current Main and Main 10 HEVC compared with AVC High and High 10 Profiles for UHD-1 content and the additional gains provided by using Main 10 HEVC profile (10 bit content) compared with Main profile on UHD 8 bit material and thanks Technicolor and Rai for their work on this contribution.

- JCTVC-M0234 Combined Scalable + Multiview HEVC coding results [D.-K. Kwon, M. Budagavi (TI)]

Not presented yet (no presenter available Wed 15:30).

- JCTVC-M0329 Comparison of Compression Performance of HEVC Draft 10 with AVC High Profile [B. Li (USTC), G. J. Sullivan, J. Xu (Microsoft)]

This contribution is a further study of the relative objective (i.e. PSNR-based) compression performance of HEVC Main Profile and AVC High Profile. It builds upon the prior work reported in JCTVC-G399 JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, updating the results by using the latest available reference software (JM-18.4 and HM-10.0). Experimental results (without class F) reportedly show that 1) for the Main Profile all-intra configuration, HM-10.0 can save about 21.9% in bit rate; 2) for the Main Profile random-access configuration, HM-10.0 can save about 34.3% in bit rate; and 3) for the Main Profile low-delay configuration, HM-10.0 can save about 36.8% in bit rate when compared with JM-18.4.

The group acknowledge the very positive results of the current Main HEVC compared with AVC High Profiles for a variety of video material and thanks USTC and Microsoft for their work on this contribution.