| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M0462 |
| --- | --- |

| Title: | BoG report on range extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JCT-VC |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | D. Flynn | Email: | dflynn@blackberry.com |
| Source: | Range Extensions BoG |  |  |

_____________________________

# Abstract

A break out group was held on the 24th of April to examine contributions related to the range extensions.  Contributions covering topics relating to bitdepth greater than 10-bit were presented, and a summary of contributions relating to intra prediction for the 4:2:2 chroma format was produced and reviewed.

# Non-CE Technical Contributions

## Range extensions

### General

JCTVC-M0178 AHG5: Range Extensions and High Bit Depths [K. Sharman, N. Saunders, J. Gamei (Sony)]

This document presents the current capabilities of HEVC Range Extensions when operating at low (including negative) QPs, and proposes that such low QPs will be needed if the codec is to truly support high bit depths, as required by the mandate of the Range Extensions amendment. Possible sources of errors that may be caused by internal accuracy limitations currently present in HEVC are explored. It is claimed that some changes to those accuracies can mitigate the errors and thereby extend the operating range of HEVC. In addition two independent changes to the entropy coding are presented, which are stated to benefit both efficiency and practicality. These changes have been included in this proposal rather than in two separate proposals due to their applicability to high operating points.

This contribution covers three areas relating to handling higher bit depths.  Firstly, issues that affect the PSNR at high bit depths; secondly, issues that affect the bitrate and problems of data expansion at high bit depths; finally, issues that affect the throughput of coding bypass bins.

All examination has been performed using test material that is in 16 bit log formats, which originated from noisy film scanned material from SVT. Filtered to prevent selection of 4x4 TUs due to noise at high rates.

[The contribution was presented by colleagues of the authors, who were unable to attend the meeting]

Examination of PSNR

The contribution provides a spreadsheet that contains a large amount of data that can be selected on charts to see how performance changes for a given set of parameters.

Q: It was remarked that calculation of lambda has been investigated before in AVC to attempt to solve similar issues.  Is the lambda correct for negative QPs?

Q: All results are RGB, has what happens in the YCbCr domain been checked?

The authors suggest increasing transform dynamic to bitDepth + 5 (currently fixed at 15), increasing the transform coefficient range to bitDepth + 6 (currently 16) and increasing the transform matrix coefficient precision to 14bit.

Comment: Discomfort was expressed with the use of 8bit, 10bit and 12bit PSNR values being compared.  Perhaps compare PSNRs all at the same bit depth.

Examination of bitrate

At low QPs for 16- and 14bit coding, data expansion occurs if one wants an MSE that is lower than the expected MSE for a specific bitdepth.

The contribution introduces a concept of fixed bits.  Each coefficient is split into MSBs and LSBs.  MSBs are coded using the current coefficient coding.  LSBs are then coded in bypass.  Division point is signalled in slice header, a fixed relation between QP and the division point was used in practice.

Q: How does sign data hiding behave with this?  Would SDH have any use in this?  What would happen if SDH is disabled, as it is perhaps just adding noise for no bitrate benefit since the rates are so high.

The methods provides a 35% saving at QP −48 (16bit), at QP=−12 (10bit), 35% and 10%.

Comment: Consideration of low QP also needs to consider the lossless case.  Set lossless to Nbit and N-1bit and that will give a range of bitrates to then decide a desirable QP range.  Some of this is addressed in the submission.

Examination of throughput

Many bins at negative QPs are bypass bins, which is a fairly complex method for transmission of many EP bins.  Optimisation is possible when the current interval range is 256, the EP bins are directly contained, allowing multiple bins to be extracted.  The authors propose methods to force alignment.

- force range to 256,will waste 0.55 bits per alignment on average

- by extracting an EP bin in a particular case. 0.30 bits per alignment
At QP 22, method costs 0.84%, QP=0 cost is 0.28%, for negative qp the cost is minimal

Suggests such a scheme should be only used for high bitrates and levels.

Comment: If three years ago we had adopted pipe or v2v this wouldn't have been an issue.  There were schemes proposed for decoding multiple EP bins, comment at the time was that we didn't need it at normal rates.

Comments on profiling

An additional section on profiling was not presented.

General comments

A comparison of all proposals against AVC, HEVC, and proposed system was presented.

No headline figure available, but an interesting graph is shown using bitdepth 14.  Proposed methods at high rates help significantly.

Conclusion

Aims: to raise awareness of the issue and present the results of an extensive study into the issues.  Not proposing to have any adoption at this meeting, but would like some form of formal activity to investigate these issues further, perhaps in an AHG or CE.

Interesting even for HDR applications,

Recommend an AHG (K. Sharman)

JCTVC-M0256 Suggested fixes for HEVC Range Extensions draft text [D. Flynn, J. Sole, T. Suzuki] [miss]

No need to present as this is superseded by other contributions that address chroma intra prediction angles.

JCTVC-M0335 AhG5: Offset Scaling in SAO for High Bit-depth Video Coding [W.-S. Kim, J. Sole, M. Karczewicz (Qualcomm)]

Discussion chaired by C. Rosewarne.

In the last meeting deficiency of SAO was reported when bit-depth is larger than 10. In this contribution offset scaling methods in SAO are proposed when bit-depth is larger than 10 to improve coding efficiency. In the first method, different scale factor is used for luma and chroma offsets with linear scaling according to bit-depth. In case of luma offset is left-shifted by (bitDepth – 11) and in case of chroma offset is left shifted by (bitDepth – 10). In the second method, it is proposed to apply either linear or non-linear offset scaling for luma and chroma separately with independent scale factors. These parameters can be signalled in the slice header so that offset scaling can be adaptive to input sequence characteristics. In both methods, no change is applied for entropy coding/decoding part.

Experimental results reportedly show coding efficiency improvements by 0.2/0.2/0.2 (%) for Y/Cb/Cr in case of All-Intra-HE-Main-Tier with internal bit-depth increase by 2 (IBDI 2), and 0.3/0.3/0.4 (%) with internal bit-depth increase by 4 (IBDI 4). In case of RA-HE-Main-Tier, the gains are reported as 0.5/0.1/0.2 (%) and 0.5/0.3/0.3 (%) for IBDI 2 and 4, respectively. In case of LB-HE-Main-Tier, the gains are reported as 0.6/0.6/0.6 (%) and 0.7/0.8/0.7 (%) for IBDI 2 and 4, respectively.

Currently sao_offset_abs is restricted to 5bits for bit depths greater than or equal to 10 and the calculation of saoOffsetVal is scaled linearly to compensate for the restriction.  Analysis has shown that such scaling may not match the distribution of effective offset values, where offsets of lower magnitude are used more frequently.  Two solutions are proposed:

- A linear scaling using a different slope than currently used for luma.  The previous approach has a steeper gradient of four and the courser quantisation of lower values results in losses.  The proposed approach uses a gradient of two.

- A non-linear scaling function is used comprising of a piece-wise linear model. This allows finer quantisation at low values while maintaining the full range of the current approach.

It was commented that the first proposal is very simple and obtains gains while the second one looks more complex and the gains seem not much higher and would support adoption of the first method.  It was also commented that the second approach is much simpler in terms of implementation than as presented, we should adopt the first method and would support further study of the second option.

The cross-checker reported that the second method appears too complex and that AhG study seems appropriate.

Recommend: Adopt the first method, and conduct further study of the second method.

JCTVC-M0405 AHG5: Crosscheck of JCTVC-M0335 on offset scaling in SAO for high bit-depth video coding [C.-W. Hsu, Y.-W. Huang (MediaTek)] [late] [miss]

JCTVC-M0118 Removal of the offset quantization in SAO [C.-W. Hsu, C.-Y. Chen, C.-Y. Tsai, Y.-W. Huang, S. Lei (MediaTek)]

In this contribution, it is proposed to remove the quantization process of offsets in SAO for sample bit depths larger than 10. When applied to HEVC range extensions common test conditions, the proposed method reportedly achieves 0.1%, 0.2%, 0.2%, 0.3%, 0.2%, 0.3%, and 0.2%, BD-rate reductions in AI Main-tier, AI High-tier, AI Super High-tier, RA Main-tier, RA High-tier, LB Main-tier, and LB High-tier, respectively, with unchanged run times.

This contribution removes the current limit on the range of sao_offset_val currently limited to 5 bits at bit depths greater than or equal to 10, allowing it to increase linearly with bit depth.  It was reported that there is an issue with the worst case number of bins since sao_offset_val is unary coded.  The maximum bin counts would increase from 31 bins to 127 bins for 12 bit, and to 511 bins for 14 bit configurations.  The proponent suggested that clipping might fix this issue.

Recommend further study since adopting this solution would require a further fix to resolve the worst case bin count issue.

JCTVC-M0344 Cross-check for Removal of the offset quantization in SAO [S.Jeong, J.Min, E. Alshina] [late]

### Intra prediction

Previously it was identified that four contributions, JCTVC-M0127, JCTVC-M0138, JCTVC-M0156, and JCTVC-M0232 all identify and attempt to modify the intra chroma angle derivation for 4:2:2 chroma formats.  After an initial review was conducted in the main JCT-VC meeting track, it was suggested that a summary of the four proposals should be provided along with a clear description of the current and modified behaviours.

The observation

Currently in 4:2:2, the chroma angle is adjusted from the normal angle that would have been used for a given angular mode index. Two consequences of this have been noted:

- This process produces new angles that would not otherwise be used in 4:2:0 or 4:4:4.

- The mode-dependent scan direction is based upon the mode index and is unaware of the angle adjustment.

How things currently happen (spec)

The following derivation processes/chains occur:

- IntraPredModeY → IntraPredModeC → IntraPredAng →4:2:2adj→ IntraPredAngAdj
[	IntraPredModeY → IntraPredModeC → IntraPredAng (using cIdx dependent table) ]

- IntraPredModeC → ScanIdx (MDCS):

  - IntraPredModeC ∈ [6..14]	=> ScanIdx = 2

  - IntraPredModeC ∈ [22..30]	=> ScanIdx = 1

  - Otherwise	=> ScanIdx = 0

Summary of the proposals

There are four proposals that address these two observations, by either:

- Making the angles use pre-existing angles for 4:2:2.

  - By changing the angle during prediction:	M0127(1A, 2A, 3A), M0138

  - By changing the IntraPredModeC derivation:	M0127(1B, 2B, 3B), M0156

- Making the ScanIdx related to the effective IntraPredModeC:

  - By changing the IntraPredModeC derivation:	M0127(1B, 2B, 3B), M0156

  - By changing the decision range for MDCS:	M0232

Implies M0232 need only be considered with M0127(1A, 2A, 3A), M0138.

Information about various tables:

- M0127 has three tables, table (1) performs best. Tables (2) and (3) are able to be algorithmically derived from the current 4:2:0 table.  This algorithmic derivation isn't trivial.

- M0156 has a table that does not adjust the non-DM angle, ie it continues to use mode 34, not 31.

- M0138 has two tables, the second is identical to M0127 table (1)

The following table summarises the various contributions, listing the performance and proposed modification.  The tableID identifies solutions with the same effective mapping of chroma mode to chroma angle (this can be achieved either by changing the IntraPredModeC derivation, or the IntraPredAng derivation).

| Contribution | TableID | Proposed modfication | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y | U | V | Y | U | V | Y | U | V |
| 127 – 1A | 1 | IntraPredAng | 0.0% | −0.1% | −0.1% | 0.0% | −0.1% | −0.1% | 0.0% | 0.0% | 0.0% |
| 127 – 1B | 1 | IntraPredModeC | −0.1% | −0.1% | −0.1% | −0.1% | −0.1% | −0.1% | 0.0% | −0.1% | −0.1% |
| 127 – 2A | 2 | IntraPredAng | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 127 – 2B | 2 | IntraPredModeC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 127 – 3A | 3 | IntraPredAng | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 127 – 3B | 3 | IntraPredModeC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 138 – 1 | 4 | IntraPredAng | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 138 – 2 | 1 Same as 127–1A | IntraPredAng | 0.0% | −0.1% | −0.1% | 0.0% | −0.1% | −0.1% | 0.0% | 0.0% | 0.0% |
| 156 | ? (except non-DM) | IntraPredModeC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 156+127.table1 | 1 (except non-DM) | IntraPredModeC | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| 156+127.table2 | 2 (except non-DM) | IntraPredModeC | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| 156+127.table3 | 3 (except non-DM) | IntraPredModeC | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| 232 | N/A | scanIdx | 0.0% | −0.1% | −0.2% | 0.0% | −0.1% | −0.1% | 0.0% | 0.0% | 0.0% |

Discussion

It was commented that the proposals are a small fix and if there is little to distinguish them, the highest performing method should be picked.  The highest performing methods are highlighted in bold above.

It was remarked that the method of M0156 forces a specific implementation method due to the exceptional handling of non-DM modes (ie, the horizontal, vertical and diagonal angles), whereas other methods may be implemented in multiple ways.

Recommend: use method 1B of M0127.

References

JCTVC-M0127 AHG5: Unified intra prediction angles for 4:2:2 chroma format [H. Nakamura, M. Ueda, S. Fukushima, T. Kumakura (JVC Kenwood)]

JCTVC-M0372 AHG5: Cross-check on Unified intra prediction angles for 4:2:2 chroma format (JCTVC-M0127) [A. Minezawa, S. Sekiguchi (Mitsubishi)] [late] [miss]

JCTVC-M0383 Cross-check of proposal ‘AHG5: Unification of chroma intra prediction angles between 4:2:2 chroma format’ (M0127) by JVC Kenwood [C. Rosewarne, M. Maeda (Canon)] [late]

JCTVC-M0138 AHG5: On chroma intra prediction for 4:2:2 [C. Rosewarne, M. Maeda (Canon)]

JCTVC-M0424 Cross-check of chroma intra prediction for 4:2:2 (JCTVC-M0138) [J. Sole (Qualcomm)] [late]

JCTVC-M0156 AHG5: Simplification of chroma intra prediction process for YUV4:2:2 coding [A. Minezawa, S. Sekiguchi, T. Murakami (Mitsubishi)]

JCTVC-M0409 Cross-check of simplified chroma-intra-prediction process for YUV4:2:2 coding (JCTVC-M0156) [K. Kawamura, S. Naito (KDDI)] [late]

JCTVC-M0232 AHG5: Intra Prediction Mode-Dependent Coefficient Scanning for 4:2:2 Chroma Extended Format [Z. Chen, S. Liu, S. Lei (MediaTek)]

JCTVC-M0368 Cross-check of Intra Prediction Mode-Dependent Coefficient Scanning for 4:2:2 (JCTVC-M0232) [J. Sole (Qualcomm)] [late]