| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-Mxxxx |
| --- | --- |

| Title: | Tile boundary alignment and inter-layer prediction constraints for SHVC and MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Sühring Robert Skupin Gerhard Tech Thomas Schierl Krishna Rapaka Wei Pu Xiang Li Jianle Chen Ye-Kui Wang Marta Karczewicz Kemal Ugur Miska M. Hannuksela | Email: | karsten.suehring@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de gerhard.tech@hh.fraunhofer.de thomas.schierl@hhi.fraunhofer.de krapaka@qti.qualcomm.com wpu@qti.qualcomm.com lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com yekuiw@qti.qualcomm.com martak@qti.qualcomm.com kemal.ugur@nokia.com miska.hannuksela@nokia.com |
| Source: | Fraunhofer HHI, Qualcomm Incorporated,Nokia |  |  |

_____________________________

# Abstract

The Joint BoG on high level syntax discussed the proposals JCTVC-M0198, JCTVC-M0202, JCT3V-D0051, JCTVC-M0334, and asked the proponents to create a joint text based on the discussion as follows:

BoG suggests to revisit after side activity led by K. Suehring to draft a document aligned with M0035 containing the following:

- VUI flag to indicate strict tile alignment semantic constraint

- SPS extension flag to indicate upsampling filter restriction normative decoding process (SHVC) only

- VUI or SPS flag to indicate temporal MV Prediction restriction for MV-HEVC and SHVC, and for MV-HEVC also constrains the inter-layer motion vector

# VUI flag to indicate strict tile alignment

A flag is proposed to indicate strict alignment of base and enhancement layer tiles as a constraint flag in the VUI syntax. The encoder can set the flag to indicate that tile boundaries are aligned in the whole CVS. Although no normative decoding action is associated with the constraint, the information is considered helpful for view parallel decoding of tiles.

Changes relative to [1] are highlighted in yellow.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| aspect_ratio_info_present_flag | u(1) |
| … |  |
| if( bit stream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| if ( nuh_layer_id > 0 ) |  |
| tile_boundaries_aligned_flag | u(1) |
| motion_vectors_over_pic_boundaries_flag | u(1) |
| restricted_ref_pic_lists_flag | u(1) |
| min_spatial_segmentation_idc | u(8) |
| max_bytes_per_pic_denom | ue(v) |
| max_bits_per_mincu_denom | ue(v) |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| } |  |
| } |  |

Alternatively the constrain flag can be

tile_boundaries_aligned_flag equal to 1 indicates that all tile boundaries of the corresponding base layer picture have corresponding tile boundaries in the given enhancement layer and there exist no additional tile boundaries in the enhancement layer. That means, for any two enhancement layer picture samples that lie within the same enhancement layer tile, the collocated base layer samples shall also lie within the same reference layer tile and for any two reference layer picture samples that lie within the same reference layer tile, the collocated enhancement layer samples shall also lie within the same enhancement layer tile. tile_boundaries_aligned_flag equal to 0 indicates that there is no restriction on the tile configuration between corresponding enhancement layer and the given reference layer.

# SHVC upsampling filter restriction

It is proposed to normatively constrain the upsampling filter at slice and tile boundaries. Each of the constraints can be activated separately by using a flag in the SPS extension.

| if( sps_extension_flag ) |  |
| --- | --- |
| sps_inter_layer_mfm_enable_flag | u(1) |
| resampling_filter_across_slices_enabled_flag | u(1) |
| resampling_filter_across_tiles_enabled_flag | u(1) |

resampling_filter_across_slices_enabled_flag signals whether the resampling filter shall be constrained at slice boundaries. If resampling_filter_across_slices_enabled_flag is equal to 0, slice boundaries are treated in the same way as that of picture boundaries for resampling process. That is, when a pixel sample outside the current slice (may be inside or outside the current picture) is needed for resampling, the pixel is generated with pixel padding from those inside the current tile. The padding process is the same as that for defined for pixels outside the current picture.. If resampling_filter_across_slices_enabled_flag is equal to 1 the upsampling filter is not constrained at slice boundaries.

It is a requirement of bitstream conformance that loop_filter_across_slices_enabled_flag shall be equal to 0 in all picture parameter sets that are referred to in all direct reference layers.

resampling_filter_across_tiles_enabled_flag signals whether the spatial upsampling filter shall be constrained at tile boundaries. If resampling_filter_across_tiles_enabled_flag is equal to 0, tile boundaries are treated in the same way as that of picture boundaries for resampling process. That is, when a pixel sample outside the current tile (may be inside or outside the current picture) is needed for resampling, the pixel is generated with pixel padding from those inside the current tile. The padding process is the same as that for defined for pixels outside the current picture... If resampling_filter_across_tiles_enabled_flag is equal to 1 the upsampling filter is not constrained at tile boundaries.

It is a requirement of bitstream conformance that loop_filter_across_tiles_enabled_flag shall be equal to 0 in all picture parameter sets that are referred to in all direct reference layers.

The changes to the decoding process are provided in a separate document.

# SHVC and MV-HEVC motion interpolation constraint

For independent/parallel tile decoding motion vector and motion vector based inter prediction can be constrained at the tile boundaries. This can be achieved by an encoder restriction. No normative decoding process change is associated with the restriction.

We propose to use the same constraint for both SHVC and MV-HEVC. The constraint is written in a way that it covers all inter motion prediction processes generically. Thus it can be applied without the need to be adapted for each change of interpolation freedom that could be introduced in the standardization process.

Whether the constraint applies is signalled by a flag in the SPS extension. This flag, being equal to 1, means that three bitstream constraints are applied.

- Motion vectors used for inter-layer prediction are constraint in a way that the reference samples are located in the same reference layer tile.

- When the collocated picture is in a layer different to the layer of the current picture, syntax elements shall be constraint such that the collocated motion vector only needs to be derived, when the position of the collocated block is in the (collocated) reference picture tile.

- Moreover loop filtering over tile boundaries shall be disabled in the reference layer.

Proposed Syntax and semantics changes

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_layer_pred_tile_constraint_flag | u(1) |
| inter_view_mv_vert_constraint_flag | u(1) |
| } |  |

Definitions:

inter-layer reference picture: A reference picture that is used for inter-layer prediction.

collocated reference layer tile A tile including the collocated position of the top-left position of a block that uses the picture of the tile for inter-layer prediction.

Semantics

inter_layer_pred_tile_constraint_flag specifies whether inter-layer prediction at tile boundaries is constrained in the CVS.

–	If inter_layer_pred_tile_constraint_flag is equal to 1, the following applies:

–	When the decoding process of a block of a picture uses inter-layer prediction, the coded picture shall be constraint such that the inter-layer prediction only depends on data elements of the inter-layer reference picture that can be derived from the collocated reference layer tile of the block.

–	The syntax element loop_filter_across_tiles_enabled_flag of all inter-layer reference pictures used by the current picture shall be equal to 0.

–	Otherwise, ( inter_layer_pred_tile_constraint_flag is equal to 0 ), no constraints are signalled by this flag.

When not present the inter_layer_pred_tile_constraint_flag is inferred to be equal to 0.

# Additional bitstream restriction

The following text is proposed to be added to restrict the tile alignment, when independent tile decoding is used in the bitstream:

It is a requirement of bitstream conformance, that when resampling_filter_across_tiles_enabled_flag is equal to 0 and tile_boundaries_aligned_flag is present in the same SPS, tile_boundaries_aligned_flag shall be equal to 1. when resampling_filter_across_tiles_enabled_flag is equal to 0 and tile_boundaries_aligned_flag is not present in the same SPS, the value of tile_boundaries_aligned_flag shall be inferred to be equal to 1.

It is a requirement of bitstream conformance, that when inter_layer_pred_tile_constraint_flag is equal to 1 and tile_boundaries_aligned_flag is present in the same SPS, tile_boundaries_aligned_flag shall be equal to 1. When inter_layer_pred_tile_constraint_flag is equal to 1 and tile_boundaries_aligned_flag is not present in the same SPS, the value of tile_boundaries_aligned_flag shall be inferred to be equal to 1.

# References

- B. Bross, “High Efficiency Video Coding (HEVC) text specification draft 10“, JCTVC-K1003, Shanghai, CN, Oct. 2012

- “Joint Call for Proposals on Scalable Video Coding Extensions of High Efficiency Video Coding (HEVC)“, ISO/IEC JTC1/SC29/WG11 N12957, Stockholm, Sweden, July 2012

- K. Suehring , R. Skupin, Y. Sanchez, T. Schierl, “Independent tile upsampling for SHVC“, JCTVC-M0198, Incheon, KR, Apr. 2013

- G. Tech, Y. Sanchez, K. Suehring, R. Skupin, T. Schierl, “Restriction of inter-view prediction on tile boundaries“, JCT3V-Dxxxx, Incheon, KR, Apr. 2013

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).