| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M1101 |
| --- | --- |

| Title: | HEVC Scalable Extensions Core Experiment SCE1: Support for additional resampling phase shifts |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Alshina Elena Xiang Li Jie Dong | Tel: Email: | elena_a.alshina@samsung.com lxiang@qti.qualcomm.com Jie.Dong@interdigital.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document is a description of Core Experiment 1 on support for additional resampling phase shift. Interpolation filters with additional to SHM2.0 resampling phase shift will be studied in context of support synchronization up-sampling and down-sampling processes down-sampling phase shift and Chroma samples location relatively to Luma samples.

# Introduction

This document defines Core Experiment 1 (SCE1) support for additional resampling phase shift in SHVC, to be performed for the 14th JCT-VC meeting. The SCE consists of proposals which suggest adding to the standard signaling for phase shift used during down-sampling process location of Chroma samples relatively to Luma samples. Synchronization of up-sampling and down-sampling filter processes and accurate Chroma positions alignment requires adding resampling filter not supported yet in SHM. So up-sampling filter phases are calculated with 1/16 precision in SHM, but up-sampling filter coefficients for only specific phases are defined (Table 1, “SHM”). During these phases calculation zero displacement between enhancement and baser layer and vertical ½ pel shift of Chroma sample relative to Luma in 4:2:0 sampling mode are assumed.

Table 1. Phase filters used for 1.5x and 2x up-sampling

| Ratio | Color Component & Direction | Applicable Phase |  |
| --- | --- | --- | --- |
|  |  | “0” EL&BL displacement (SHM) | “non-zero” EL&BL displacement (M0465) |
| 2x | Luma | 0, 8 | 4, 12 |
|  | Chroma Horizontal | 0, 8 | 6, 14 |
|  | Chroma Vertical | 6, 14 | 4, 12 |
| 1.5x | Luma | 0, 5, 11 | 2, 8, 13 |
|  | Chroma Horizontal | 0, 5, 11 | 4, 10, 15 |
|  | Chroma Vertical | 4, 9, 15 | 3, 8, 14 |

Filling blanks as shown in Table 2 in SHM re-sampling filters set is needed and will be studied in this SCE.

Table 2. Existing and additional re-sampling filters.

| 16-phase resampling filter |  |
| --- | --- |
| Luma | Chroma |
| phase p; interpolation filter coefficients. fL[ p, 0 ]; fL[ p, 1 ]; fL[ p, 2 ]; fL[ p, 3 ]; fL[ p, 4 ]; fL[ p, 5 ]; fL[ p, 6 ]; fL[ p, 7 ]. 0; 0; 0; 0; 64; 0; 0; 0; 0. 1; n/a; n/a; n/a; n/a; n/a; n/a; n/a; n/a. 2; NEW, will be used in x1,5 test. 3; n/a; n/a; n/a; n/a; n/a; n/a; n/a; n/a. 4; NEW, will be used in x2 test. 5; −1; 4; -11; 52; 26; -8; 3; -1. 6; n/a; n/a; n/a; n/a; n/a; n/a; n/a; n/a. 7; n/a; n/a; n/a; n/a; n/a; n/a; n/a; n/a. 8; −1; 4; -11; 40; 40; -11; 4; -1. 9; n/a; n/a; n/a; n/a; n/a; n/a; n/a; n/a. 10; n/a; n/a; n/a; n/a; n/a; n/a; n/a; n/a. 11; −1; 3; -8; 26; 52; -11; 4; -1. 12; NEW, will be used in x2 test. 13; NEW, will be used in x1,5 test. 14; n/a; n/a; n/a; n/a; n/a; n/a; n/a; n/a. 15; n/a; n/a; n/a; n/a; n/a; n/a; n/a; n/a. | interpolation filter coefficients. fC[ p, 0 ]; fC[ p, 1 ]; fC[ p, 2 ]; fC[ p, 3 ]. 0; 64; 0; 0. n/a; n/a; n/a; n/a. n/a; n/a; n/a; n/a. NEW, will be used in x1,5 test. −4; 54; 16; −2. −6; 52; 20; −2. −6; 46; 28; −4. NEW, will be used in x1,5 test. −4; 36; 36; −4. −4; 30; 42; −4. need to be specified, will be used in x1,5 test. −2; 20; 26; −6. NEW, will be used in x2 test. n/a; n/a; n/a; n/a. −2; 10; 58; −2. 0; 4; 62; −2. |

This SCE also contains proposal addressing the same issue by signaling resampling filter coefficients in a bit-stream.

# Participants

| Participants | Affiliation |
| --- | --- |
| E. Alshina (elena_a.alshina@samsung.com), A. Alshin (alexander_b.alshin@samsung.com), A. Saxena (asaxena@sta.samsung.com) | Samsung |
| Li Xiang (lxiang@qti.qualcomm.com), Guo Liwei (liweig@qti.qualcomm.com), J. Chen (cjianle@qti.qualcomm.com) | Qualcomm |
| Kemal Ugur (kemal.ugur@nokia.com | Nokia |
| Dong, Jie (Jie.Dong@interdigital.com), He, Yuwen (Yuwen.He@interdigital.com), Ye, Yan (Yan.Ye@interdigital.com) | Inter Digital |
| Koohyar Minoo (koohyar.minoo@arrisi.com), David Baylon (david.baylon@arrisi.com) | Arris |
| ONNO Patrice (Patrice.Onno@crf.canon.fr) | Canon |
| Shan Lui(shan.liu@meidatek.com), Tzu-Der Chuang (peter.chuang@mediatek.com), Yu-Wen Huang (yuwen.huang@mediatek.com), P. Lai (polin.lai@mediatek.com), Mei Guo (Mei.Guo@mediatek.com), Zhenzhong Chen (zhenzhong.chen@mediatek.com) | MediaTek |
| Andrew Segall (asegall@sharplabs.com) | Sharp |
| Madhukar Budagavi (madhukar@ti.com), Do-Kyoung Kwon (d-kwon@ti.com) | TI |
| Auyeung, Cheung (Cheung.Auyeung@am.sony.com), Jun Xu (jun.xu@am.sony.com) | Sony |

# Crosschecking assignments

| Tests | Title | # tests | Tester | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1.1 | Performance with default parameter values | ×2 (1 test); ×1,5 (1 test); | Inter Digital | Qualcomm |
| 1.2 | The accuracy of down-sampler filter phase | ×2 (1 test); ×1,5 (3 tests); | Samsung Inter Digital Qualcomm | Arris |
| 1.3 | Accurate Chroma position alignment | ×2 (1 test); ×1,5 (1 test); | Samsung Qualcomm | Arris |
| 2.1 | Phase Compensation by signalling filter coefficients on sequence level | ×2 (1 test); ×1,5 (1 test); | Qualcomm | Samsung |
| 2.2 | Phase Compensation by signaling filter coefficients at PPS with sample shift | ×2 (1 test); ×1,5 (1 test); | Arris, | Inter Digital |

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The SHVC common test conditions defined in JCTVC-M1009 Error: Reference source not found shall be used for this CE.

The CE shall test following cases.

|  | AI | RA | LD-B | LD-P |
| --- | --- | --- | --- | --- |
| Scalability | 2x, 1.5x |  |  |  |

Even if LD-P test is no mandatory in JCTVC-M1009 it will be performed in this SCE in order to simplify the detection of abnormal behaviour associated with non-accurate re-sampling filter design.

Additionally to test sequences specified in JCTVC-M1009 Error: Reference source not found videos generated using alternative down-sampler will be included into the test. It is agreed to use test sequences generated by InterDigital [1]. The sampling grid offsets between BL and EL pictures are the same as those used during the development of H.264/AVC SVC extension. The relative luma/chroma sampling grids are the same as those of the CTC sequences, i.e., vertical ½-pel shift and horizontal zero shift. Additional test sequences, down-sampler s/w, command line example and MD5 sum are available upon request .

  - Software

Reference software SHM-2.0 will be used as the anchor and base software in this CE.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures, and complexity. Complexity assessment is defined in 4.3.1. BD-rate results shall be reported compare to SHM-2.0 anchor and ctc sequences (“0” EL&BL displacement) [2]. Since base layer is modified reporting performance vs HEVC single layer (optional in CTC) is mandatory for this SCE tests.

### Complexity assessment

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed either with or without output of decoded YUV data for both the Anchors and the Test. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

To evaluate memory bandwidth requirement and computational complexity related to up-sampling and MC interpolation filter, methodologies defined in AhG 17 (worst case theoretical analysis and average memory access measurement using s/w module distributed by AhG17) shall be used.

# Topics to be studied

The study of following aspects was requested:

1) Possibility of usage lower precision for signalled down-sampler phase shift.

2) Performance impact of accurate Chroma position alignment.

3) Performance analysis of fixed vs adaptive coefficients solution.

Proponents are encouraged to provide 3 sets of test results using down-sampler phase shift accuracy 1/16, 1/8 and 1/4. Proponents are also encouraged to provide performance test with and without taking into account accurate Chroma position alignment (for phases grid 1/16 only).

2 possible solutions with fixed and adaptively selected re-sampling filters will be studied in 2 different categories.

# Tools to test

## Category 1: Resampling filter with predetermined coefficients.

## Test 1.1 Performance with default parameter values

## Goal:

Verify if the signaling method in JCTVC-M0465 can provide bit-exact results of SHM-2.0 anchor with default parameter values on CTC sequences.

## Based on

## JCTVC-M0465 : “Signalling of Phase Offset in Up-sampling Process and Chroma Sampling Location” (Qualcomm, Nokia, InterDigital)

Test sequences

CTC[2] sequences and test conditions will be used in this test.

Test description

Specification text for this test (identical to recent SHM text).

The variables phaseXC and phaseYC are derived by

phaseXC      = 0 	// not signalled
phaseYC     = 1     // not signalled

vertical_phase_offset16=0	   // not signalled

horizontal_phase_offset16=0  	// not signalled

The variable xRef16 is derived as follows:

If cIdx is equal to 0, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX =  1 << ( iShiftX - 5 );

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) – horizontal_phase_offset16;

–	Otherwise, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX  = ( ( (PicWRL* phaseXC) << (iShiftX - 2) ) +(ScaledW >> 1) ) / ScaledW+ (1 << ( iShiftX - 5 ));

iDeltaX= 4* phaseXC;

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) – iDeltaX –(horizontal_phase_offset16>>1);

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY =  1 << ( iShiftY - 5 );

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16  = ( (yP*iScaleY + iAddY) >> iShiftYM4) – vertical_phase_offset16;

–	Otherwise, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY  = ( ( (PicHRL* phaseYC) << (iShiftY - 2) ) +(ScaledH >> 1) ) / ScaledH+ (1 << ( iShiftY - 5 ));

iDeltaY= 4*phaseYC;

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16 = ((yP*iScaleY + iAddY) >> iShiftYM4) – iDeltaY – (vertical_phase_offset16>>1);

Up-sampling filter coefficients are specified in SHM text.

## Test 1.2. “The accuracy of down-sampler filter phase”.

## Goal:

## Study performance impact of signaled down-sampler phase shift precision. It helps to choose amount of additional filters needed. If precision of signalled down-sampler phase shift is decided then amount of additional up-sampling filters for Luma is known.

Based on

## JCTVC-M0465 : “Signalling of Phase Offset in Up-sampling Process and Chroma Sampling Location” (Qualcomm, Nokia, InterDigital)

JCTVC-M0263: “AHG13: SHVC Upsampling with phase offset adjustment” (Arris)

## JCTVC-M0425: “About phase calculation and up-sampling filter coefficients in JCTVC-M0188 and JCTVC-M0322” (Samsung)

Test sequences

Additional sequences down-sampled using “non-zero” phase shift and specified in section 4.1 shall be used in this test.

Test description

Actual down-sampler phase offset is fixed in this test. The series of tests with different accuracy for signalled down-sampler phase offset will be done. Values of signalled down-sampler phase are specified in Table 3.

Table 3. Parameters horizontal_phase_offset16= vertical _phase_offset16 (both signaled in SPS) are summarised in the table:

| ×2 | ×1,5 | phase_offset accuracy |
| --- | --- | --- |
| NA | 3 | 1/16 |
| NA | 2 | 1/8 |
| 4 | 4 | 1/4 |

The position calculation is as follows (changes compare to recent SHM text are highlighted in yellow):

The variables phaseXC and phaseYC are derived by

phaseXC      = 0 	// not signalled
phaseYC     = 1     // not signalled

vertical_phase_offset16	   // signalled

horizontal_phase_offset16  	// signalled

The variable xRef16 is derived as follows:

If cIdx is equal to 0, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX =  1 << ( iShiftX - 5 );

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) – horizontal_phase_offset16;

–	Otherwise, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX  = ( ( (PicWRL* phaseXC) << (iShiftX - 2) ) +(ScaledW >> 1) ) / ScaledW+ (1 << ( iShiftX - 5 ));

iDeltaX= 4* phaseXC;

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) – iDeltaX –(horizontal_phase_offset16>>1);

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY =  1 << ( iShiftY - 5 );

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16  = ( (yP*iScaleY + iAddY) >> iShiftYM4) – vertical_phase_offset16;

–	Otherwise, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY  = ( ( (PicHRL* phaseYC) << (iShiftY - 2) ) +(ScaledH >> 1) ) / ScaledH+ (1 << ( iShiftY - 5 ));

iDeltaY= 4*phaseYC;

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16 = ((yP*iScaleY + iAddY) >> iShiftYM4) – iDeltaY – (vertical_phase_offset16>>1);

Up-sampling filter coefficients are specified in JCTVC-M0425 and JCTVC-M0465.

## Test 1.3 “Accurate Chroma position alignment”.

## Goal:

## Study performance impact of accurate Chroma alignment in re-sampling process.

Based on

## JCTVC-L0083: “TEA1: Fixed up-sampling filter tests by Samsung (tests 4.1 and 4.4)” (Samsung).

JCTVC-M0465: “Signaling of Phase Offset in Up-sampling Process and Chroma Sampling Location” (Qualcomm, InterDigital, Nokia)

Test sequences

CTC[2] sequences and test conditions will be used in this test.

Test description

Visual quality impact shall be studied in this test.

The position calculation is as follows (changes compare to recent SHM text are highlighted in yellow):

The variables phaseXC and phaseYC are derived by

phaseXC      = 0 	// signalled
phaseYC     = 0  // signalled

vertical_phase_offset16=0	   // not signalled

horizontal_phase_offset16=0  	// not signalled

The variable xRef16 is derived as follows:

If cIdx is equal to 0, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX =  1 << ( iShiftX - 5 );

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) – horizontal_phase_offset16;

–	Otherwise, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX  = ( ( (PicWRL* phaseXC) << (iShiftX - 2) ) +(ScaledW >> 1) ) / ScaledW+ (1 << ( iShiftX - 5 ));

iDeltaX= 4* phaseXC;

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) – iDeltaX –(horizontal_phase_offset16>>1);

- The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY =  1 << ( iShiftY - 5 );

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16  = ( (yP*iScaleY + iAddY) >> iShiftYM4) – vertical_phase_offset16;

–	Otherwise, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY  = ( ( (PicHRL* phaseYC) << (iShiftY - 2) ) +(ScaledH >> 1) ) / ScaledH+ (1 << ( iShiftY - 5 ));

iDeltaY= 4*phaseYC;

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16 = ((yP*iScaleY + iAddY) >> iShiftYM4) – iDeltaY– (vertical_phase_offset16>>1);

## Category 2: Adaptive resampling filters.

This category of tests studies the advantage of signaling filter coefficients only based on the knowledge of down-sampling process. As such the filters are not optimized based on the sequence content. The two tests under this sub-category would also show the effectiveness of the fixed resampling filters, under different down-sampling condition (especially, arbitrary phase offset, and narrower-band anti-aliasing filter).

## Test 2.1 “Phase Compensation by signalling filter coefficients on sequence level”.

## Goal:

## Study performance of adaptive signalling of filter coefficients, to match the known phase-offset required for up-sampling. In this test filter coefficients are designed only based on the down-sampling phase offset information. Results for this test will be reported by BDR comparison vs. corresponding tests in SCE1.1 Test 2, where BL and EL are not exactly aligned, for different phase accuracies in table 3. under column 1.5x scalability. Note that the results of this test can be used as the anchor for the SCE1.1-Test 2, to see how much is gained by exact phase shifting with 1/6=(2.67/16), as opposed to 3/16 (for study of 1/16 phase offset accuracy), 2/16 (for study of 1/8 phase offset accuracy) and 4/16 (for study of 1/4 phase offset accuracy).

Based on JCTVC-M0263: “AHG13: SHVC Upsampling with phase offset adjustment” (Arris)

JCTVC-M0267: “SCE4: Results of Test 4.2.2 on Adaptive Re-Sampling Filter” (Qualcomm)

Test sequences

Additional sequences down-sampled using “non-zero” phase shift and specified in section 4.1 shall be used in this test.

Test description

## Filter coefficients to be signalled at sequence level.

Signalling of the filter coefficients are done through the syntax in Table 4.

Table 4.  Syntax for signaling of filter information in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| seq_auf_flag | u(1) |
| if(seq_auf_flag) { |  |
| for(i=0;i<16;i++) { |  |
| seq_auf_luma_phase_x[i] | u(1) |
| if(seq_auf_ luma _phase_x[i]) { |  |
| for(j=0;j<8;j++) |  |
| seq_auf_ luma _coeff_x[i][j] | se(v) |
| seq_auf_luma_phase_y[i] | u(1) |
| if(seq_auf_ luma _phase_y[i]) { |  |
| for(j=0;j<8;j++) |  |
| seq_auf_ luma _coeff_y[i][j] | se(v) |
| } |  |
| } |  |
| for(i=0;i<16;i++) { |  |
| seq_auf_chroma_phase_x[i] | u(1) |
| if(seq_auf_ chroma _phase_x[i]) { |  |
| for(j=0;j<4;j++) { |  |
| seq_auf_ chroma0 _coeff_x[i][j] |  |
| seq_auf_ chroma1 _coeff_x[i][j] | se(v) |
| } |  |
| seq_auf_chroma_phase_y[i] | u(1) |
| if(seq_auf_ chroma _phase_y[i]) { |  |
| for(j=0;j<4;j++) { |  |
| seq_auf_ chroma0 _coeff_y[i][j] | se(v) |
| seq_auf_ chroma1 _coeff_y[i][j] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

seq_auf_flag indicates whether AUF will be applied when generating inter-layer reference picture for this layer. seq_auf_flag equal to 1 specifies that AUF will be applied. seq_auf_flag equal to 0 specifies that AUF will not be applied. When seq_auf_flag is not present, it is inferred as 0.

seq_auf_luma_phase_x[i], seq_auf_luma_phase_y[i] specify whether luma AUF coefficients will be signaled for phase i in x and y directions.

seq_auf_ luma _coeff_x[i][j], seq_auf_ luma _coeff_y[i][j] specify the luma AUF coefficients for phase i in x and y directions. If they are not present, they are set equal to the default up-sampling filter at the same phase position..

seq_auf_chroma_phase_x[i], seq_auf_chroma_phase_y[i], specify whether chroma AUF coefficients will be signaled for phase i in x and y directions.

seq_auf_ chroma0 _coeff_x[i][j], seq_auf_ chroma0 _coeff_y[i][j], seq_auf_ chroma1 _coeff_x[i][j], seq_auf_ chroma1 _coeff_y[i][j] specify the chroma AUF coefficients for phase i in x and y directions. If they are not present, they are set equal to the default up-sampling filter at the same phase position.

Derivation of reference layer sample location is the same as SHM2.0

Additionally results then re-sampling filter coefficients are refined on picture level will be provided. These supplementary data show performance impact of with technology studied in SCE3. Test description for picture level refinement of re-sampling filter coefficients can be found in SCE3 description.

### Test 2.2 “Phase Compensation by signaling filter coefficients at PPS with sample shift”

Goal:

To study the effectiveness of compensating phase offsets between EL and BL by signaling resampling filter coefficients at PPS. Also negative phase offset compensation is signaled by “pixel-shift” flag per filter.

Based on

JCTVC-M0263: “AHG13: SHVC Upsampling with phase offset adjustment” (Arris)

Test sequences

Additional sequences down-sampled using “non-zero” phase shift and specified in section 4.1 shall be used in this test.

Test description

Filter coefficients to be signalled at sequence level.

Signalling of the filter coefficients are done through the syntax in Table 5.

Table 5.  Syntax for signaling of filters with desired phase offsets

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id > 0 && InterLayerTextureRlEnableFlag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| Log2_num_ phase_offsets[ i ] | ue(v) |
| for( j = 0; j < num_ phase_offsets[ i ]; j++ ) { |  |
| luma_pixel_shift_flag[ i ][ j ] | u(1) |
| ref_luma_filter_indx[ i ][ j ] | ue(v) |
| for( k = 0; k < 8; k++ ) { |  |
| delta_luma_filter_coef[ i ][ j ][ k ] | se(v) |
| } |  |
| chroma_pixel_shift_flag[ i ][ j ] | u(1) |
| ref_chroma_filter_indx[ i ][ j ] | ue(v) |
| for( k = 0; k < 4; k++ ) { |  |
| delta_chroma_filter_coef[ i ][ j ][ k ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| . . . |  |
| } |  |

log2_num_phase_offsets[ i ] is used to set the variable num_phase_offset[i] which indicates the number of desired phase offsets for the upsampling process per dimension i. Horizontal dimension is indicated by  i=0 and vertical dimension is indicated by i=1.

luma_pixel_shift_flag[ i ][ j ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_ phase_offsets[ i ]-1), whether the filter is to be applied to shifted input luma samples. When this flag is set to 1, the filter is applied to input samples that are shifted by one pixel; otherwise, input samples are not shifted.

ref_luma_filter_indx[ i ][ j ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_ phase_offsets[ i ]-1), the filter index of one of the four HEVC reference filters used for sub-pixel luma interpolation at 0, 1/4, 1/2 and 3/4 phase offsets.

delta_luma_filter_coef[ i ][ j ][ k ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_phase_offsets[ i ]-1) and filter coefficient index k (k = 0,…, 7), the incremental value which should be added to the corresponding coefficient of the reference filter with index ref_luma_filter_indx[ i ][ j ] to obtain the actual filter coefficients for the current filter phase index.

chroma_pixel_shift_flag[ i ][ j ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_phase_offsets[ i ]-1), whether the filter is to be applied to shifted input chroma samples.  When this flag is set to 1, the filter is applied to input samples that are shifted by one pixel; otherwise, input samples are not shifted.

ref_chroma_filter_indx[ i ][ j ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_phase_offsets[ i ]-1), the filter index of one of the eight HEVC reference filters used for sub-pixel chroma interpolation at 0, 1/8, 1/4, 3/8, 1/2, 5/8, 3/4and 7/8 phase offsets.

delta_chroma_filter_coef[ i ][ j ][ k ] indicates per dimension i (i = 0, 1) and filter phase index j (j = 0,…, num_phase_offsets1[ i ]-1) and filter coefficient index k (k = 0,…, 3), the incremental value which should be added to the corresponding coefficient of the reference filter with index ref_chroma_filter_indx[ i ][ j ] to obtain the actual filter coefficients for the current filter phase index.

Derivation of Reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx,

Output of this process is a sample location (xRefphase, yRefphase ) specifying the reference layer sample location in units of phase grid resolution per dimension relative to the top-left sample of the reference layer picture.

–	The variables xRefphase and yRefphase are derived as follows:

xRefphase = (xP * PicWRL << (log2_num_ phase_offsets[ 0 ]) ) / Scaled

yRefphase = (yP * PicHRL << (log2_num_ phase_offsets[ 1 ]) ) / Scaled

–	The variables xRef and xPhase are derived by

xRef     = ( xRefphase >> (log2_num_ phase_offsets[ 0 ]) )

xPhase = ( xRefphase & (num_ phase_offsets[ 0 ] - 1) )

–The variables yRef and yPhase are derived by

yRef     = ( yRefphase >> (log2_num_ phase_offsets[ 1 ]) )

yPhase = ( yRefphase & (num_ phase_offsets[ 1 ] - 1) )

y_Ref would be added to the top-left sample which is adjusted by luma_pixel_shift_flag[dir][yPhase] or chroma_pixel_shift_flag[dir][yPhase] for the given color component

# Time-line

2013-05-06:	Release of test material with alternative down-sampler

2013-05-16:	Release of reference software SHM2.0 from AHG ‘SHVC software

2013-05-17:	SCE-1 description finalized and uploaded.

2013-05-17:	Release of anchor performance for additional test sequences.

2013-05-24:	Release of reference software SHM2.0 with complexity assessment modules (from Samsung/InterDigital). Release of the Excel template for Complexity evaluation.

2013-06-14:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE coordinators

2013-06-14:	Proponents provide complexity analysis results to cross-verifiers(s) and CE coordinators

2013-06-28:	Cross-verifiers report results to CE participants

2013-07 -12:	Contribution documents uploaded, the document number and titles are reported to coordinators.

2013-07 -25:	Summary report uploaded

# References

[1] J. Dong, Y. He, Y. Ye, "Downsampling filter for anchor generation for scalable extensions of HEVC," m23485, 99th MPEG meeting, San Jose, USA, Feb. 1-10, 2012.

[2] X. Li, J. Boyce, P. Onno, Y. Ye, "Common SHM test conditions and software reference configurations," JCTVC-M1009, 13th JCTVC meeting, Incheon, Korea, April, 2013.