| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M1102r2 |
| --- | --- |

| Title: | Description of Core Experiment SCE2: Combination of inter-layer syntax prediction and motion data compression |  |  |
| --- | --- | --- | --- |
| Status: | Output Document |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Kazushi Sato Christophe Gisquet | Tel: Email: | Kazushi.Sato@jp.sony.com christophe.gisquet@crf.canon.fr |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Core Experiment 2 (SCE2) on combination of inter-layer syntax prediction and motion data compression in SHVC.

# Introduction

During the 13th JCT-VC meeting in Incheon, combinations of tools in SCE5, specifically 5.1.5, 5.2.1 and 5.2.2, were reported. It investigated a good trade-off between coding efficiency and memory size. In addition to SCE5 tools, JCTVC-M0112 “Position derivation for using Base Layer motion in SHVC” and JCTVC-M0258 was proposed. It was then decided to coordinate for further study and testing combinations of those methods in a core experiment.

# Participants

| Participant | Person | Email address | P | C |
| --- | --- | --- | --- | --- |
| Sony | K. Sato X. Jun T. Suzuki A. Tabatabai | Kazushi.Sato@jp.sony.com Jun.Xu@am.sony.com teruhikos@jp.sony.com Ali.Tabatabai@am.sony.com | X | X |
| Canon | C. Gisquet P. Onno G. Laroche E. Francois | christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr guillaume.laroche@crf.canon.fr Edouard.francois@crf.canon.fr | X | X |
| MediaTek | P. Chuang Y.W. Huang S. Liu M. Guo Shih-Ta Hsiang | Peter.Chuang@mediatek.com yuwen.huang@mediatek.com shan.liu@mediatek.com mei.guo@mediatek.com Shih-ta.Hsiang@mediatek.com |  |  |
| I2R | Y. H. Tan | yhtan@i2r.a-star.edu.sg |  |  |
| Sharp | K. Misra A. Segall | misrak@sharplabs.com asegall@sharplabs.com | X | X |
| LG | C. Kim | chulkeun.kim@lge.com |  | X |
| ETRI | J. Lee | jinosoul@etri.re.kr |  | X |

# Experimental Conditions

## Test Sequences, Bit Rates and Coding Conditions

The CE will use the common test condition [1] provided by AhG10 with additionally included SNR test case.

Reference frame insertion framework is used as an anchor.

## Software

SHVC test model SHM2.0 software will be available on May 17th, 2013.

## Evaluation of CE Results

Results of the CE will be evaluated on the basis of BD-measures and complexity. Complexity assessment is performed on the base of decoding complexity measurement.

### Complexity assessment

Both HW and SW complexity of proposed tools for both encoder and decoder will be evaluated.

To measure software run time, software with the anchor configuration and the proposal implemented on the software shall be used. To measure decoding time, the decoding shall be executed on the same machine without output of decoded YUV data. The computational time must be measured for each test sequence and test case for both anchor and proposal. Relative computational time calculated against the anchor must be presented.

Required Memory for storing motion data will be provided like the one in [2].

# Description of Experiments

The combinations of the following tools will be evaluated in this SCE.

Related proposals are:

|  | Method | Contribution | Proponent |
| --- | --- | --- | --- |
| a | BL position rounding | JCTVC-M0112 | Canon |
| b | 2-stage motion data compression (8x8 compressed BL MV) | JCTVC-M0141 | Sony |
| c | Motion field buffer update | JCTVC-M0258 | Sharp |

The tests in this CE are classified into two categories:

- Combination of tools in this SCE

- Single tool test

The following experiments will be conducted and evaluated in SCE2.

| Test | Technique | Proponent | Cross-checker |
| --- | --- | --- | --- |
| Combination |  |  |  |
| 1.1 | a + b | Canon and Sony | Sharp |
| 1.2 | a + c | Canon and Sharp | Sony |
| 1.3 | b + c | Sony and Sharp | LG |
| 1.4 | a + b + c | Canon, Sony and Sharp | LG |
| Single Tools |  |  |  |
| 2.1 | a | Canon | Sony |
| 2.2 | b | Sony | Sharp |
| 2.3 | c | Sharp | ETRI |

# The descriptions of tools

## Inter-layer motion information prediction

### JCTVC-M0112 (Canon)

In the case that motion data of the base layer is compressed by 4:1, the collocated position of the base layer is calculated as

xRL  = ( ( xRL + R ) >> S ) << S

yRL  = ( ( yRL + R ) >> S ) << S

where (R=4, S=4) when tested in SHM1.0 (16x16 motion compression).

Other values of R and S may be more relevant when M0112 proposal is combined with the other proposal from this SCE. Therefore different values of R and S (e.g. R=2 and S=3 for 8x8 compression) will be evaluated for each of the tested combinations.

## Motion data compression and memory reduction

### JCTVC-M0141 (Sony)

2-stage motion data compression as follows: first BL motion data is compressed by 2:1 after encoding/decoding of base layer and then by 2:1 after encoding/decoding of the enhancement layer.

## Motion field buffer update

### JCTVC-M0258 (Sharp)

For the motion field mapped from the base layer the motion information is replaced with motion information from a candidate enhancement layer motion field, if the candidate enhancement layer motion information is valid and does not reference its own base layer. In particular, the motion information includes - prediction mode, motion vector, reference picture POC, reference picture used as long term status.

The replacement process is controlled at the sequence and slice level using syntax elements sps_override_mfm_flag and el_collocated_enabled_flag respectively. The candidate enhancement layer motion field is identified using slice level syntax elements el_collocated_from_flag and el_collocated_ref_idx.

Optionally the proposed tool may be evaluated with combination of other tools within the CE.

# Time-line and Responsibilities

2013-May-17:		SCE2 description finalized and uploaded.

2013-May-17:		Software SHM2.0 distributed by AhG on SHVC Software.

2013-May-17:		Configuration files and anchor performance will be distributed.

2013-May-31:		Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE coordinators.

2013-Jun-09: 		Cross-verifiers report results to CE participants (2 weeks from cross-check start).

2013-Jul-18: 		Contribution documents uploaded, the document number and title are reported to coordinators.

2013-Jul-25:		Summary report uploaded.

# Reference

[1] X. Li, J. Boyce, P. Onno, and Y.Ye, “Common Test Conditions and Software Reference Configurations for the Scalable Test Model”, JCTVC-M1109, April. 2013, Incheon, Korea.

[2] K. Sato, “SCE5: Buffer Size Estimation of SCE5.2.x”, JCTVC-M0142, April 2013, Incheon, Korea