| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 |  |  | Document: JCTVC-M1103_v2 |  |
| --- | --- | --- | --- | --- |
| Title: | Description of HEVC Scalable Extension Core Experiment SCE3: Inter-layer filtering |  |  |  |
| Status: | Output Document of JCTVC |  |  |  |
| Purpose: | Core Experiment Description |  |  |  |
| Author(s) or Contact(s): | Jianle Chen (primary) Andrew Segall Elena Alshina Shan Liu Jie Dong | Tel: Email: |  | +1-858-651-8028 cjianle@qti.qualcomm.com asegall@sharplabs.com elena_a.alshina@samsung.com shan.liu@mediatek.com jie.dong@interdigital.com |
| Source: | CE coordinators |  |  |  |

_____________________________

# Abstract

This document provides the description of HEVC Scalable Extension Core Experiment SCE - 4. The topic of the Core Experiment is testing Inter-layer filters for SHVC.

# Introduction

During the 13th JCT-VC meeting in Incheon, multiple proposals for fixed, adaptive and additional inter-layer filters for the SHVC design were proposed. It was then decided to coordinate for further study and testing of these filters in this Core Experiment (CE).

# Participants List

| Nr. | Name | Company | Email | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Wei Pu Vadim Seregin Jianle Chen Marta Karczewicz Xiang Li | Qualcomm | wpu@qti.qualcomm.com v.seregin@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com lxiang@qti.qualcomm.com | P, C |
| 2 | Andrew Segall Tomoyuki Yamamoto Kiran Misra Jie Zhao | Sharp | asegall@sharplabs.com yamamoto.tomoyuki@sharp.co.jp misrak@sharplabs.com jzhao@sharplabs.com | P, C |
| 3 | Elena Alshina Ankur Saxena Felix Fernandes Alexander Alshin | Samsung | elena.alshina@gmail.com elena_a.alshina@samsung.com asaxena@sta.samsung.com ffernandes@sta.samsung.com alexander_b.alshin@samsung.com | P, C |
| 4 | Shang Liu Mei Guo Polin Lai | MediaTek | shan.liu@mediatek.com mei.guo@mediatek.com polin.lai@mediatek.com | P, C |
| 5 | Jie Dong Yuwen He Yan Ye | Interdigital | jie.dong@interdigital.com yuwen.he@interdigital.com yan.ye@interdigital.com | P, C |
| 6 | Francois Edouard Onno Patrice Laroche Guillaume Gisquet Christophe | Canon | edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr Guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr | P |
| 7 | Madhukar Budagavi Do-Kyoung Kwon | TI | madhukar@ti.com d-kwon@ti.com |  |
| 8 | Chulkeun Kim | LGE | chulkeun.kim@lge.com |  |
| 9 | Yi-jen Chiu Wenhao Zhang | Intel | yi-jen.chiu@intel.com wenhao.zhang@intel.com | P, C |
| 10 | Jinwen Zan Haitao Yang Haoping Yu | Huawei | jinwen.zan@huawei.com haitao.yang@huawei.com haoping.yu@huawei.com |  |
| 11 | Lu, Shuo Ikeda, Masaru Auyeung, Cheung Kazushi SATO Jun, Xu | Sony | shuo.lu@jp.sony.com Masaru.Ikeda@am.sony.com Cheung.Auyeung@am.sony.com Kazushi.Sato@jp.sony.com jun.xu@am.sony.com | C |
| 12 | Tan Yih Han | I2R | yhtan@i2r.a-star.edu.sg |  |
| 13 | Jung Won Kang | ETRI | jungwon@etri.re.kr |  |
| 14 | Woo-Jin Han | Gachon University | hurumi@gmail.com |  |
| 15 | Kemal Ugur | Nokia | Kemal.Ugur@nokia.com |  |
| 16 | Satoru Sakazume Hiroya Nakamura | JVC Kenwood | sakazume.satoru@jvckenwood.com nakamura.hiroya@jvckenwood.com |  |

# Tools to test

The tools in this CE are as follows:

- SCE-3.1 Fixed alternative resampling filter (JCTVC-M0273)

- SCE-3.2 Adaptive upsampling filter (JCTVC-M0267)

- SCE-3.3 Interlayer SAO (JCTVC-M0215)

- SCE-3.4 cross-color inter-layer filter

  - SCE-3.4.1 Chroma samples enhancement by using luma samples (JCTVC-M0183)

  - SCE-3.4.2 Simplified design of cross-color inter-layer filter (JCTVC-M0089)

- SCE-3.5 Bilateral filter

  - SCE-3.5.1 Simplified Separable bilateral filters (JCTVC-M0215)

  - SCE-3.5.2 De-ringing Filter for Inter-layer Prediction (JCTVC-M0055)

- SCE-4.6 Inter-layer Fixed Directional Filtering (JCTVC-M0224)

Summary of the testing tools is provided in the following table.

| Section | Technique | Proponent | Crosschecker |
| --- | --- | --- | --- |
| 3.1 | Fixed alternative resampling filter | Wei, Pu (wpu@qti.qualcomm.com) Elena Alshina (elena_a.alshina@samsung.com) | Jie Dong (jie.dong@interdigital.com) |
| 3.2 | Variable coefficients upsampling filter | Wei Pu (wpu@qti.qualcomm.com) | Elena Alshina (elena_a.alshina@samsung.com) |
| 3.3 | Interlayer SAO | Onno Patrice (Patrice.Onno@crf.canon.fr) | Elena Alshina (elena_a.alshina@samsung.com) |
| 3.4.1 | Chroma samples enhancement by using luma | Jie Dong (jie.dong@interdigital.com) | Xu, Jun (Jun.Xu@am.sony.com) |
| 3.4.2 | Simplified design of cross-color inter-layer filter | Elena Alshina (elena_a.alshina@samsung.com) Jie Dong (jie.dong@interdigital.com) | Xiang Li (lxiang@qti.qualcomm.com) |
| 3.5.1 | Simplified Separable bilateral filters | Jie Zhao (jzhao@sharplabs.com) | Felix C. A. Fernandes (ffernandes@sta.samsung.com) |
| 3.5.2 | De-ringing Filter for Inter-layer Prediction | Felix C. A. Fernandes (ffernandes@sta.samsung.com) | Jie Zhao (jzhao@sharplabs.com) |
| 3.6 | Inter-layer Fixed Directional Filtering | Polin Lai (polin.lai@mediatek.com) | Elena Alshina (elena_a.alshina@samsung.com) |

## Fixed alternative resampling filter (JCTVC-M0273)

This test will study switchable fixed filters for filtering integer positions in SHVC. Integer position filter coefficients are fixed. But filter is switchable. Two methods of switching will be evaluated. In the first one, the encoder selectively enables the fixed alternavie filter for each picture and signals the decision using one bit in slice header. The second scheme, the reconstructed interlayer reference picture and the filtered interlayer reference picture are both inserted into the enhancement reference picture list. The similar alternavie filter for spatial scalability might be also studied in this test. The alternative upsampling filter for spatial scalability is designed as the convolution of the current upsampling filter and the tested integer position filter.

## Variable coefficients upsampling fitler (JCTVC-M0267)

A 2D separable filter used as up-sampling filter with filter coefficients signaled in the bitstream will be tested in this test. The filtering process is also applied to the integer sample positions. As a result, filtering process is used in both SNR and spatial scalability cases.The filter set for inter-layer prediction can be switched between the adaptive filter set and the fixed filter set. A flag is signaled to the decoder at picture level to indicate the choice of the filter.

The short length filter will also be investigated in this test and provided as optional results.

## Interlayer SAO (JCTVC-M0114)

The highfrequency pass inter-layer SAO from JCTVC-M0114 will be tested in this test. This tested method relates to inter-layer filtering to improve the prediction signal as for the denoising filter presented in M0273. Technically speaking it is a simplification of the inter-layer SAO contribution proposed in JCTVC-M0265. The simplification consists in directly applying the principle of the high-pass pre-filtering step of JCTVC-M0265 at the time of the determination of the edge index in the SAO process.

Cross-color inter-layer filter

### Chroma samples e`nhancement filtering by luma samples (JCTVC-M0183)

Fig. 1 shows the relative luma and chroma pixel positions in 4:2:0 color format. Each chroma pixel (red dot) is enhanced by adding an appropriate offset, where the offset is the result of filtering the surrounding 3×4 luma pixels (white squares) by a high-pass filter.

Fig. 1 Relative Luma and Chroma pixel positions in 4:2:0 color subsampling format.

On the encoder side, for each enhancement layer slice, two high-pass filters are designed for Cb and Cr planes, respectively, using Least Minimum Mean Squared Error (LMMSE) estimator. As the solution, the high-pass filters have real-valued coefficients, and therefore need quantizing before transmission. Without the loss of generality, only the processing for Cb is introduced, and for Cr, the processing is the same. Each filter coefficient is quantized by a 16-level uniform quantizer, of which the quantization step size, controlled by two parameters QCb and NCb, is equal to QCb/2NCb. All the filter coefficients and parameters are transmitted in the slice header, including a binary flag indicating whether this tool is used for each chroma plane. Altogether, for each of the two chroma planes, 65 bits are needed to signal the side information when chroma enhancement is enabled:

- 1-bit flag

- 3×4 filter coefficients, each of which with 4-bit precision

- QCb represented by 11 bits (10 bits for magnitude and 1 bit for sign)

- NCb represented by 5 bits

For higher level syntax, one binary flag is added to SPS to indicate whether the proposed tool is used in the current coded video sequence. To enhance a Cb pixel with position (x,y) (take the red circle in Fig. 1 as an example), integer high-pass filter fCb is first applied to the surrounding 3×4 luma pixels to generate the intermediate result z(x,y). Then, z(x,y) is scaled by QCb/2NCb to the normal range, denoted as o(x,y). The enhanced Cb pixel, denoted as Cbenh(x,y), is obtained by Cbenh(x,y)=Cb(x,y)+o(x,y).

### simplified design of cross-color inter-layer filter (test 4.2.4) (JCTVC-M0089)

Cross-color inter-layer filtering will be tested in this experiment. As originally proposed by InterDigital after re-sampling process Chroma samples are refined by adding the offset

(1)

In this test constant shift value =16 will be studied.

Values  and  are signaled in slice-header for Cb and Cr (separately). Additionally tool can be switched On and Off Cb and Cr separately (with corresponding flag encoding in slice header).

If slice level on/off flag for this tool is false then Chroma samples will not be modified. The same will happen if  in (1) is equal to 0. In order to avoid duplicated functionality of syntax elements in a slice header following modification for coding will be tested:

(encoded using 11-bits fixed length code).

Fig. 2. Cross-color inter-layer filter: circles – Luma samples in high resolution, stars –Chroma samples in high resolution, red star – current Chroma sample, Red dots surround Luma samples taken into account for current Chroma sample processing.

In original design proposed by Inter Digital Luma samples in high resolution are used in (1). But this leads to some latency in inter-layer filter implementation as shown on Fig. 3. In order to avoid this  latency Luma samples from reconstructed base layer picture will be used in (1) (as shown in Fig. 4).

RecBL Y

RecBL Cr&Cb

Up-sampled Y

Up-sampled Cr&Cb

Cross-color filter

Inter Layer Texture

Prediction

***Latency

Fig. 3. Original design

RecBL Y

RecBL Cr&Cb

Up-sampled Y

Up-sampled Cr&Cb

Cross-color filter

Inter Layer Texture

Prediction

Fig. 4. Tested design.

## Bilateral filters

### Adaptive up-sampling of base layer picture using Simplified Separable bilateral filters (JCTVC-M0215)

In M0215, an approximation of the upsampling and bi-lateral processes was proposed with significantly fewer multiplies and additions when compared to the sequential application of an upsampler and traditional two-dimensional bi-lateral filter.  In addition, the approximation does not require additional line buffers.  As a first step in the approximation, the bi-lateral filter was embedded into the upsampling framework.  This is achieved by considering a separable operation for both the interpolation and content adaptive components. Secondly, all the dependencies on neighboring pixels that are upsampled was removed, using only integer-pel pixels in the content adaptive process. This allows the bi-lateral filtering to be done in place in the same pass as the upsampling of a pixel. A block diagram of the proposed filter is shown in Figure below.

Fig. 4. Upsampling and Bilateral fitler.

This CE is going to test the proposed Separable bilateral filters in SHM2.0 REF_IDX frame work. When spatial scalability factor is not 1, two versions of upsampled base layer picture are inserted into DPB, and available for inter layer prediction. The first up-sampled base layer corresponds to the fixed up-sampler of SHM-2.0, while a second up-sampled version is generated by combined upsampling and a separable bilateral filter. Alternative DPB insertion strategies may also be considered.

### De-ringing Filter for Inter-layer Prediction (JCTVC-M0055)

The switchable de-ringing filters from JCTVC-M0055 will be tested under RefIDX framework, where up to two versions of the upsampled and filtered BL pictures are placed into DPB of EL to facilitate inter-layer prediction as done in JCTVC-M0273. Two implementations of the 3x3 non-seperable de-ringing filter , devision-based and Lookup-Table based, specified in JCTVC-M0055 will be tested.

In this experiment, a 2D-separable version which mimics the 2D non-seperable de-ringing filter will also be investigated and provided as supplmentary information.

## Inter-layer Fixed Directional Filtering (JCTVC-M0224)

This test will evaluate the inter-layer fixed directional filtering scheme, JCTVC-M0224, presented in the 13th JCTVC meeting. The filtering scheme will be tested under Ref_Idx framework, where up to two versions of the upsampled and filtered BL pictures are placed into DPB of EL to facilitate inter-layer prediction (same as done in JCTVC-M0273). To determine which fixed directional filter to be applied to a given pixel, the local orientation is determined by computing and comparing local gradients.

During the meeting, it was identified that the comparator is relative expensive to implement (refer to meeting note under M0224). Thus a simplifications of the orientation determination and filtering processes, e.g. reducing the number of comparison operations, will be investigated and provided as supplementary information. The simplications includess: 1.) instead of using 3 comparator to distinguish 4 directionalities as in JCTVC-M0224, only identify subset of the directionalities (using fewer comparators) together with subset of the fixed filters corresponding to the reduced directionalities. 2.) Replacing comparator operations by LUT for the computed gradients.

# Experimental Conditions

  - Test Sequences, Bit Rates and Coding Conditions

The SHVC common test conditions defined in JCTVC-M1009 shall be used for this CE. In addition, SNR scalablity case and LD-P test configuration are also mandatory to be tested if the tested method is applicable.

  - Software

Reference software SHM-2.0 will be used as the anchor and base software in this CE.

  - Evaluation of CE Results

Results of the CE will be evaluated on the basis of coding efficiency and complexity.

### Coding efficiency assessment

Coding efficiency will be measured based BD-rate saving based on enhancement layer PSNR and total bit rate of base layer and enhancement layer.

It was generally agreed that visual test might be needed to show the actual benefit of the tested methods in this CE. The sequences and QP points for vsiusal test has not been intensively discussed and could be determined later.

### Complexity assessment

Complexity assessment is performed on the basis of encoding abd decoding complexity measurement. To measure software run time, the software of the proposed method and the anchor software shall be tested on homogenous cluster or a single PC.

In addition, methodologies defined in JCTVC-M0455 is used to evaluate memory bandwidth requirement and computational complexity.

# Time-line and Responsibilities

2013-May-17:	SCE-4 description finalized and uploaded. Release of the Excel template for Complexity evaluation

2013-May-16:	Release of reference software SHM2.0 from AHG ‘SHVC software’

2013-May-24:	Release of reference software SHM2.0 with complexity assessment modules

2013-Jun-07:	Cross-verification begins, proponents provide software, draft of contribution text and results to cross-verifiers(s) and CE coordinators

2013-Jun-14:	Proponents provide complexity analysis results to cross-verifiers(s) and CE coordinators

2013-Jun-28:	Cross-verifiers report results to CE participants

2013-Jul-15:	Contribution documents uploaded, the document number and title are reported to coordinators.

2013-Jul-25:	Summary report uploaded

# References

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference”, JCTVC-M1009, Incheon, South Korea, 18–26 Apr. 2013

- E. Alshina, E. François, “BoG Report on SHVC complexity assessment”, JCTVC-M0455, Incheon, South Korea, 18–26 Apr. 2013