| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Incheon, KR, 18–26 Apr. 2013 | Document: JCTVC-M1122_r1 |
| --- | --- |

| Title: | HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | Rajan Joshi Peter Amon Robert Cohen Sunil Lee Matteo Naccari | Email: | rajanj@qti.qualcomm.com p.amon@siemens.com cohen@merl.com sunil.lee@samsung.com matteo.Naccari@bbc.co.uk |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document is a description of the HEVC Range Extensions Core Experiment 2 on prediction and coding techniques for transform-skip and transform-bypass blocks.

# Introduction

This document defines HEVC Range Extensions Core Experiment 2 (RCE2) on prediction and coding techniques for transform-skip and transform-bypass blocks, to be performed for the upcoming July 2013 JCT-VC meeting.

# Participants

| Participant | Email address | Affiliation | P | C |
| --- | --- | --- | --- | --- |
| R. Joshi J. Sole | rajanj@qti.qualcomm.com joels@qti.qualcomm.com | Qualcomm | X | X |
| M. Budagavi D.-K. Kwon | madhukar@ti.com d-kwon@ti.com | TI | X |  |
| P. Amon | p.amon@siemens.com | Siemens | X |  |
| S. H. Kim A. Segall | kimse@sharplabs.com asegall@sharplabs.com | Sharp | X |  |
| R. Cohen | cohen@merl.com | MERL | X | X |
| A. Minezawa | Minezawa.akira@ds.mitsubishielectric.co.jp | Mitsubishi Electric Corporation |  |  |
| M. Naccari M. Mrak R. Weerakkody | matteo.Naccari@bbc.co.uk marta.mrak@bbc.co.uk rajitha.weerakkody@bbc.co.uk | BBC | X | X |
| S. Lee A. Saxena | sunil.lee@samsung.com asaxena@sta.samsung.com | Samsung |  | X |
| H. Y. Kim | hykim5@etri.re.kr | ETRI |  |  |
| C. Rosewarne | chris.rosewarne@cisra.canon.com.au | Canon |  | X |
| K. Ugur | kemal.ugur@nokia.com | Nokia |  |  |
| J.-Z. Xu | jzxu@microsoft.com | Microsoft |  | X |
| H. Yu J. Ye | haoping.yu@huawei.com jing.ye@huawei.com | Huawei |  | X |
| W. Zhang Z. Deng | wenhao.zhang@intel.com zhipin.deng@intel.com | Intel |  | X |
| E. Wige | wige@lnt.de | Universität Erlangen-Nürnberg | X | X |

# Tools to be tested

## Rice parameter update

Test A.1 (M0304, Sharp)

In this test, cRiceParam signalling method of JCTVC-M0304 will be tested. In the first test, cRiceParameter is signalled from the set {0, 1, 2, 3, 4}. In the second test, cRiceParam is signalled from the subset {0, 2, 4}. .Truncated unary (TU) binarization method is used to code cRiceParameter for both test cases.

Test A.2 (M0316, TI)

In this test, cRiceParam modification of JCTVC-M0316 will be tested. In the first test, cRiceParam fast update with check absCoeff[idx] > 24*(1<<uiGoRiceParam))  will be tested. Additional results with (absCoeff[idx]>>3) > 3*(1<<uiGoRiceParam) will be provided. In the second test, cRiceParam maximum value of 5 will be tested. Combination of both tools will be tested too.

Test A.3 (M0366, Qualcomm)

In this subtest, a modification of the update process for the Rice parameter of the syntax element coeff_abs_level_remaining is evaluated. The proposed modification removes one condition check. For the lossy case, using the proposed update process only for transform-skip blocks is also tested.

## Transform coding changes (rotation, scan, contexts)

Test B.1 (M0333, Qualcomm)

In this subtest, the rotation of the prediction residues for the non-transformed coefficients (transform-skip and lossless) and the usage of a constant context for the significance map for the non-transformed coefficients is investigated. In lossless mode, the rotation is applied to all the blocks. Rotation and constant context are tested separately and in combination. The case where the rotation is restricted to 4×4 blocks in lossless mode is also tested.

Test B.2 (J0313/J0212/J0202, MERL)

This subtest is identical to Test B.1 except that when transform-skip or bypass is used, instead of rotating the coefficient scan order, the horizontal and vertical scans are swapped.

Test B.3 (K0294, BBC)

In this subtest, mirroring of the coefficients is applied over intra-coded blocks when transform-skip is selected. The mirroring depends on the selected coefficient scanning mode. More precisely, when diagonal scan is used the mirroring across the anti-diagonal is applied, otherwise mirroring across the main diagonal (transpose) is applied.

## Prediction improvements

Test C.1 (M0442, BBC)

In this subtest, residual DPCM is applied to inter coded blocks. In particular, three different modes are allowed: no residual DPCM, residual DPCM along the horizontal and vertical direction. For each mode, the SAD over the obtained residues is measured and the mode with the lowest SAD is selected. The chosen mode is transmitted to the decoder. Two levels of signalling are tested: CU and TU level. Furthermore, the method will be also tested for lossy coding in inter blocks where transform-skip is selected as described in Section 3.4 (Test D.1).

Test C.2 (M0351, Qualcomm)

In this subtest, the residual DPCM method as specified for intra-coded blocks is extended to lossy coding conditions. It is applied to horizontal and vertical intra prediction modes in transform-skip blocks.

Test C.3 (M0052, M0193, Siemens, Universität Erlangen-Nürnberg)

In this subtest, template based sample-adaptive weighted prediction as a replacement for the planar mode in conjunction with a sample-based DC prediction mode is evaluated. Further modifications such as removal of division operation, reduction in the size of look-up tables, reduction of size of template/neighborhood will also be tested.

Test C.4 (M0291, Qualcomm)

In this subtest, a replacement for DC prediction mode is evaluated. A DC prediction value is calculated on a sample by sample basis using a causal neighborhood. This method will be tested for both lossless and lossy conditions.

Test C.5 (M0288, Qualcomm)

In this subtest, the residual DPCM method as specified for intra-coded blocks is extended to near-horizontal and near-vertical directions. For these directions, the prediction is performed as in HEVC. But then DPCM is performed on the prediction residual before coding. This method will be tested for both lossless and lossy conditions.

## Combinations

Test D.1 (M0288, M0351 Qualcomm, M0442 BBC)

In this subtest, a combination of C.1, C.2 and C.5 will be tested for lossy coding. Residual DPCM is applied to intra-coded transform-skip blocks and residual DPCM (signalled) is applied to inter coded transform-skip blocks. Two cases will be tested: (1) Residual DPCM is applied to only to horizontal and vertical modes for intra (2) Residual DPCM is applied extended to near-horizontal and near-vertical modes for intra.

Test D.2 (M0288 Qualcomm, J0313/J0212/J0202 MERL)

In this subtest, a combination of B.2 and C.5 will be tested for both lossless and lossy coding. This test affects only intra-coded blocks. The horizontal and vertical scans are swapped for intra-coded transform-skip (lossy) or transform-bypass blocks (lossless). Furthermore, for intra prediction modes using vertical scans, vertical residual DPCM is used and for intra prediction modes using horizontal scans, horizontal residual DPCM is used.

Test D.3 (M0052, M0193 Siemens, M0291 Qualcomm )

In this subtest, a combination of Tests C.3 and C.4 will be tested for lossless coding. Whereas the template based sample-adaptive prediction from Test C.3 replaces the PLANAR mode, the modified sample based DC prediction from Test C.4 replaces the regular DC mode for lossless coding. The tested predictors only affect the intra-coded blocks.

# Cross-checks

Table 1 specifies the cross-checker(s) for each subtest.

| Subtest | Cross-checker(s) |
| --- | --- |
| A.1 | TI, Huawei |
| A.2 | Qualcomm, Huawei |
| A.3 | Sharp |
| B.1 | MERL |
| B.2 | BBC |
| B.3 | Qualcomm |
| C.1 | Samsung |
| C.2 | Huawei |
| C.3 | Intel |
| C.4 | Siemens / Universität Erlangen-Nürnberg |
| C.5 | BBC |
| D.1 | Canon |
| D.2 | Huawei |
| D.3 | MERL |

Table 1: Cross-checks

# Test conditions

## The software, test sequences and test conditions for lossy and lossless configurations are specified in the accompanying document “SCC_Test_Conditions.docx”.

Table 2 specifies whether the lossy and/or lossless coding conditions are used for each subtest.

| Subtest | Coding conditions to be tested |
| --- | --- |
| A.1, A.2, A.3 | Lossy / lossless |
| B.1, B.2, B.3 | Lossy / lossless |
| C.1, C.4, C.5 | Lossy / lossless |
| D.2 | Lossy / lossless |
| C.3, D.3 | Lossless |
| C.2, D.1 | Lossy |

Table 2: Coding conditions to be tested for each subtest

### Evaluation of CE Results

Results of the CE will be evaluated on the basis of bit rate and complexity. Complexity assessment is performed on the base of hardware and encoding/decoding complexity.

### Complexity assessment

To measure software run times, range extension software with the anchor configuration (the default frame level lossless coding is turned on) and the proposals implemented on range extension software shall be used. The computational time must be measured for each test sequence and test case for both anchor and proposals. Relative computational time calculated against the anchor must be presented.

In addition to the simulation time, the following two metrics shall be provided to measure the decoding implementation complexity.

- Number of operations to decode one sample

- Throughput (e.g. number of residual samples which can be decoded in parallel)

Additional evaluation of the HW and SW complexity of the proposed tools for both encoder and decoder is encouraged.

# Timeline

- May 17th 2013: CE description finalized and uploaded.

- May 17th 2013: Range extension reference software available.

- May 17th 2013: Test sequences and test conditions are finalized with AHG8.

- May 24th 2013 (or earlier): Patch for lossless coding is distributed to CE participants.

- Patch distribution date + 2 weeks: Cross-check begins: Proponents for tests A, B, and C provide software, draft specification text and results to CE participants.

- Patch distribution date + 4 weeks: Proponents for test D provide software, draft specification text and results to CE participants.

- July 15th 2013: Input documents and summary report uploaded.