| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0016 |
| --- | --- |

| Title: | JCT-VC AHG report: Single-Loop Scalability (AHG16) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Jill Boyce Madhukar Budagavi Kiran Mishra Kemal Ugur | Email: | wien@ient.rwth-aachen.de jill@vidyo.com madhukar@ti.com misrak@sharplabs.com Kemal.Ugur@nokia.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This report summarizes the activities of the Ad Hoc Group on Single-loop Scalability between the 13th JCT-VC meeting held Incheon, Korea and the 14th JCT-VC meeting held in Vienna, Austria.

# Mandates

- Study and compare single-loop and multi-loop scalability (focus on SNR as the more demanding case).

- Analyze and compare the memory bandwidth of single-loop and multi-loop scalability.

- Study the rate-distortion performance and complexity impact of single-loop scalability using residual refinement techniques and by signalling inter-layer prediction restrictions on a multi-loop design.

- Study and analyze the performance and complexity impact of inter-layer inter prediction tools for single-loop and multi-loop scalability.

# Reflector activity

There AhG used the JCT-VC reflector for discussion.  There was no activity for the AhG on the reflector.

# Related Contributions

## Proposal documents

JCTVC-N0129 	MV-HEVC/SHVC HLS: On single layer for non-IRAP pictures 	[B. Choi, Y. Cho, M.W. Park, J.Y. Lee, H. Wey, C. Kim (Samsung)]

JCTVC-N0161 	Using decoded pictures from higher layer as references in SHVC	[K. Rapaka, J. Chen, M. Karczewicz (Qualcomm)]

JCTVC-N0186 	AHG16: Single-loop decoding of SNR scalability for refIdx based SHVC [X. Xiu, Y. Ye, Y. He, Y. He (InterDigital)]

JCTVC-N0187 	Investigation on using H-ILP picture for refIdx based SHVC [X. Xiu, Y. Ye, Y. He, Y. He (InterDigital)]

JCTVC-N0202 	AHG16: Key picture concept and single loop decoding [C. Feldmann, F. Jäger, M. Wien (RWTH Aachen University)]

## Cross-check documents

JCTVC-N0203 	AHG16: Cross-Check of JCTVC-N0186 Single-loop decoding of SNR scalability for refIdx based SHVC [C. Feldmann, M. Wien (RWTH Aachen University)]

JCTVC-N0297 	AHG16: cross-verification of JCTVC-N0202 on Key picture concept and single loop decoding [X. Xiu (InterDigital)]

# Proposals

## High-Level Summary

Seven contributions are noted to be related to this AHG (JCTVC-N0129, JCTVC-N0161, JCTVC-N0186, JCTVC-N0187, JCTVC-N0202, JCTVC-N0203, JCTVC-N0297).

Document JCTVC-N0129 proposes modifications to the VPS. A new syntax element indicates that all non-IRAP pictures in output layer sets can be decoded in a single loop.

Document JCTVC-N0161 proposes to apply the key-picture concept of SVC in the multi-loop approach of SHVC. Thereby, reconstructed enhancement layer (EL) pictures are used for inter prediction in base layer (BL) pictures. The scheme improves the compression efficiency in the EL. The induced drift if reconstructing only the BL has to be further studied. A similar approach is included in contribution JCTVC-N0202.

Document JCTVC-N0186 proposes a single-loop decoding approach for SHVC. The BL motion information is applied to the EL reference pictures for inter prediction and the BL residual is added in addition to the EL residual. Constrained intra prediction is applied in the base layer. The scheme enables single-loop decoding and does not induce drift into the reconstructed BL pictures. The reported simulation results are verified in contribution JCTVC-N0203.

Document JCTVC-N0187 investigates the performance of multi-loop decoding using the hybrid inter-layer prediction which is proposed in JCTVC-N0186 for SNR scalability. The multi-loop decoding results are obtained by using both the inter-layer prediction (ILP) picture and the hybrid inter-layer prediction (H-ILP) picture for EL prediction. The experimental results show that H-ILP picture can further improve the EL coding efficiency for multi-loop decoding based scheme.

Document JCTVC-N0202 proposes a single-loop decoding approach for SHVC using the key-picture concept as applied in SVC. The reconstructed EL pictures are used for inter prediction in BL non-key-pictures.  BL motion vector coding is applied without reference to EL syntax elements. The proposed scheme induces drift to the reconstructed BL pictures if only the BL is reconstructed. The drift is controlled by periodic insertion of key-pictures (at tid=0). The contribution presents results with multi-loop decoding as well as with single-loop decoding. In the latter case, constrained intra prediction is applied in the base layer. The induced drift if reconstructing only the BL has to be further studied. A similar multi-loop approach is proposed in contribution JCTVC-N0161. The reported simulation results are verified in contribution JCTVC-N0297.

## JCTVC-N0129: MV-HEVC/SHVC HLS: On single layer for non-IRAP pictures

The syntax and semantics modification related to the syntax element single_layer_for_non_irap_flag and a new syntax inter_layer_prediction_disabled_for_non_irap_flag are proposed for single loop decoding of non-IRAP pictures in HEVC multi-layered extensions. When single_layer_for_non_irap_flag is equal to 1, IRAP access units or pictures may have multiple layers while non-IRAP access units or pictures shall have a single layer, in the proposed text. In addition, the syntaxes that can be inferred without signalling when single_layer_for_non_irap_flag is equal to 1 are optionally signaled according to the value of single_layer_for_non_irap_flag. The proposed syntax inter_layer_prediction_disabled_for_non_irap_flag indicates that all non-IRAP pictures in output layer sets can be decoded in a single loop.

## JCTVC-N0161: Using decoded pictures from higher layer as references in SHVC

This contribution proposes a design to enable using the decoded pictures from higher quality layer as reference for lower layer pictures for SNR scalability in SHVC. The document proposes

- Signaling a flag in VPS to specify that immediate higher layer picture is used as reference for lower layer

- The concept of key picture to reduce error drift for base layer decoding

When the SHM 2.0 anchor is modified to enable temporal scalability, the proposed method reports an average of -2.5%, -3.1% and -3.2% luma BD-rate reductions (EL+ BL) for RA-SNR, LD-P and LD-B SNR scalability cases respectively. It is also reported that over SHM 2.0 anchor an average of -2.3%, 0.2% and 1.4% luma BD-rate reductions (EL+ BL) is obtained for RA-SNR, LD-P and LD-B SNR scalability cases respectively.

## JCTVC-N0186: Single-loop decoding of SNR scalability for refIdx based SHVC

This contribution describes a single-loop decoding scheme for SNR scalability of reference index based SHVC. In the current reference index based SHVC, the reconstructed base layer (BL) picture (after up-sampling if necessary) is used for the inter-layer prediction (ILP) of enhancement layer (EL) coding. This implies that multiple motion compensation (MC) operations have to be performed for all dependent layers. In this contribution, a single-loop decoding scheme is achieved by introducing an alternative ILP picture by applying BL motion information on EL temporal reference pictures. In addition, BL residue is added to the alternative ILP picture for quality improvement. As both BL motion information and BL residue could be obtained without the full reconstruction of BL picture, the single-loop decoding requirement is fulfilled by replacing the conventional ILP picture with the alternative ILP picture for EL prediction. Compared to the existing single-loop decoding approaches studied in AHG16, the proposed scheme does not require any low-level changes to single-layer HEVC.

Experimental results shows that compared to SHM 2.0 reference index based multi-loop decoding scheme, the proposed sing-loop decoding scheme can reportedly reduce the average memory access by 34% for block-based implementation and 32% for picture-based implementation, with average BD-rate increase of 1.3% and 1.5% for RA and LDB configurations respectively. In addition, the average decoding time is reduced by 25%. If the combined prediction between the alternative ILP picture and EL temporal reference picture is disallowed for EL prediction, the average decoding time reduction is increased to 28% and the worst case of memory access is the same as single-layer HEVC with BD-rate increase of 2.2% and 3.5%.

## JCTVC-N0187:performance investigation of using H-ILP picture for refIdx based SHVC

The reference index based solution of current SHVC Test Mode (SHM-2.0) is built upon a multi-loop decoding scheme, where reconstructed base layer (BL) picture (after up-sampling if necessary) is used for the inter-layer prediction (ILP) of enhancement layer (EL) coding. One hybrid inter-layer prediction (H-ILP) picture, which is generated using the motion information and the residue information of BL picture and the texture of EL temporal reference picture, is proposed to replace the conventional ILP picture for the single-loop decoding of SNR scalability of SHVC in JCTVC-N0186. This contribution investigates the performance of H-ILP picture based multi-loop decoding by using both ILP picture and H-ILP picture as inter-layer reference pictures for EL prediction. As a part of the investigation, bilinear filter is applied to up-sample the residue of BL picture for the generation of H-ILP picture in the spatial scalability cases. Our investigation shows that H-ILP picture can further improve the performance of SHVC reference index based framework by providing 2.1%, 2.4% and 1.6% BD-rate savings on average for RA, LDB and LDP respectively.

## JCTVC-N0202: Key picture concept and single loop decoding

In this document a performance analysis of the key picture concept in SHVC is presented. The scheme has similarity with MGS as used in SVC. A multi-loop and a single-loop variant are presented. In the multi-loop variant, the enhancement layer performance is reportedly increased by 3.1% BD rate on average. Due to the induced drift, the quality of the reconstructed base layer is reportedly reduced by 4.5% BD rate.

As a modification of the scheme, a variant is presented that adds restrictions to the encoder in order to allow for single loop decoding of the enhancement layer. The enhancement layer performance is reportedly comparable to the multi-loop approach of SHM while the quality of the base layer is reportedly reduced by 0.23 dB BD PSNR on average.

# Recommendations

The AHG recommends to

- study the contributions related to the AHG,

- study the drift impact on reconstructed base layer pictures in case of application of the key-picture concept,

- set up a core experiment on the application of single-loop decoding as well as the application of key-pictures in SHVC.