| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July–2 August 2013 | Document: JCTVC-N0019 |
| --- | --- |

| Title: | Verification test preparation (AHG19) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | TK Tan | Tel: Email: | +65 9799 3861 tktan@m-sph.com |
|  | Vittorio Baroncini | Email: | vittorio@fub.it |
|  | Marta Karczewicz, Wade Wan J. Wen | Email: Email: Email: | martak@qti.qualcomm.com wwan@broadcom.com jtwen@tsinghua.edu.cn |
| Source: | AHG on verification test preparation |  |  |

_____________________________

# Introduction

This AHG was established to prepare the plan for the verification test of the HEVC Version 1 specification.

The mandates of this AHG are:

- Identify test conditions and test sequences for verification of HEVC compression capability, including consideration of use case scenarios for random access and low delay operation

- Identify bit rates, picture resolutions and bit depths appropriate for HEVC verification

- Identify appropriate encoder usage for comparison of HEVC and AVC compression capability

- Communicate with subjective test coordinator to ensure test design validity

- Generate and collect candidate HEVC and AVC encoded bitstreams for HEVC verification testing

# Activities of the AHG

## Reflector

The e-mail reflector is [jct-vc@lists.rwth-aachen.de].  To receive email, please subscribe to the e-mail reflector: http://mailman.rwth-aachen.de/mailman/listinfo/jct-vc. For e-mail exchange, it is recommended to put [AHG19] in the subject line for easy grouping.

# Proposed test conditions.

The following test conditions were proposed in the kick off message for the activities and discussed on the reflector.

- 4 sequences in each of the following resolutions (480p, 720p 1080p and 4K), perhaps 5 for 720p and 480p.

  - Comment: At least one sequence with film grain and other types of noise (e.g. compression noise after a sequences goes through MPEG-2 and/or AVC compression) that is present in some real world sequences.

- Bit depths of 8 bits for 480p, 720p and 1080p, 10 bits for 4K. (so we test both Main and Main 10)

  - Comment: It will be worthwhile to test 4k at 8 bits as well

- To decide on the following parameters once we select the sequences

  - Random access and/or low delay coding options be used depending on the nature of the content that we select.

  - This would cover the streaming/storage and communications use cases, respectively.

  - The bitrate selected should roughly correspond to the bitrate that is used in the industry but adjusted to cover the whole MOS range as much as possible.

    - 4 bit rate points per sequences. Bit rates selection is the most important point for the good outcome of the test.

- To use the HM (latest) for the HEVC and JM18.5 for the AVC encoding.

- To use encoding parameters close to the CC conditions, including fixed QP.

# Identification of Test Sequences

Some sequences were available on the JCTVC FTP site, however many were found to be unsuitable.  There are no 720p or 480p sequences available.

Most of these sequences were found to be unsuitable for performing subjective testing because of one or more of the following reasons:

- Too easy to code.

- Static background with very little moving objects.

- Blur and do not have any high-resolution spatial details.

- Contain too much noise.

Some test sequences were collected and encoding with the HM and JM software were done. This will be available for screening during the JCTVC meeting.

# Other discussions

## “Aligning” lambda parameters that are used by HM and JM.

Comment: One comment on test conditions, which might not be at this point so important, but should also one try to "align" the lambda parameters that are used between the two software?  The HM for example uses a different lambda weight factor (QPfactor) for each picture thus providing additional control and flexibility on RD versus what is done in the JM with fixed lambda parameters. In fact for both low delay B and random access it seems that the "default" implicit lambda computation for B slices is used in the JM, which was "designed" (if it can be called that) mainly for use with non-reference B slices. This can have a severe impact on performance (both subjectively and objectively). Has this been looked at at all?

## Verification of multiple tiles

One member commented that as the parallel processing using Tiles is one of the features of the Main and Main10 profiles of HEVC, should we also test 4k with let us say 4 independent Tiles with no cross Tile processing to test the performance when 4 encoders are used in parallel.

Another member replied that when you test 4 independent Tiles with no cross Tile processing, this needs to be evaluated subjectively too because without cross boundary loop filtering it is very likely that you'll see the tile boundaries. However, as a general comment I think we should not inflate this first verification test by adding the parallel tools because I would then also be interested in testing the 4K performance with wavefronts and so on.

The first member argued that the verification testing will involve subjective viewing. Unlike Tiles, as I understand, Wavefront does not impact in a significant way the video quality. So, Wavefront specific verification testing may not be high on the list and may not be needed. However, as you also mention, that may not be the case with Tiles. The visual quality may depend significantly on some other constraints (on design / architecture) in the presence of Tiles. So, I think, it will be important to verify the performance with Tiles (as it is with the presence or absence of B pictures) under different conditions (e.g., as you mention, with or without cross Tile deblocking and/or others) so that if their presence makes a significant impact on architecture and/or quality, it will be good to verify and understand that. And, that may also provide some helpful guidance to the applications standards bodies and others.

# Recommendations

The AHG recommends

- JCTVC participants to contribute test sequences suitable as subjective test material covering all resolutions of interest.

- to adopt and refine the proposed test conditions.

- to draft a video verification test plan.