| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0031 |
| --- | --- |

| Title: | SCE1: Summary Report of SHVC Core Experiment on support for additional re-sampling phase shifts |  |  |
| --- | --- | --- | --- |
| Status: | CE report input to JCT-VC |  |  |
| Purpose: | CE Summary |  |  |
| Author(s) or Contact(s): | Elena Alshina Xiang Li Jie Dong | Email: | elena_a.alshina@samsung.com lxiang@qti.qualcomm.com Jie.Dong@interdigital.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in SCE3 on support for additional re-sampling filters. Two problems were investigated: enhancement and base layer picture misalignment and relative Luma and Chroma samples misalignment. Solving of both problems requires modification of re-sampling process beyond SHM2.0: additional re-sampling filters are needed. Two types of solutions were studied: fixed predetermined re-sampling filters with additional information about filters choice signaling (so-called category 1 tests) and variable coefficients (so-called category 1 tests).  Tools have been evaluated on the test conditions defined in document JCTVC-M1101.

# List of test methods

Tools listed in the table below were evaluated in this CE. 4 related non-SCE contributions were submitted for this meeting. Non-SCE1 contributions are marked with * in Table 1 below.

Table 1. List of tested tools and cross-checks availability.

| Tests | Title | Tester | Cross-checkers |
| --- | --- | --- | --- |
| Category 1: fixed coefficients of re-sampling filters |  |  |  |
| About relative El and BL pictures displacement |  |  |  |
| 1.1 | Performance with default parameter values | Inter Digital JCTVC-N0182 | Qualcomm |
| 1.2 | The accuracy of down-sampler filter phase | Inter Digital, Qualcomm, Nokia JCTVC-N0182 | Arris JCTVC-N0225 |
| 1.2* | Non-SCE1: Results of test 1.2 on sampling offset signaling with accurate interpolation filter | Samsung JCTVC-N0149 | Qualcomm JCTVC-N0317 |
| 1.2* | Non SCE1: On handling resampling phase offsets with fixed filters | Arris JCTVC-N0272 |  |
| About relative Luma and Chroma samples position |  |  |  |
| 1.3 | Accurate Chroma position alignment | Qualcomm JCTVC-N0045 | Arris JCTVC-N0226 |
| Category 2: variable coefficients of re-sampling filters |  |  |  |
| 2.1 | Phase Compensation by signalling filter coefficients on sequence level | Qualcomm JCTVC-N0046 | Samsung JCTVC-N0308 |
| 2.2 | Phase Compensation by signaling filter coefficients at PPS with sample shift | Arris JCTVC-N0078 | Inter Digital JCTVC-N0193 |
| General comments on re-sampling process |  |  |  |
| * | Non-SCE1: Dynamic range control of intermediate data in re-sampling process | Qualcomm JCTVC-N0214 | Samsung JCTVC-N0218 |
| * | Non-SCE1: On arbitrary spatial ratio scalability in SHVC | Qualcomm, Samsung, Nokia JCTVC-N0219 | MediaTek JCTVC-N0315 |

# Summary of Results

The test conditions for CE1 were specified in JCTVC-M1101, where All Intra, Random Access, Lowdelay-D and Lowdelay-P are all mandatory to be tested. The BD-rate is calculated based on EL+BL bit rates with anchor as SHM 2.0. Additionally performance vs HM10.1 single layer was reported.

Anchor data for non-ctc content was generated by Qualcomm and verified by InterDigital.

Complexity in terms of memory access was analyzed using s/w module provided by AhG17. Both performance and complexity measurement results of all tests are summarized in the Table 2 (for EL and BL picture alignment) and Table 3 (for Luma and Chroma samples alignment).

EL and BL pictures alignment.

In this part of SCE special test (non-ctc sequences) specially generated for SCE1 was used. Down-sampled sequences were generated with displacement between samples grid in high and low resolution pictures as shown on Fig. 1.

Fig. 1 Displacement between EL and BL pictures.

Test results are summarized in Table 2. In Table 2 SHM2.0 tested with non-ctc sequences is an anchor.

All tools from category 1 use fixed (predetermined) interpolation filters set and signalling for down-sampling phase shift (which supposed to be known). Tools from category 2 solve the same problem by signalling filters coefficients (which need to be determined at encoder side).

SHM2.0 re-sampling process was designed assuming no displacement between EL and BL pictures. So for the content was generated using non-zero phase shift in down-sampling SHM2.0 re-sampling is not adequate. Taking into account adjustment between EL and BL samples grid improves performance compare to HM2.0 by 6,6% (×2) – 6,3%(×1,5) in terms of Luma BD-rate gain.

Table 2. Performance and complexity summary for tests on EL and BL pictures alignment.

| Test |  | x2 |  |  |  |  | x1,5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s/w |  | BD-rate |  | Memory |  |  | BD-rate |  | Memory |  |
|  | Luma | U | V | PU | Pic | Luma | U | V | PU | Pic |
| SHM2.0 | 0,00 | 0,00 | 0,00 | 100% | 100% | 0,00 | 0,00 | 0,00 | 100% | 100% |
| Category 1: fixed coefficients of re-sampling filters |  |  |  |  |  |  |  |  |  |  |
| N0182_16 | -6,65 | -5,21 | -5,11 | 97% | 97% | -6,29 | -5,86 | -5,51 | 97% | 98% |
| N0149_16 |  |  |  |  |  | -6,26 | -5,81 | -5,49 | 97% | 98% |
| N0182_8 |  |  |  |  |  | -5,71 | -5,29 | -4,94 | 98% | 98% |
| N0149_8 |  |  |  |  |  | -5,96 | -5,53 | -5,16 | 98% | 98% |
| N0182_4 |  |  |  |  |  | -3,85 | -3,70 | -3,43 | 99% | 99% |
| N0149_4 |  |  |  |  |  | -3,97 | -3,86 | -3,57 | 99% | 99% |
| Category 2: variable coefficients of re-sampling filters |  |  |  |  |  |  |  |  |  |  |
| N0046(Seq) | -6,62 | -5,18 | -5,08 | 97% | 97% | -6,26 | -5,82 | -5,49 | 97% | 98% |
| N0046(Pic) | -6,84 | -5,62 | -5,45 | 97% | 97% | -6,91 | -6,68 | -6,23 | 97% | 97% |
| N0078 | -6,65 | -5,21 | -5,11 | - | - | -6,88 | -6,41 | -5,99 | - | - |

Series of tests with 1/16…1/4 accuracy for down-sampling filter phase shift signalling allows to identify relationship between performance and bits needed for down-sampling filter phase shift signalling. Graph on Fig. 2 shows dependency between size of side information signalling (horizontal axis) and gain achieved in non-ctc content (vertical axis). Blue dots show performance of tests with fixed coefficients; red dots – for adaptive filter solutions.

Figure 2. Bits for down-sampling phase shift signaling vs performance gain.

The Table 3 shows performance and complexity measurement results relatively to HM10.1 single layer decoder. This set of test data is needed to compare 2 scalable systems: zero-phase shift and non-zero phase shift in down-sampling process. Since base layers are different for these 2 systems the comparison using usual methodology is difficult, so comparison with single layer HM10.1 coding (for high-resolution layer, which is the same for both) was used instead.

Performance of HM10.1 single layer codec was independently generated by Qualcomm and Samsung; these data are included into official template for test results report distributed by AhG 10. Memory access measurement for HM10.1 single layer was provided by Samsung and verified by Qualcomm as part of AhG17 activity.

Table 3. Performance and complexity summary vs HM10.1 single layer.

| Test | Test |  | x2 |  |  |  |  | x1,5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| content | s/w | BD-rate |  |  | Memory |  | BD-rate |  |  | Memory |  |
|  |  | Luma | U | V | PU | Pic | Luma | U | V | PU | Pic |
| ctc | SHM2.0 | 21,8 | 31,3 | 31,3 | 105% | 155% | 18,6 | 26,1 | 27,5 | 110% | 160% |
| non-ctc | N0182_16 | 22,2 | 31,5 | 31,7 | 106% | 155% | 19,4 | 26,8 | 28,1 | 112% | 160% |
| non-ctc | N0149_16 |  |  |  |  |  | 19,5 | 26,9 | 28,1 | 112% | 160% |
| non-ctc | N0182_8 |  |  |  |  |  | 20,2 | 27,6 | 28,8 | 112% | 160% |
| non-ctc | N0149_8 |  |  |  |  |  | 19,9 | 27,2 | 28,5 | 112% | 160% |
| non-ctc | N0182_4 |  |  |  |  |  | 22,6 | 29,7 | 30,8 | 113% | 161% |
| non-ctc | N0149_4 |  |  |  |  |  | 22,4 | 29,5 | 30,6 | 113% | 161% |
| ctc | N0045 | 21,8 | 31,5 | 31,7 | 105% | 155% | 18,6 | 26,2 | 27,6 | 112% | 159% |
| non-ctc | N0046(Seq) | 22,3 | 31,6 | 31,8 | 106% | 155% | 19,5 | 26,8 | 28,1 | 112% | 160% |
| non-ctc | N0046(Pic) | 22,0 | 31,0 | 31,3 | 105% | 155% | 18,7 | 25,8 | 27,2 | 112% | 159% |
| non-ctc | N0078 | 22,2 | 31,5 | 31,7 | - | - | 18,7 | 26,1 | 27,4 | - | - |

From Table 3 we can see that the scalable system currently used under ctc (SHM2.0 + zero-phase during down-sampling process) performs the best.

Relative Chroma and Luma samples alignment.

For evaluation the performance impact of taking into account accurate Luma and Chroma samples alignment tests ctc content was used. SHM2.0 re-sampling process takes into account ½ pel vertical displacement between Luma and Chroma samples of ctc sequences (4:2:0 color format). In the test performed by Qualcomm for SCE1 re-sampling for Chroma was done w/o consideration of this displacement.  As shown in Table 4. Performance impact of taking into account accurate Luma and Chroma samples alignment is minor; 0,03…0,05% in terms of Luma BD-rate and 0,05…0,23% in terms of Chroma BD-rate.

Table 4. Performance and complexity summary for tests on relative Luma and Chroma samples alignment.

| Test |  | x2 |  |  |  |  | x1,5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Memory |  | BD-rate |  |  | Memory |  |
| s/w | Luma | U | V | PU | Pic | Luma | U | V | PU | Pic |
| SHM2.0 | 0,00 | 0,00 | 0,00 | 100% | 100% | 0,0 | 0,0 | 0,0 | 100% | 100% |
| N0045 | 0,05 | 0,13 | 0,23 | 100% | 100% | 0,03 | 0,10 | 0,05 | 100% | 100% |

All tables show the average data of AI, LDB, LDP and RA configurations. Memory access is reported comparing to single layer HEVC in Table 3; but relatively to SHM2.0 in Tables 2 and 4.

According to N0226, the ignored phase shift of chroma in N0045 is 1/8 Chroma sample which is not the worst case.

# Brief description of SCE1 tools

The main technical features of each SCE1 test are summarized in the Table 5.

| Test | Test or proposed change description Summary |
| --- | --- |
| 1.1JCTVC-N0182 | - Adding more interpolation filters doesn’t change performance under ctc |
| 1.2 JCTVC- N0182 | - Down-sampling phase shift is signaled in SPS extension and taken into account during re-sampling process - Additional interpolation filters are needed, filters set described in JCTVC-M0465 was used - Experiments with 1/16; 1/8 1nd ¼ accuracy for down-sampling phase shift were performed |
| 1.3 JCTVC-N0045 | - Chroma re-sampling process doesn’t take into account relative displacement between Luma and Chroma samples (while SHM2.0 support this feature) |
| 2.1 JCTVC-N0046 | - Alignment between AL and BL picture is achieved by signaling re-sampling filter coefficients in APS - Both sequence level and picture level adaptation were tested |
| 2.2 JCTVC-N0078 | - Alignment between AL and BL picture is achieved by signaling re-sampling filter coefficients in PPS - For this experiment only one PPS adaptation is used for the entire sequence |

# Details of SCE1 related contributions

Technical features of SCE1 related contributions are summarized in the Table 6.

| Test | Test or proposed change description Summary |
| --- | --- |
| 1.2* JCTVC-N0149 | - The same with test 1.2, but filter set from JCTVC-M0425 was used (only 2 Luma interpolation filters for 1/8 and 7/16 are different, remaining are the same) |
| 1.2* JCTVC-N0272 | - SCE1 test2.2 was modified to use 3/16 and 8/16 and 13/16 filters for 1.5x upsampling of SCE1 BLs. By comparing this test and other SCE1 tests, it was inferred that 0.2% loss of coding efficiency is due to inaccurate phase representation of 16 filter-set and 0.4% loss was due to inaccurate phase-offset signaling. An alternative syntax is proposed to make the signaled phase offset accurate, thus saving 0.4% |
| General comment JCTVC-N0214 | - Re-sampling process is generalized to support both spatial and bit-depth scalability |
| General comment JCTVC-N0219 | - The support of arbitrary scalability ratio is requested for SHVC; - 1/8 phase accuracy in interpolation filters vs 1/16 (in SHM2.0) tested |

# Additional interpolation filters studied

Table 7. SHM2.0 and additional Luma interpolation filters studied in SCE1.

fL[ 16-p, k ] = fL[ p, 7-k ] (0≤k≤7)

| phase p | Luma interpolation filter coefficients |  |  |  |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |  |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 | HEVC |
| 1 | 0 | 1 | -3 | 63 | 4 | -2 | 0 | 0 | SCE1 |
| 2* | 0 | 2 | -6 | 61 | 9 | -3 | 1 | 0 | SCE1 |
| 3 | -1 | 3 | -8 | 60 | 13 | -4 | 1 | 0 | SCE1 |
| 4 | -1 | 4 | -10 | 58 | 17 | -5 | 1 | 0 | HEVC |
| 5 | −1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 | SHM 2.0 |
| 6 | NA | NA | NA | NA | NA | NA | NA | NA | Not tested |
| 7* | -1 | 4 | -11 | 45 | 34 | -10 | 4 | -1 | SCE1 |
| 8 | −1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 | HEVC |

Filters listed in Table 7 were analysed in SCE1 test 1.2 and for several fractional positions more accurate filter design was suggested. Namely for target phase shift 1/8

{-1,     2,    -5,    62,     8,    -3,     1,     0}/64   (1)

and for target phase shift  7/16

{ -1,     4,   -11,    44,    35,   -10,     4,    -1} /64 (2)

were studied in non-SCE1 JCTVC-N0149. Fractional positions where alternative filters were used are marked by * in Table 6.

The filter design principle in JCTVC-N0149 is minimization the difference between frequency response of designed FIR and ideal filter. All filters previously used in HEVC and SHVC design follow this principle. Minimal difference between frequency response of designed FIR and ideal filter leads to minimal error accumulation during interpolation.

Luma interpolation filter for fractional position 6/16 was not used in SCE1 tests.

Non-SCE1 contribution JCTVC-N0219 on arbitrary ratio spatial scalability support uses coefficients from Table 6 except fractional position 1/8 where (1) was used and 6/16 where  not-needed for SCE1 tests

{-1,     4,   -11,    50,    28,   -8,     2,    0}/64   (3)

was used.

Table 8. SHM2.0 and additional Chroma interpolation filters studied in SCE1.

fC[ 16-p, k ] = fC[ p, 3-k ] (0≤k≤3)

| phase p | interpolation filter coefficients |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |  |
| 0 | 0 | 64 | 0 | 0 | HEVC |
| 1 | -2 | 62 | 4 | 0 | SHM 2.0 |
| 2 | -2 | 58 | 10 | -2 | HEVC |
| 3 | -4 | 56 | 14 | -2 | SCE1 |
| 4 | −4 | 54 | 16 | −2 | HEVC |
| 5 | −6 | 52 | 20 | −2 | SHM 2.0 |
| 6 | −6 | 46 | 28 | −4 | HEVC |
| 7 | -4 | 42 | 30 | -4 | SHM 2.0 |
| 8 | −4 | 36 | 36 | −4 | HEVC |

For all SCE1 and SCE1 related contributions this filter set for Chroma was used.