| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0032 |
| --- | --- |

| Title: | SCE2: Summary Report of SHVC Core Experiment on combination of inter-layer syntax prediction and motion data compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Kazushi Sato | Tel: Email: | christophe.gisquet@crf.canon.frKazushi.Sato@jp.sony.com |
| Source: | SCE coordinators |  |  |

_____________________________

# Abstract

This contribution summarizes the activities and test results performed in SCE2 on combination of inter-layer syntax prediction and motion data compression.

The tools have been studied individually and in combination.

# Tests

## Short summary of individual techniques

### Position derivation for using Base Layer motion in SHVC (JCTVC-M0112)

The collocated position of the base layer is calculated as;

xRL  = ( ( xRL + R ) >> S ) << S

yRL  = ( ( yRL + R ) >> S ) << S

where:

- Tests 1.1, 1.4: R=-2, S=3 (compression is 2:1);

- Tests 1.2, 2.1: R=4, S=4 (compression is 4:1, i.e. unchanged)

### Two-stage motion data compression (JCTVC-M0141)

2-stage motion data compression as follows: first BL motion data is compressed by 2:1 after encoding/decoding of base layer and then by 2:1 after encoding/decoding of the enhancement layer.

### Motion buffer modification (JCTVC-M0258)

For the motion field mapped from the base layer the motion information is replaced with motion information from a candidate enhancement layer motion field, if the candidate enhancement layer motion information is valid and does not reference its own base layer. In particular, the motion information includes - prediction mode, motion vector, reference picture POC, reference picture used as long term status.

The replacement process is controlled at the sequence and slice level using syntax elements sps_override_mfm_flag and el_collocated_enabled_flag respectively. The candidate enhancement layer motion field is identified using slice level syntax elements el_collocated_from_flag and el_collocated_ref_idx.

## List of tests

In order to reduce the number of tests, it was agreed with SCE2 participants that for combinations, only one of the proponents may run a test, in addition to the cross-check.

| Test | Technique | Proponent | Cross-checker |
| --- | --- | --- | --- |
| Combination |  |  |  |
| 1.1 | 2.1 + 2.2 | Canon and Sony | Sharp |
| 1.2 | 2.1 + 2.3 | Canon and Sharp | Sony |
| 1.3 | 2.2 + 2.3 | Sony and Sharp | LG |
| 1.4 | 2.1 + 2.2 + 2.3 | Canon, Sony and Sharp | LG |
| 1.5 | 2.3+2.4 | withdrawn |  |
| Single Tools |  |  |  |
| 2.1 | M0112 | Canon | Sony |
| 2.2 | M0141 | Sony | Sharp |
| 2.3 | M0258 | Sharp | ETRI |
| 2.4 | M0141 | withdrawn |  |

# Document list

| Test | Proposal | Cross-check |
| --- | --- | --- |
| 1.1 | JCTVC-N0239 (Sony/Canon), “SCE2: Result of Test 1.1” | JCTVC-N0257 (Sharp) |
| 1.2 | JCTVC-N0302 (Canon/Sharp), “SCE2: Result of Test 1.2” | JCTVC-N0240 (Sony) |
| 1.3 | JCTVC-N0255 (Sony/Sharp), “SEC2 test 1.3 results” | JCTVC-N0300 (LG) |
| 1.4 | JCTVC-N0241 (Sony/Canon/Sharp), “SCE2: Result of Test 1.4” | JCTVC-N0301 (LG) |
| 2.1 | JCTVC-N0139 (Canon), “SCE2: Results on test 2.1” | JCTVC-N0243 (Sony) |
| 2.2 | JCTVC-N0245 (Sony), “SCE2: Result of Test 2.2” | JCTVC-N0259 (Sharp) |
| 2.3 | JCTVC-N0252 (Sharp), “SCE2 test 2.3 motion buffer modification results” | JCTVC-N0122 (ETRI) |

# Summary

The results of the various tests are reported below.

## Results for each test, configuration and ratio

The listed values are an aggregate of BDR gains for all components through formula: (8*gainY + gainU + gainV)/10 for different encoder configurations and spatial scalability factors.

|  | RA_2x | RA_1.5x | RA_SNR | LDB_2x | LDB_1.5x | LDB_SNR |
| --- | --- | --- | --- | --- | --- | --- |
| Test 1.1 | -0.44% | -0.07% | 0.00% | -0.41% | -0.05% | 0.00% |
| Test 1.2 | -0.49% | -0.18% | -0.16% | -0.11% | 0.02% | 0.00% |
| Test 1.3 | -0.75% | -0.22% | -0.11% | -0.41% | -0.01% | 0.08% |
| Test 1.4 | -0.75% | -0.25% | -0.17% | -0.41% | -0.05% | 0.00% |
| Test 2.1 | -0.15% | 0.01% | 0.00% | -0.11% | 0.02% | 0.00% |
| Test 2.2 | -0.44% | -0.04% | 0.05% | -0.41% | -0.01% | 0.08% |
| Test 2.3 | -0.40% | -0.19% | -0.17% | 0.00% | 0.00% | 0.00% |

Optional low delay P results:

|  | LDP_2x | LDP_1.5x | LDP_SNR |
| --- | --- | --- | --- |
| Test 1.1 | -0.38% | -0.08% | 0.00% |
| Test 1.2 | -0.08% | 0.00% | 0.00% |
| Test 1.3 | -0.38% | -0.03% | 0.08% |
| Test 1.4 | -0.38% | -0.08% | 0.00% |
| Test 2.1 | -0.08% | 0.00% | 0.00% |
| Test 2.2 | -0.38% | -0.03% | 0.08% |
| Test 2.3 | 0.00% | 0.00% | 0.00% |

## Average performance across all encoder configurations, all ratios

In the table below, we have :

- The value reported for a component being its gain average for RA, LD-B and LD-P and all ratios;

- The YUV value is an aggregate of above BDR gains for all components through formula: (8*gainY + gainU + gainV)/10.

|  | Y | U | V | YUV |
| --- | --- | --- | --- | --- |
| Test 1.1 | -0.15% | -0.19% | -0.22% | -0.16% |
| Test 1.2 | -0.13% | -0.03% | -0.02% | -0.11% |
| Test 1.3 | -0.21% | -0.14% | -0.17% | -0.19% |
| Test 1.4 | -0.24% | -0.19% | -0.23% | -0.23% |
| Test 2.1 | -0.04% | -0.02% | -0.01% | -0.03% |
| Test 2.2 | -0.12% | -0.14% | -0.15% | -0.12% |
| Test 2.3 | -0.10% | -0.02% | -0.02% | -0.08% |

## Overall results

Previous table can be better visualized with the following chart: