| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0033 |
| --- | --- |

| Title: | SCE3: Summary Report of SHVC Core Experiment on Inter-layer Filtering |  |  |
| --- | --- | --- | --- |
| Status: | CE report input to JCT-VC |  |  |
| Purpose: | CE Summary |  |  |
| Author(s) or Contact(s): | Jianle Chen (primary) Andrew Segall Elena Alshina Shan Liu Jie Dong | Email: | cjianle@qti.qualcomm.com asegall@sharplabs.com elena_a.alshina@samsung.com shan.liu@mediatek.com jie.dong@interdigital.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities and test results performed in SCE3 on inter-layer filtering. Six tools have been evaluated on the test conditions defined in document JCTVC-M1103.

# List of test methods

Six tools listed in the table below were evaluated in this CE. Two tests were withdrawn.

Table 1 summary of SCE3 documents

| Test | Methods | Proposal documents | Cross-checking documents |
| --- | --- | --- | --- |
| 3.1 | Switchable alternative inter-layer filter | JCTVC-N0151 (Qualcomm, Samsung) | JCTVC-N0194 (Interdigital) JCTVC-N0343 (Nokia) |
| 3.2 | Adaptive re-sampling filter | JCTVC-N0047 (Qualcomm) | JCTVC-N0303 (Samsung) |
| 3.3 | Interlayer SAO filtering | JCTVC-N0140 (Canon) | JCTVC-N0304 (Samsung) |
| 3.4.1 | Chroma enhancement for inter-layer prediction | JCTVC-N0184 (Interdigital) | JCTVC-N0251 (Sony) |
| 3.4.2 | Simplified cross-color inter-layer prediction | JCTVC-N0152 (Samsung) | JCTVC-N0208 (Qualcomm) |
| 3.5.1 | Separable Bilateral inter-layer filter | JCTVC-N0220 (Sharp) | JCTVC-N0305 (Samsung) |
| 3.5.2 | De-ringing filter for inter-layer prediction | Withdraw |  |
| 3.6 | Inter-layer fixed directional filtering | Withdraw |  |

# Summary of Results

The test condition for this CE was specified in JCTVC-M1103, where All Intra, Random Access, Lowdelay-D and Lowdelay-P are all mandatory to be tested. The BD-rate is calculated based on EL+BL bit rates with anchor as SHM 2.0. The results of all tests are summarized in the following tables.

. For spatial scalability case, it shows average value of AI, LDB, LDP and RA configurations of 2.0x and 1.5x ratio cases. For spatial SNR case, it shows the average value of LDB, LDP and RA configurations. All the data is calculated using SHM2.0 as anchor.

| Test | Spatial scalability |  |  |  |  |  | SNR scalability |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  | Memory Access (Pic-based) |  | Running time |  | BD-rate |  | Memory Access (Pic-based) |  | Running time |  |
|  | Luma | Chroma | Avg. | Worst | Enc | Dec | Luma | Chroma | Avg. | Worst | Enc | Dec |
| 3.1Pic | −0.1% | 0.0% | 100% | 100% | 100% | 97% | −1.5% | −0.2% | 100% | 100% | 100% | 120% |
| 3.1PU | −0.2% | 0.1% | 137% | 128% | 107% | 123% | −1.9% | −0.1% | 137% | 109% | 103% | 135% |
| 3.2AUF | −0.3% | −0.5% | 100% | 100% | 103% | 100% | −2.0% | −1.4% | 99% | 100% | 101% | 133% |
| 3.3ILSAO | −0.5% | −0.5% | 100% | 105% | 101% | 104% | −1.6% | −1.3% | 99% | 101% | 100% | 112% |
| 3.4.1Chr1 | −0.4% | −8.4% | 100% | 100% | 105% | 109% | −0.3% | −6.5% | 100% | 100% | 107% | 119% |
| 3.4.2Chr2 | −0.4% | −8.2% | 101% | 100% | 99% | 113% | −0.3% | −6.5% | 103% | 100% | 100% | 116% |
| 3.5.1BLF | −0.4% | −0.0% | 143% | 128% | 106% | 136% |  |  |  |  |  |  |

Table 2 coding performance, memory access and running time summary of SCE3 tests

Table 3 shows the memory access data for both picture-based and PU-based interlayer filtering process by using HEVC high layer data as anchor.

| Test | Spatial scalability |  |  |  |  |  | SNR scalability |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  | Memory Access Avg. |  | Memory Access worst |  | BD-rate |  | Memory Access Avg. |  | Memory Access worst |  |
|  | Luma | Chroma | PU | Pic | PU | Pic | Luma | Chroma | PU | Pic | PU | Pic |
| SHM2.0 |  |  | 108% | 157% | 145% | 160% |  |  | 98% | 160% | 200% | 218% |
| 3.1Pic | −0.1% | 0.0% | 107% | 157% | 145% | 160% | −1.5% | −0.2% | 98% | 161% | 200% | 219% |
| 3.1PU | −0.2% | 0.0% | 106% | 218% | 145% | 204% | −1.9% | −0.1% | 97% | 220% | 200% | 237% |
| 3.2AUF | −0.3% | −0.5% | 107% | 157% | 145% | 160% | −2.0% | −1.4% | 97% | 163% | 200% | 220% |
| 3.3ILSAO | −0.5% | −0.5% | 110% | 157% | 145% | 160% | −1.6% | −1.3% | 101% | 159% | 200% | 220% |
| 3.4.1Chr1 | −0.4% | −8.4% | 108% | 158% | 145% | 160% | −0.3% | −6.5% | 99% | 160% | 200% | 219% |
| 3.4.2Chr2 | −0.4% | −8.2% | 109% | 159% | 145% | 160% | −0.3% | −6.5% | 99% | 164% | 200% | 219% |
| 3.5.1BLF | −0.4% | −0.0% | 108% | 228% | 145% | 204% |  |  |  |  |  |  |

Table 3 memory access summary using HEVC high layer as anchor

# Summary of Techniques

The main technical features of each test are summarized in the table below.

Table 4 Technique summary of SCE3

| Test | Technique Summary |
| --- | --- |
| 3.1 JCTVC-N0151 Switchable alternative inter-layer filter | - Low pass smoothing filter on integer Luma samples (5 taps) - Picture level switch on/off - Pu level switch: Both filtered and unfiltered pictures are used as reference - Alternative filter for all phases in Luma resampling process - Picture level switch on/off - Pu level switch: for spatial case, two upsampled inter layer pictures are used as reference |
| 3.2 JCTVC-M0047 Adaptive re-sampling filter | - The adaptive up-sampling filter has same length and coefficient accuracy with the existing up-sampling filter - Filtering process is also applied to integer pixel position - Picture level switch on/off - Filter parameters are signaled at picture level |
| 3.3 JCTVC-N0140 Interlayer SAO filtering | - Edge index is determined by 5 samples instead of 3 samples in the original SAO - Inter layer SAO parameters are coded at slice level. |
| 3.4.1 JCTVC-N0184 Chroma enhancement for inter-layer prediction | - Enhance the Chroma samples by adding a offset which is derived by using the surrounding Luma samples - For each Chroma sample in the interlayer reference picture, applies an adaptive high pass filter to surrounding 4x3 Luma samples of the upsampled base layer picture to derive the offset - The adaptive high pass filter is derived at encoder side for each Chroma plane of a picture - Picture level on/off, filter parameters are signaled at slice header, |
| 3.4.2 JCTVC-N0152 Simplified cross-color inter-layer prediction | - A variant of test3.4.1. The main differences between of test 3.4.1 and test 3.4.2 are - The input of high pass filter of 3.4.1 is the Luma samples of the upsampled base layer picture. - The input of high pass filter of 3.4.2 is the Luma samples of the base layer picture |
| 3.5.1 JCTVC-N0220 Separable Bilateral inter-layer filter | - 3x1 kernel bilateral filter is applied separately for both horizontal and vertical direction - A lookup table is used to replace the final division operation - Both default upsampled picture and bilateral filtered picture are used as interlayer reference pictures |

# List of non-CE proposals

Non-SCE3 proposals are listed in the table below.

Table 5 list of non-SCE3 proposals

| Proposal documents | Proposal Title | Proponents |
| --- | --- | --- |
| JCTVC-N0061 | Non-SCE3.3: Inter-layer interpolation-based SAO filtering for SHVC | A. Filippov, V. Rufitskiy (Huawei) |
| JCTVC-N0070 | Non-SCE3: Inter-layer prediction modes based on base layer sharpness filter | M. Sychev, V. Anisimovskiy, S. Ikonin (Huawei) |
| JCTVC-N0229 | Non-SCE3: Region based Inter-layer Cross-Color Filtering | X. Li, W. Pu, J. Chen, M. Karczewicz (Qualcomm), E. Alshina, A. Alshin, Y. Cho (Samsung) |
| JCTVC-N0250 | Non-SCE3.3: Modified Interlayer SAO with highpass processing | S.-T. Hsiang, C.-M. Fu, Y.-W. Huang, S. Lei (MediaTek) |

# Appendix 1: Result summary of supplementary CE test and non-SCE3 proposals,

Table 6 Results summary of supplementary SCE3 test and non-SCE3 proposals

| Test | Spatial scalability |  |  |  |  |  | SNR scalability |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  | Memory Access (Pic-based) |  | Running time |  | BD-rate |  | Memory Access (Pic-based) |  | Running time |  |
|  | Luma | Chroma | Avg. | Worst | Enc | Dec | Luma | Chroma | Avg. | Worst | Enc | Dec |
| 3.1 no-switch |  |  |  |  |  |  | −1.3% | −0.9% | 101% | 100% | 100% | 126% |
| 3.1Pic-all | −0.3% | 0.0% | 100% | 100% | 100% | 97% | −1.5% | −0.2% | 100% | 100% | 100% | 118% |
| 3.1PU-all | −0.5% | 0.3% | 137% | 128% | 107% | 123% | −1.9% | −0.1% | 137% | 109% | 103% | 133% |
| Non-SCE3.3 ILSAO2 JCTVC-N0250 | −0.4% | −0.8% |  |  | 100% | 108% | −1.7% | −1.4% |  |  | 100% | 114% |
| Non-SCE3.4 Region-base JCTVC-N0229 | −0.7% | −16.2% | 101% | 100% | 100% | 109% | −0.5% | −14.5% | 102% | 100% | 97% | 108% |