| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0034 |
| --- | --- |

| Title: | RCE1: Summary Report of HEVC Range Extensions Core Experiment 1 on Inter-Component Decorrelation Methods |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE Report |  |  |
| Author(s) or Contact(s): | T. Nguyen J. Sole J. Kim | Email: | tung.nguyen@hhi.fraunhofer.de joels@qti.qualcomm.com jungsun.kim@lge.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarises the results of the HEVC Range Extensions (RExt) Core Experiment 1 (RCE1) on inter-component decorrelation methods.

# Experiments

Seven experiments were planned in RCE1 [1] based on three different proposals. In experiment 1, 2, and 3, the proposed schemes were simulated using the latest reference software implementation [2]. A variant of LM chroma (LMC [3]) was analysed in experiment 4 and a variant of the adaptive inter-plane prediction (ILP [4]) was analysed in experiment 5. The combination of the different proposals was tested in experiment 6 and 7.

Experiment 2 and 7 were withdrawn by the proponent and there is no cross check result for experiment 6. A short description of the experiments is given in the following.

- LMC: TU-based parameter derivation

- LMC: CU-based parameter derivation

- ILP: TU-based signalisation of
			α=−1,−0.5,…,−12n

- LMC: Alternative reference sample set
			Provided implementation uses even instead of odd positions from the above 				samples as reference in 4:2:2 sub-sampled content for parameter derivation

- ILP: LMC like variants
			α0 is derived from the left and above neighbours
			αenc is calculated using luma reconstructed and chroma unquantized residuals
			αΔ is transmitted in the bit stream (encoder decision whether αΔ=0)
			αdec=α0+αΔ

- Combination of LMC (Experiment 1) and ILP (Experiment 5)

- Combination of LMC (Experiment 2) and ILP (Experiment 5)

# Results

### Experiment 1: LMC

### Experiment 3: ILP

### Experiment 4: LMC using different sample positions for 4:2:2

### Experiment 5: ILP using LM-based parameter derivation

### Experiment 6: Combination of ILP (LM-based) and LMC

# Observations

- Similar results for 4:4:4 and 4:2:2 YCbCr sequences relative to the previous meeting cycle is observed. However, the performance on screen content is constantly higher than for natural content. A degeneration of the improvement is observed for inter-predicted GOP structures.

- The results are mainly the same as reported by the proponent for RGB content. There are mostly no gain or loss for 4:4:4 YCbCr sequences. In inter-predicted GOP structures, the performance for the first component measured in both colour spaces, i.e., RGB and YCbCr, is higher than in the intra-only configuration.

- The results for 4:4:4 content is exactly the same as in experiment 1. A negligible loss can be observed for 4:2:2 sub-sampled content.

- The results show an improvement for mainly 4:4:4 YCbCr sequences while the performance for RGB content is dropped relative to the scheme in experiment 3.

- Combination of both schemes lead to small improvements for YCbCr relative to experiment 1 and small improvements for RGB content relative to experiment 5. Still the performance for inter-predicted configurations is lower than the performance of the scheme in experiment 3. The combination results in higher performance for screen content.

One participant observed that the bit allocation between the luma and chroma components of the ILP approach (Experiment 3) is mainly a result of the chroma QP offset (QpOffset). The ILP scheme increases the bit depth of the chroma components internally by one for RGB content due to the residual characteristic of the prediction signal. This lead to a higher quantization parameter for the chroma components. The same effect can be achieved by setting the QpOffset for the chroma components equal to 6.

### Experiment 1: LMC with QpOffset equal to 6 for chroma

### Experiment 3: ILP with QpOffset equal to 6 for chroma, without bit depth increase

- The performance for YCbCr is improved relative to experiment 1 for RGB content and the bit allocation between the components are similar to that in experiment 3.

- Similar results can be observed relative to the original experiment 3.

# Complexity

There are two main approach on which the different techniques rely on. The first approach is LM-based and backward-driven and the second approach is forward-driven.

Backward-driven:	The LM-based techniques require a parameter derivation at both decoder and 			encoder. The complexity is increased consequently at both encoder and decoder.

Forward-driven:	The appropriate mode is transmitted in the bit stream. Encoder complexity is 			increased but decoder complexity is mostly maintained.

The encoder run time can be limited for forward-driven schemes by applying an early skip technique. Please note that the reported run times are not always reliable because the simulations were run on different computers with different CPU, HDD, RAM etc.

# Conclusion

#### LM-based (Experiment 1, 4, 5)

- Somewhat more complexity than forward-driven schemes

- Achieve coding improvement for YCbCr sequences (max. 2% and about 1% for inter-predicted GOP structures)

#### Forward-driven (Experiment 3)

- Higher coding improvement in the context of colour space transformation

- Especially for screen content RGB

#### ILP and LMC

- ILP results in higher coding improvement for inter-predicted GOP structures relative to LMC in the context of colour space transformations

- LMC results in coding improvement for YCbCr sequences while ILP doesn’t show any effects

#### Combined Techniques

- Combining LM-based ILP and LMC results in best performance for RGB screen content

- The improvement for YCbCr sequences is maintained and the performance of ILP on RGB content can be achieved

#### QpOffset for chroma

- A QpOffset for chroma results in higher YCbCr performance for all schemes

- For the anchor itself, only a bit allocation effect is observed

# Related Non-RCE1 Documents

Two documents are identified as related to this RCE:

- J. Dong, Y. Ye, and Y. He, “Non-RCE1: In-loop Chroma Enhancement for HEVC Range Extensions,” JCTVC-N0223, Vienna, Austria, July 2013.

- W. Pu, W.-S. Kim, C. Chen, L. Guo, J. Sole, and M. Karczewicz, “Non RCE1: Inter Color Component Residual Prediction,” JCTVC-N0266, Vienna, Austria, July 2013.

# References

- T. Nguyen, J. Sole, J. Kim, “HEVC Range Extensions Core Experiment 1 (RCE1): Inter-Component Decorrelation Methods,” JCTVC-M1121, Incheon, South Korea, April 2013.

- D. Flynn, and C. Rosewarne, “Common Test Conditions and Software Reference,” JCTVC-L1006, Geneva, Switzerland, January 2013.

- J. Kim, B. Jeon, “RCE1: The Performance of Extended Chroma Mode for Non-4:2:0 Format,” JCTVC-M0097, Incheon, South Korea, April 2013.

- T. Nguyen, A. Khairat, D. Marpe, “Non-RCE1/Non-RCE2/AHG5/AHG8: Adaptive Inter-Plane Prediction for RGB Content,” JCTVC-M0230, Incheon, South Korea, April 2013.