| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0036 |
| --- | --- |

| Title: | RCE3: Summary Report of HEVC Range Extensions Core Experiment 3 on Intra Coding Methods for Screen Content |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Liwei Guo | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | CE Coordinator |  |  |

_____________________________

# Introduction

A summary of RCE3 on Intra coding methods for Screen Content for HEVC Range Extensions is reported. Three methods have been evaluated based on the CE description in JCTVC-M1123.

# Document list

## CE contributions and corresponding cross-checks

Test 3.1 Palette Coding Mode

- JCTVC-N0247 (JCTVC-M0323), “RCE3: Results of Test 3.1 on Palette Mode for Screen Content Coding”, L. Guo, M. Karczewicz and J. Sole (Qualcomm)

  - Cross-check: JCTVC-N0104 (Samsung), JCTVC-N0326 (Huawei)

Test 3.2 Multi-stage Base Color and Index Map (MBCIM)

- JCTVC-N0287 (JCTVC-M0330), “RCE3 Test 2: Multi-stage Base Color and Index Map”, W. Zhu (BJUT), J. Xu (Microsoft) and W. Ding (BJUT)

  - Cross-check: JCTVC-N0125 (Huawei)

Test 3.3 Multi-stage Base Color and Index Map (MBCIM)

- JCTVC-N0205 (JCTVC-M0350), “RCE3: Results of test 3.3 on Intra motion compensation”, D.-K. Kwon and M. Budagavi (TI)

  - Cross-check: JCTVC-N0126 (Huawei)

## Non-CE contributions

| Proposal No. | Proposal Title | Proponents |
| --- | --- | --- |
| JCTVC-N0169 | Non-RCE3: Template-based palette prediction | Huawei |
| JCTVC-N0206 | Non-RCE3: Intra motion compensation with variable length intra MV coding | TI |
| JCTVC-N0235 | Non-RCE3: base color merging for MBCIM | SONY |
| JCTVC-N0249 | Non-RCE3: Modified Palette Mode for Screen Content Coding | Qualcomm |
| JCTVC-N0254 | Non-RCE3: Pipeline Friendly Intra Motion Compensation | Qualcomm |
| JCTVC-N0256 | Non-RCE3: 2-D MV Supported Intra Motion Compensation | Qualcomm |
| JCTVC-N0285 | Non-RCE3 : Intra motion compensation for screen contents | Samsung |

# Tested methods

Test 3.1: Palette coding mode (JCTVC-N0247)

For each CU, palettes of maximum size of 4 are first transmitted for each component. The pixels in the CU can be encoded in the following methods: 1. Run-length coding of palette entry index; 2.Copying a number of pixels from the top row; 3. Escape mode (direct transmission of sample values).

Test 3.2: Multi-stage base color and index map (MBCIM) (JCTVC-N0287)

MBCIM denotes a block by several base colors and index map. A pixel will have an index to tell which base color it belongs to. For each pixel, the coding of index uses two predictors, similar to MPM in HEVC intra mode coding. The predictors are selected based on the neighboring reconstructed pixels.

Test 3.3: Intra Motion Compensation (JCTVC-N0205)

Use of Intra Motion Compensation (IMC) is signaled at a CU level. When IMC is enabled for a CU, either horizontal or vertical motion is signaled for the CU. In the first test (test A), horizontal and vertical motions are limited to a range of 0 ~ -M. In the second test (test B), the vertical motion is further limited so that displaced block does not go beyond LCU boundary. Different values of search range M will be test. (CE results was provided with M=64)

# Summary of results

The tested methods were implemented on HM-10.1+RExt-3.0. Test conditions specified in [1] are used. Both lossless and lossy coding results are provided.

Table 1 Results for Lossless Test (Bit-rate reduction)

|  | 3.1 | 3.2 | 3.3A | 3.3B |
| --- | --- | --- | --- | --- |
|  | AI Main |  |  |  |
| Class F | -1.3% | -0.6% | -2.5% | -2.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB444 | -24.9% | -30.3% | -25.2% | -21.5% |
| SC YUV444 | -11.6% | -12.8% | -20.7% | -18.5% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 113% | 112% | 138% | 136% |
| Dec Time[%] | 83% | 104% | 92% | 91% |
|  | Random Access Main |  |  |  |
| Class F | -0.6% | -0.1% | -1.4% | -1.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB444 | -20.0% | -26.0% | -19.9% | -17.0% |
| SC YUV444 | -10.2% | -12.0% | -18.9% | -16.6% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% | 101% | 114% | 110% |
| Dec Time[%] | 98% | 100% | 107% | 103% |
|  | Low delay B Main |  |  |  |
| Class F | -0.2% | -0.1% | -0.7% | -0.6% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB444 | -18.0% | -23.7% | -16.6% | -14.8% |
| SC YUV444 | -9.4% | -12.0% | -17.0% | -16.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% | 101% | 110% | 109% |
| Dec Time[%] | 98% | 102% | 103% | 104% |

Table 2 Results for Lossy Test (Luma BD-rate reduction)

|  | All Intra |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main Tier |  |  |  | High Tier |  |  |  | Super High Tier |  |  |  |
|  | 3.1 | 3.2 | 3.3A | 3.3B | 3.1 | 3.2 | 3.3A | 3.3B | 3.1 | 3.2 | 3.3A | 3.3B |
| Class F | -0.4% | -15.8% | -8.5% | -7.9% | -0.8% | -13.1% | -7.3% | -6.9% | -1.6% | -10.4% | -6.3% | -5.9% |
| Class B | 0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -14.4% | -41.1% | -31.8% | -27.5% | -18.9% | -43.1% | -31.1% | -27.0% | -24.6% | -44.2% | -30.6% | -26.6% |
| SC YUV444 | -3.7% | -26.0% | -26.7% | -24.1% | -5.9% | -25.4% | -26.0% | -23.5% | -9.3% | -26.6% | -25.4% | -23.0% |
| RangeExt | 0.3% | 0.0% | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% | 142% | 133% | 131% | 109% | 140% | 134% | 132% | 108% | 139% | 135% | 133% |
| Dec Time[%] | 93% | 92% | 88% | 90% | 90% | 93% | 88% | 89% | 87% | 94% | 88% | 89% |

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main Tier |  |  |  | High Tier |  |  |  |
|  | 3.1 | 3.2 | 3.3A | 3.3B | 3.1 | 3.2 | 3.3A | 3.3B |
| Class F | -0.4% | -11.3% | -6.2% | -5.8% | -0.7% | -9.0% | -5.5% | -5.1% |
| Class B | 0.2% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| SC RGB444 | -13.1% | -35.1% | -25.9% | -22.2% | -17.2% | -36.9% | -25.0% | -21.5% |
| SC YUV444 | -3.5% | -21.0% | -22.9% | -20.5% | -5.5% | -21.7% | -22.5% | -20.2% |
| RangeExt | 0.2% | 0.0% | -0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% | 105% | 117% | 119% | 104% | 105% | 118% | 120% |
| Dec Time[%] | 102% | 103% | 110% | 117% | 102% | 103% | 111% | 117% |

|  | Low Delay |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main Tier |  |  |  | High Tier |  |  |  |
|  | 3.1 | 3.2 | 3.3A | 3.3B | 3.1 | 3.2 | 3.3A | 3.3B |
| Class F | 0.1% | -7.7% | -3.9% | -3.5% | -0.1% | -6.3% | -3.4% | -3.1% |
| Class B | 0.2% | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% |
| SC RGB444 | -9.1% | -29.6% | -20.1% | -17.3% | -13.7% | -32.6% | -19.6% | -17.0% |
| SC YUV444 | -2.6% | -15.6% | -17.5% | -15.8% | -4.4% | -16.3% | -17.3% | -15.9% |
| RangeExt | 0.3% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% |
| Enc Time[%] | 103% | 104% | 113% | 113% | 103% | 104% | 114% | 104% |
| Dec Time[%] | 103% | 102% | 108% | 112% | 103% | 101% | 109% | 101% |

# Complexity Analysis

| Complexity Analysis | Test 1 (N0247) | Test 2 (N0287) | Test 3 (N0205) |
| --- | --- | --- | --- |
| Number of operation/Sample (typical case) | 1 lookup-table (LUT) operation1 | Not provided | Not provided |
| Number of samples coded in parallel | Fully parallel at encoder and decoder | Not provided | Not provided |
| Size of line buffer | 0 | Not provided | Not provided |

1 1 LUT operation for run-length coding and copy mode; 0 operation for escape mode.

# References

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE 3): Intra Coding Methods for Screen Content”, JCTVC-M1123, Incheon, Korea, Apr. 2013.