| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0042 |
| --- | --- |

| Title: | Restriction on the Residual DPCM block size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Residual DPCM (RDPCM) for horizontal and vertical intra prediction modes has been included in the HEVC Range Extensions design. The method, which is equivalent to a transform, is applied to all block sizes. This contribution reports the results for restricting RDPCM size from 4×4 up to 16×16. It reportedly shows that RDPCM for large block sizes does not contribute coding gains. The performance of restricting RDPCM to 16×16 and 8×8 using AhG8 common conditions for lossless coding is

| RDPCM<=16 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| RA | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| LB | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| RDPCM<=8 |  |  |  |  |  |
| AI | 0.2% | 0.0% | -0.1% | 0.0% | 0.0% |
| RA | 0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| LB | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |

# Introduction

Residual DPCM (RDPCM) for horizontal and vertical intra prediction was adopted at the Incheon meeting. This technique is applied only to lossless coding. It is equivalent to a linear transform of the residue. For example, when the vertical intra mode is applied, the decoder derives the residue r by

ri,j=k=0irk,j   ,0≤i,j≤(N−1)

Where ris the coded residue and N is the block size (from 4 to 32).

RDPCM for large block sizes introduces extra complexity in the design. For larger block sizes, the number of extra additions needed can be substantial. For example, the average number of additions for reconstructing residual rij is (N−1)/2. For a 32×32 block, this amounts to 15.5 additions per sample. In order to limit the number of operations, the calculation can be broken up into smaller horizontal or vertical stripes, as shown in [1]. However, this adds implementation complexity.

# Technical Description and Results

This contribution proposes restricting the usage of RDPCM to the smaller block sizes. Three restrictions are tested: RDPCM usage for blocks up to 16×16, 8×8 and 4×4. The lossless results obtained using common conditions [2] for the three cases are:

| RDPCM <= 16 | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.1% | 0.0% | 0.0% |
| SC YUV 444 | 0.0% | 0.0% | 0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% |

| RDPCM <= 8 | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.2% | 0.1% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.1% | -0.1% | 0.0% |
| SC YUV 444 | 0.0% | -0.1% | 0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% |

| RDPCM <= 4 | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 0.8% | 0.6% | 0.5% |
| Class B | 0.2% | 0.0% | 0.0% |
| SC RGB 444 | -0.1% | -0.1% | -0.1% |
| SC YUV 444 | 0.4% | 0.2% | 0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% |

Meanwhile, the performance of disabling RDPCM for all block sizes is

| RDPCM off | AI | RA | LB |
| --- | --- | --- | --- |
| Class F | 11.4% | 6.7% | 5.3% |
| Class B | 4.6% | 0.9% | 0.6% |
| SC RGB 444 | 15.3% | 10.6% | 10.2% |
| SC YUV 444 | 13.3% | 9.3% | 7.7% |
| RangeExt | 2.5% | 0.7% | 0.6% |

# Conclusions

Restricting RDPCM to block sizes up to 8×8 captures almost all the gain provided by RDPCM without the need of supporting it for large block sizes.

# References

- M. Naccari, M. Mrak, A. Gabriellini, “Complexity reduction for residual DPCM in HEVC lossless coding”, JCTVC-M0439, Incheon, KR, April 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” JCTVC-M1122, Incheon, KR, April 2013.

# Working Draft Text

The text change for RDPCM restricted to a maximum block size equals to 8 is highlighted:

8.4.4.1	General decoding process for intra blocks

….

5.	When cu_transquant_bypass_flag is equal to 1, nTbS is less than 16, and either predModeIntra is equal to 10, or predModeIntra is equal to 26, the directional residual modification process for intra blocks as specified in subclause 8.4.4.3 is invoked with the intra prediction mode predModeIntra, the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

….

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).