| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0043 |
| --- | --- |

| Title: | On software complexity: real time and parallel SHVC video decoder |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | W. Hamidouche, M. Raulet and Ol. Deforges IETR/INSA de Rennes 20 avenue des buttes de Coesmes 35043 Rennes Cedex, FRANCE | Tel: Email: | [phone contact number] wassim.hamidouche@insa-rennes.fr mickael.raulet@insa-rennes.fr |
| Source: | [Name of sourcing organization, AHG, etc.] |  |  |

_____________________________

# Abstract

This contribution provides an open source SHVC software decoder implementing the reference index based SHVC solution. The wave-front parallel processing approach is used to perform in parallel the video decoding of both the base layer and the enhancement one.

Experimental results carried out on a laptop fitted with a core Intel i7 processor show that the proposed software decoder achieves the decoding of 1280×720 base layer and 1920×1080 enhancement layer video sequences at 25 fps when using four concurrent threads.

# Introduction

The scalable extension of the High Efficiency Video Coding (HEVC) is currently being defined jointly by the ITU-T Video Coding Experts Group and the ISO/IEC Moving Picture Experts Group [1]. Two main solutions were proposed in the Scalable High efficiency Coding (SHVC) standard to encode the enhancement layer, namely “reference index” and “textureRL” methods.  In this contribution we have implemented the reference index solution on the top of the OpenHEVC decoder [2]. OpenHEVC is an open source software developed by the IETR laboratory 	aiming to provide a real time HEVC decoder.

The proposed implementation is based on two instances of the OpenHEVC decoder. The first instance decodes the base layer bit-stream and feeds the second instance with the decoded base layer frame. The second instance performs the up-sampling and the scaling of the base layer picture and its motion vectors. The up-sampled base layer is then used by the second Open HEVC instance as a reference frame to decode the enhancement layer video.

In this contribution the wave front parallel processing (WPP) solution is used to perform in parallel the decoding of both the base layer and the enhancement layer videos. The up-sampling of the base layer and the scaling of its motion vectors are carried out by the second OpenHEVC instance on a single thread. In the next section we demonstrate the capability of this software decoder on a laptop fitted with a Core i7 3610QM Intel processor running at 2.3 GHz.

# Experimental results

The SHM2.0 [4] reference software is used to encode three conformance video sequences in main random access configuration. A spatial scalability is considered into two configurations with 1920×1080 resolution for the enhancement layer, 960×540 and 1280×720 resolutions for the base layer, respectively, resulting in ×2 and ×1.5 spatial scalability. Four concurrent threads are used to decode four adjacent Coding Tree Clock (CTB) rows in parallel for both the base layer and the enhancement layer videos.

Table 1 illustrates the decoding performance of the proposed SHVC decoder in terms of decoding frame rate for ×2 spatial scalability configuration. The decoding frame rate with four concurrent threads exceeds 30 fps for the three considered video sequences. The decoding frame rate of the ×1.5 special configuration is provided in Table 2 for the three considered video sequences. For this spatial scalability configuration the decoding frame rate is around 25 fps.

| Video sequences | Layer resolution | QP | Bit rate (Kbps) | Frame rate (fps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original | Actual decoding |
| BasketballDrive | 960×540 | 32 | 1430 | 50 | 30 |
|  | 1920×1080 | 30 | 4195 |  |  |
| BQTerrace | 960×540 | 32 | 882 | 60 | 32 |
|  | 1920×1080 | 30 | 4064 |  |  |
| Cactus | 960×540 | 32 | 1359 | 50 | 33 |
|  | 1920×1080 | 30 | 3983 |  |  |

Table 1 – Performance of the SHVC decoder with four concurrent threads (×2 spatial scalability)

| Video sequences | Layer resolution | QP | Bit rate (Kbps) | Frame rate (fps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Original | Actual decoding |
| BasketballDrive | 1280×720 | 32 | 2052 | 50 | 25 |
|  | 1920×1080 | 30 | 3563 |  |  |
| BQTerrace | 1280×720 | 32 | 1340 | 60 | 26 |
|  | 1920×1080 | 30 | 3707 |  |  |
| Cactus | 1280×720 | 32 | 1980 | 50 | 26 |
|  | 1920×1080 | 30 | 3414 |  |  |

Table 2 – Performance of the SHVC decoder with four concurrent threads (×1.5 spatial scalability)

Figure 1 - Performance of the proposed SHVC decoder

Figure 1 illustrates the decoding performance of the proposed SHVC decoder versus the number of decoding threads. The decoding frame rate slightly increases with the number of threads used for the decoding. This limitation (i.e. low acceleration factor performance) is caused by the up-sampling of the base layer frame and the scaling of its motion vectors, since these two operations are carried out on a single thread. These two operations become the application bottleneck especially when a high number of threads are used for the decoding.

# Conclusion

This contribution shows the performance of the index reference based SHVC decoder. The proposed SHVC decoder achieves a decoding frame rate up to 25 fps for the base layer at 1280×720 resolution and enhancement layer at 1920×1080 resolution when using four concurrent threads. On the other hand, parallel processing of the up-sampling and the motion vectors scaling of the base layer frame can further improve the performance of the proposed SHVC decoder to reach a real time decoding of the considered video sequences (50 fps and 60 fps).

# Acknowledgement

This work is part the European project Hybrid HEVC Broadcast Broadband Video Services (H2B2VS).

# References

[1] Jianle Chen, Jill Boyce, Vidyo, Yan Ye, Miska M. Hannuksela, SHVC Test Model 1 (SHM 1), JCTVC-L1007, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

[2] OpenHEVC decoder: https://github.com/OpenHEVC/openHEVC

[3]  G. J. Sullivan, J. R. Ohm, W. J. Han and T. Wiegand, Overview of the high Efficiency Video Coding standard, IEEE Transactions on Circuits and Systems for Video Technology, vol. 22, no. 12, pp. 1648-1667, December 2012.

[4] SHM reference software: https://hevc.hhi.fraunhofer.de/svn/svn_SHVCSoftware/

# Patent rights declaration(s)

INSA/IETR Rennes may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).