| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0044 |
| --- | --- |

| Title: | RCE2 Test B.1: Residue rotation and significance map context |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

As part of RCE2, this proposal presents the results of the rotation of the residue for non-transformed coefficients (transform skip and lossless) and the usage of a constant context for the significance map. Restricting the block size to which the residue rotation is applied is also tested. Lossy and lossless results for these methods and their combination are provided. For lossless, the BD-rate for restricting the rotation to 4×4 is

|  | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -0.6% | -0.4% | -0.3% | -0.4% | -0.3% |
| RA | -0.4% | -0.3% | -0.1% | -0.2% | -0.3% |
| LB | -0.3% | -0.3% | -0.2% | -0.5% | -0.3% |

And the combination with constant context is

| AI | -1.3% | -0.6% | -0.2% | -0.6% | -0.4% |
| --- | --- | --- | --- | --- | --- |
| RA | -0.9% | -0.5% | -0.3% | -0.6% | -0.4% |
| LB | -0.8% | -0.5% | -0.3% | -0.7% | -0.4% |

For the lossy case, the combination results are

|  | AI RExt Main-tier |  |  | AI RExt High-tier |  |  |  | AI RExt Super-High |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | -2.1% | -1.9% | -2.1% | -2.1% | -2.0% | -2.2% |  | -2.0% | -1.9% | -2.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |  | 0.0% | 0.1% | 0.1% |  |  |  |
| SC RGB444 | -2.8% | -2.6% | -2.5% | -2.6% | -2.5% | -2.3% |  | -2.3% | -2.2% | -2.1% |  |  |  |
| SC YUV444 | -2.7% | -2.6% | -2.3% | -2.6% | -2.7% | -2.4% |  | -2.5% | -2.6% | -2.3% |  |  |  |
| RangeExt | -0.2% | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% |  | -0.2% | 0.0% | -0.1% |  |  |  |
|  | RA RExt Main-tier |  |  | RA RExt High-tier |  |  |  | LB RExt Main-tier |  |  | LB RExt High-tier |  |  |
| Class F | -1.4% | -1.4% | -1.4% | -1.4% | -1.3% | -1.4% |  | -1.0% | -0.3% | -1.0% | -1.0% | -0.6% | -1.2% |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% |  | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| SC RGB444 | -2.1% | -1.8% | -1.7% | -1.9% | -1.6% | -1.6% |  | -1.6% | -1.3% | -1.3% | -1.9% | -1.7% | -1.6% |
| SC YUV444 | -1.9% | -1.9% | -1.6% | -1.7% | -1.7% | -1.4% |  | -0.9% | -0.8% | -0.9% | -1.1% | -1.1% | -1.1% |
| RangeExt | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% |  | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.3% |

# Introduction

A target application for HEVC Range Extensions 4:4:4 is wireless display. This is a consumer application that requires visually lossless coding without the increase in complexity of professional applications. In this application it may be necessary to code screen content, natural content and a mix of both.

In this context, the residue rotation [1] and the constant context for the significance map [2], along with their combination [3], are low complexity methods that provide coding efficiency improvement for both lossy and lossless coding of screen content without affecting the coding efficiency for natural content.

# Technical Description

Residue rotation [1] changes the scan order by rotating a block of residual samples by 180 degrees, which is equivalent to flipping the residue buffer. This implies changing a line of code to reverse the reading of the residue for transform skip blocks:

pResidual[(y * uiStride) + x] =  (plCoef[(y * width) + x] + offset) >> iTransformShift;

is changed to

pResidual[(y * uiStride) + x] = (plCoef[( (height-1- y) * width) + (width -1 - x)] + offset) >> iTransformShift;

And similarly for lossless coding (i.e., flag cu_transquant_bypass_flag enabled):

rpcResidual[k*uiStride+j] = pcCoeff[(uiHeight - 1 - k)*uiWidth + (uiWidth - 1 - j)];

is changed to

rpcResidual[k*uiStride+j] = pcCoeff[k*uiWidth+j];

In a tested variant of the method, residue rotation is only applied to blocks smaller than a given size. Results are provided for a maximum size of 4×4 and 8×8.

In the second method, the significance map for transform skip and lossless is coded with one (additional) shared context [2]. Each 4×4 sub-block uses the same context for all positions. This simplifies the context derivation and separates the statistics of natural content (where transform coding is usually applied) from the screen content (where transform skipping is usually applied), which is especially helpful for mixed content.

# Results

The proposed methods are implemented in HM10.1_Rext3.0 and simulated under the RCE2 common test conditions Error: Reference source not found. The performance is compared to the anchor in terms of BD-rate savings.

## Results for the lossless case

Table 1. Residual rotation performance for losslesss (restricted to 4×4 and to 8×8, and for all sizes)

| Max 4×4 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -0.6% | -0.4% | -0.3% | -0.4% | -0.3% |
| RA | -0.4% | -0.3% | -0.1% | -0.2% | -0.3% |
| LB | -0.3% | -0.3% | -0.2% | -0.5% | -0.3% |
| Max 8×8 |  |  |  |  |  |
| AI | -0.6% | -0.3% | -0.2% | -0.5% | -0.3% |
| RA | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% |
| LB | -0.2% | -0.2% | -0.1% | -0.5% | -0.2% |
| Rotation for all sizes |  |  |  |  |  |
| AI | -0.6% | -0.3% | -0.2% | -0.4% | -0.3% |
| RA | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| LB | -0.2% | -0.1% | 0.1% | -0.4% | -0.1% |

Table 2. Constant context for lossless

|  | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -0.5% | -0.2% | 0.1% | -0.1% | -0.1% |
| RA | -0.4% | -0.1% | 0.0% | 0.0% | -0.1% |
| LB | -0.3% | -0.1% | 0.0% | 0.1% | -0.1% |

Table 3. Combination of residue rotation 4×4 and constant context for lossless

|  | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -1.3% | -0.6% | -0.2% | -0.6% | -0.4% |
| RA | -0.9% | -0.5% | -0.3% | -0.6% | -0.4% |
| LB | -0.8% | -0.5% | -0.3% | -0.7% | -0.4% |

## Results for the lossy case

Table 4. Residual rotation performance for lossy (transform skip 4×4)

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.5% | -1.7% | -1.9% | -1.4% | -1.5% | -1.7% | -1.3% | -1.3% | -1.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.2% | -2.2% | -2.1% | -1.9% | -2.0% | -1.9% | -1.7% | -1.7% | -1.6% |
| SC YUV 444 | -2.4% | -2.4% | -2.2% | -2.2% | -2.4% | -2.2% | -2.1% | -2.2% | -2.0% |
| RangeExt | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 103% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -1.3% | -1.4% | -0.9% | -1.1% | -1.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.6% | -1.5% | -1.4% | -1.5% | -1.4% | -1.3% |  |  |  |
| SC YUV 444 | -1.5% | -1.7% | -1.4% | -1.4% | -1.6% | -1.4% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.8% | -0.9% | -0.4% | -0.6% | -0.7% | -0.6% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.9% | -0.8% | -0.9% | -1.0% | -0.9% | -0.9% |  |  |  |
| SC YUV 444 | -1.1% | -1.0% | -0.9% | -1.0% | -1.0% | -1.0% |  |  |  |
| RangeExt | 0.0% | -0.3% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 103% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |  |  |  |

Table 5. Constant context for lossy

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.1% | -0.7% | -0.8% | -1.1% | -0.9% | -1.0% | -1.2% | -1.0% | -1.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | -0.1% | 0.2% | 0.1% |
| SC RGB 444 | -1.3% | -1.0% | -1.0% | -1.2% | -1.0% | -1.0% | -1.1% | -1.0% | -1.0% |
| SC YUV 444 | -0.8% | -0.6% | -0.5% | -0.8% | -0.6% | -0.6% | -0.8% | -0.7% | -0.6% |
| RangeExt | -0.1% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  | 103% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.7% | -0.4% | -0.4% | -0.7% | -0.4% | -0.5% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -0.8% | -0.5% | -0.5% | -0.9% | -0.7% | -0.7% |  |  |  |
| SC YUV 444 | -0.2% | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% |  |  |  |
| RangeExt | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 96% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | 0.5% | 0.1% | -0.5% | 0.0% | -0.3% |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -0.6% | -0.4% | -0.5% | -0.9% | -0.6% | -0.7% |  |  |  |
| SC YUV 444 | -0.2% | 0.1% | -0.1% | -0.2% | -0.1% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.4% |  |  |  |
| Enc Time[%] | 102% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  |  |  |  |

Table 6. Combination of residue rotation 4×4 and constant context for lossy

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.1% | -1.9% | -2.1% | -2.1% | -2.0% | -2.2% | -2.0% | -1.9% | -2.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| SC RGB 444 | -2.8% | -2.6% | -2.5% | -2.6% | -2.5% | -2.3% | -2.3% | -2.2% | -2.1% |
| SC YUV 444 | -2.7% | -2.6% | -2.3% | -2.6% | -2.7% | -2.4% | -2.5% | -2.6% | -2.3% |
| RangeExt | -0.2% | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 103% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.4% | -1.4% | -1.4% | -1.4% | -1.3% | -1.4% |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| SC RGB 444 | -2.1% | -1.8% | -1.7% | -1.9% | -1.6% | -1.6% |  |  |  |
| SC YUV 444 | -1.9% | -1.9% | -1.6% | -1.7% | -1.7% | -1.4% |  |  |  |
| RangeExt | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 97% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -0.3% | -1.0% | -1.0% | -0.6% | -1.2% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.6% | -1.3% | -1.3% | -1.9% | -1.7% | -1.6% |  |  |  |
| SC YUV 444 | -0.9% | -0.8% | -0.9% | -1.1% | -1.1% | -1.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 104% |  |  |  |  |  |

# Conclusions

Restriction of the residue rotation to 4×4 is a very low complexity method that provides consistent gains for the lossy and lossless methods. Similarly, constant context for non-transformed residual provides gains, and the gains for the combination of both methods are additive.

# References

- D. He, J. Wang and G. Martin-Cocher, “Rotation of Residual Block for Transform Skipping”, JCTVC-J0093, Stockholm, SE, July 2012.

- T. Tsukuba and T. Yamamoto, “Constant coefficient context for intra transform skipping”, JCTVC-J0069, Stockholm, SE, July 2012.

- J. Sole, R. Joshi, M. Karczewicz, “AhG8: Residue rotation and significance map context for screen content coding”, JCTVC-M0333, Incheon, KR, April 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, JCTVC-M1122, Incheon, KR, April 2013.

# Working Draft Text

The change to intra transform skipping in the HEVC working draft is highlighted below.

## Rotational of the residue block

### Scaling and transformation process

…

The (nW)x(nH) array of residual samples r are derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, (nTbS)x(nTbS) array r is set equal to (nTbS)x(nTbS) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ]. the residual sample value rij with i=0..(nTbS)−1, j=0..(nTbS)−1 of the (nTbS)x(nTbS) array r is derived from the (nTbS)x(nTbS) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ] as follows:

rij = transCoeffLevel[ xT ][ yT ][ cIdx ][ nTbS-1-i ][ nTbS-1-j ]

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ nTbS -1-x ][ nTbS -1-y  ]  <<  7 )	(8-1)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

## Restricted rotational of the residue block to 4x4

### Scaling and transformation process

…

The (nW)x(nH) array of residual samples r are derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, (nTbS)x(nTbS) array r is set equal to (nTbS)x(nTbS) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ]. the residual sample value rij with i=0..(nTbS)−1, j=0..(nTbS)−1 of the (nTbS)x(nTbS) array r is derived from the (nTbS)x(nTbS) array of transform coefficients transCoeffLevel[ xT ][ yT ][ cIdx ] as follows:

rij = transCoeffLevel[ xT ][ yT ][ cIdx ][ nTbS-1-i ][ nTbS-1-j ]

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1 and TbS is equal to 4, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ nTbS -1-x ][ nTbS -1-y  ]  <<  7 )	(8-2)

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1 and TbS is greater than 4, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ x ][ y  ]  <<  7 )	(8-3)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

## Significance map context

9.3.4.2.5	Derivation process of ctxInc for the syntax element sig_coeff_flag

Inputs to this process are the colour component index cIdx, the current coefficient scan location ( xC, yC ), the scan order index scanIdx, and the transform block size log2TrafoSize and the transform_skip_flag[ x0L ][ y0L ][ cIdx ].

Output of this process is the variable ctxInc.

The variable sigCtx depends on the current location ( xC, yC ), the colour component index cIdx, the transform block size, and previously decoded bins of the syntax element coded_sub_block_flag. For the derivation of sigCtx, the following applies:

–	If transform_skip_flag[ x0L ][ y0L ][ cIdx ] is equal to 1, sigCtx is derived  as follows:

sigCtx = ( cIdx == 0 ) ? 27 : 15	(9-4)

–	Otherwise, if log2TrafoSize is equal to 2, sigCtx is derived using ctxIdxMap[ ] specified in Table 9 39 as follows:

…

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| … | … | … | … | … | … |
| residual_coding( ) | transform_skip_flag[ ][ ][ 0 ] | Error: Reference source not found | 0 | 1 | 2 |
|  | transform_skip_flag[ ][ ][ 1 ] transform_skip_flag[ ][ ][ 2 ] | Error: Reference source not found | 3 | 4 | 5 |
|  | last_sig_coeff_x_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..53 |
|  | last_sig_coeff_y_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..53 |
|  | coded_sub_block_flag[ ][ ] | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | sig_coeff_flag[ ][ ] | Error: Reference source not found | 0..431 | 442..873 | 884..13125 |
|  | coeff_abs_level_greater1_flag[ ] | Error: Reference source not found | 0..23 | 24..47 | 48..71 |
|  | coeff_abs_level_greater2_flag[ ] | Error: Reference source not found | 0..5 | 6..11 | 12..17 |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).