| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0046 |
| --- | --- |

| Title: | SCE1: Results of Test 1.2.1 on Adaptive Re-Sampling Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li Wei Pu Jianle Chen Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-658-3923 lxiang@qti.qualcomm.com Wei.Pu@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the performance of adaptive re-sampling filter when non-zero phase offset exists. To compensate the phase offset, re-sampling filter coefficients are signaled at sequence level. As supplementary results, performance of filter coefficient update at picture level is also reported.

# Technical description

Adaptive re-sampling filtering was proposed in [1][2]. In addition to coding performance improvement, adaptive re-sampling filter can also be used to compensate non-zero phase offset between base and enhancement layers.

In this test, filter coefficients are signaled in SPS for non-CTC sequences. The filter coefficients, which are derived based on the fixed filter coefficients and phase offsets between the base and enhancement layers, are used as input to the encoder with configuration files.

As supplementary results, performance of coefficient update at picture level is also provided.

# Related syntax table and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| seq_auf_flag | u(1) |
| if(seq_auf_flag) { |  |
| for(i=0;i<16;i++) { |  |
| seq_auf_luma_phase_x[i] | u(1) |
| if(seq_auf_ luma _phase_x[i]) { |  |
| for(j=0;j<8;j++) |  |
| seq_auf_ luma _coeff_x[i][j] | se(v) |
| seq_auf_luma_phase_y[i] | u(1) |
| if(seq_auf_ luma _phase_y[i]) { |  |
| for(j=0;j<8;j++) |  |
| seq_auf_ luma _coeff_y[i][j] | se(v) |
| } |  |
| } |  |
| for(i=0;i<16;i++) { |  |
| seq_auf_chroma_phase_x[i] | u(1) |
| if(seq_auf_ chroma _phase_x[i]) { |  |
| for(j=0;j<4;j++) { |  |
| seq_auf_ chroma0 _coeff_x[i][j] |  |
| seq_auf_ chroma1 _coeff_x[i][j] | se(v) |
| } |  |
| seq_auf_chroma_phase_y[i] | u(1) |
| if(seq_auf_ chroma _phase_y[i]) { |  |
| for(j=0;j<4;j++) { |  |
| seq_auf_ chroma0 _coeff_y[i][j] | se(v) |
| seq_auf_ chroma1 _coeff_y[i][j] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

seq_auf_flag indicates whether adaptive resampling filter is applied for generating inter-layer reference picture to decode the pictures refer to the SPS. seq_auf_flag equal to 1 specifies that adaptive resampling filter is applied. seq_auf_flag equal to 0 specifies that adaptive resampling filter is not be applied. When seq_auf_flag is not present, it is inferred as 0.

seq_auf_luma_phase_x[i], seq_auf_luma_phase_y[i] specify whether luma resampling filter  coefficients are signaled for phase i in x and y directions, respectively.

seq_auf_ luma _coeff_x[i][j], seq_auf_ luma _coeff_y[i][j] specify the luma resampling filter coefficients for phase i in horizontal and vertical directions. If they are not present, they are set equal to the default resampling filter coefficients at the same phase position..

seq_auf_chroma_phase_x[i], seq_auf_chroma_phase_y[i], specify whether chroma resampling filter coefficients is signaled for phase i in horizontal and vertical directions.

seq_auf_ chroma0 _coeff_x[i][j], seq_auf_ chroma0 _coeff_y[i][j], seq_auf_ chroma1 _coeff_x[i][j], seq_auf_ chroma1 _coeff_y[i][j] specify the chroma resampling filter coefficients for phase i in horizontal and vertical directions. When not present, they are set equal to the default resampling filter coefficients at the same phase position.

# Test results

As defined in [4], the proposed method was tested under SHVC common test conditions [3] except that non-CTC sequences with non-zero phase offsets between base and enhancement layers were used. Please note that the timing information is not reliable.

## RD performance of sequence level AUF when compared to SHM2.0

From the results above, it can be concluded that sequence level AUF is able to compensate non-zero phase offsets between base and enhancement layers.

## BD-rate reduction of sequence level AUF when compared to the phase offset signaling SCE1.2 1/16-pixel precision

From the results above, it can be concluded that the coding performance of sequence level AUF is quite close to that of phase offset signaling (1/16 pixel accuracy) studied in SCE1 test 1.2.

## BD-rate reduction of picture level AUF when compared to SHM2.0

As expected, picture level AUF is able to provide better performance when compared to sequence level AUF.

## BD-rate reduction of picture level AUF when compared to the phase offset signaling SCE1.2 1/16-pixel precision

Compared to the phase offset signaling (1/16 pixel accuracy) studied in SCE1 test 1.2, additional coding efficiency was obtained by picture level AUF.

# Conclusions

This test shows that sequence level adaptive re-sampling filter can be used to compensate the non-zero phase offset between base and enhancement layers. The performance of sequence level AUF is quite close to that of signaling phase offset. When picture level adaption is enabled, additional gain can be obtained. In general, AUF is a flexible solution. It not only can compensate phase offset between base and enhancement layers, but also is able to provide better coding performance.

# References

- J. Chen, X. Li, and M. Karczewicz, “TE-B4: Results of test 4.2.1 on adaptive up-sample filter,” JCTVC-L0309, Geneva, Switzerland, Jan. 2013.

- W. Pu, J. Chen, X. Li, and M. Karczewicz, “SCE4: Results of Test 4.2.2 on Adaptive Re-Sampling Filter”, JCTVC-M0267, Incheon, Korea, Apr. 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-M1009, Incheon, Korea, Apr. 2013.

- E. Alshina, X. Li, J. Dong, “HEVC Scalable Extensions Core Experiment 1 (SCE1): Support for additional resampling phase shifts”, JCTVC-M1101, Incheon, Korea, Apr. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).