| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0047_r1 |
| --- | --- |

| Title: | SCE3: Results of Test 3.2 on Adaptive Re-Sampling Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Xiang Li Jianle Chen Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-651-7749 Wei.Pu@qti.qualcomm.com lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This proposal contains performance and complexity analysis of SCE3.2 on adaptive re-sampling filter for SHVC. The coefficients of 2D separable 8-tap re-sampling filter for luma component and 4-tap re-sampling filter for Chroma components are signaled at picture level. The filtering process is applied to both integer and fractional pixel positions. It is reported that compared with SHM2.0 anchor, the proposed method achieves an average luma BD-rate reduction of 0.3% in spatial cases and 2.0% in SNR scalability cases. It is further claimed that the proposed method can also be used to compensate phase offset between base and enhancement layers.

# Technical description

In SHM2.0, the up-sampling process applies fixed 2D separable 8-tap filters to luma fractional sample positions and fixed 2D separable 4-tap filters to chroma fractional sample positions in spatial scalability cases.

Adaptive re-sampling filtering was proposed in [1][2] and was included into SCE3.2 study. The adaptive filter coefficients are derived at the encoder side [2] and signaled to the decoder for each picture. The adaptive filter does not change or introduce additional filtering process so that the decoding complexity of the worst case is kept unchanged. The filter coefficients are coded with Exp-Golomb code in the adaptive picture set (APS) syntax. The dynamic range of filter coefficients is restricted to [-128, 127] at encoder side.

To support the functionality to be tested in SCE1 [4], filter coefficients may also be signaled at sequence level in SPS with the proposed software. However in this test, no filter coefficients are signaled at sequence level.

# Test results

The table below summarizes the RD performance of the tested method in SHVC common test conditions [3]. Compared to SHM2.0, 0.3%, 0.7%, 0.5%, and 1.3% luma BD-rate reduction was obtained for AI, RA, LD-B, and LD-P cases, respectively. Compared to the anchor, slightly lower memory access on average was observed. Full results including complexity evaluation are provided in the attached Excel sheets.

Table 1. BD-rate reduction of SCE3.2 when compared to SHM2.0 Anchor

Table 2 Picture level memory assessment

Table 3 PU level memory assessment

# Conclusions

In this proposal, adaptive re-sampling filter is proposed for interlayer filtering in SHVC. Compared to SHM2.0 anchor, an average of 0.3% and 2.0% luma BD-rate reduction is achieved for spatial and SNR cases, respectively.

# References

- J. Chen, X. Li, and M. Karczewicz, “TE-B4: Results of test 4.2.1 on adaptive up-sample filter,” JCTVC-L0309, Geneva, Switzerland, Jan. 2013.

- W. Pu, J. Chen, X. Li, and M. Karczewicz, “SCE4: Results of Test 4.2.2 on Adaptive Re-Sampling Filter”, JCTVC-M0267, Incheon, Korea, Apr. 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-M1009, Incheon, Korea, Apr. 2013.

- E. Alshina, X. Li, J. Dong, “HEVC Scalable Extensions Core Experiment 1 (SCE1): Support for additional resampling phase shifts”, JCTVC-M1101, Incheon, Korea, Apr. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).