| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, Austria, AT, July 29 – Aug 2, 2013 | Document: JCTVC-N0048 |
| --- | --- |

| Title: | Extensions to support layer addition and removal, access unit structure and changes to HRD model in scalable HEVC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Information and Proposal |  |  |  |  |
| Author(s) or Contact(s): | Sam Narasimhan, Ajay Luthra Arris 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 1-858-404-3985 Sam.narasimhan@arrisi.com Ajay.luthra@arrisi.com |  |  |
|  |  |  |  |  | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |

# Abstract

The current VPS structure in HEVC requires a change to the VPS to signal removal or addition of layers. This contribution recommends addition of a syntax element in VPS to signal presence or absence of layers that are not included in the VPS so that VPS does not have to be altered at re-multiplexing or re-distribution points (see JCTVC-K0206). In addition, this contribution suggests a change to the access unit structure in SHVC.

The contribution also suggests alignment of the systems use case (which requires base layer and enhancement layer combinations to be transmitted in separate streams) with extensions to HRD model so that HRD and STD models can be aligned.

# Introduction and Proposal

The access unit structure in AVC scalability (SVC and MVC) requires non-VCL NAL units (such as SPS extension, SEI etc) for all the enhancement layers to be present in the base layer with a few exceptions. This makes it difficult to add or remove layers after archival or at re-distribution points without a significant change to the underlying video stream. The transport layer for AVC scalability currently allows signaling the presence of some non-VCL NAL unit in the enhancement layer or base layer.

This contribution recommends including layer-specific parameters (including vps_extension, sps_extension, sps_extension_vui_parameters as well as layer specific SEI’s such as HEVC_scalability_SEI) in each of the enhancement layers of SHVC access unit instead of base layer. This access unit ordering may exist in the current SHVC draft as NAL units are ordered based on the nuh_layer_id values. This ordering of access units assists in the removal or addition of enhancement layers without a major change to the underlying video stream.

Even though layer removal is supported in contribution JCTVC-M0043, signaling this when layers are removed from a video stream requires a change to video stream (to include this SEI). This also requires a change to the syntax and structure of VPS as it signals the parameters of all included enhancement layers in the base layer access unit making it difficult to remove or add layers without a change to the video stream. This contribution proposes to adopt the signaling in VPS based on JCTVC-K0206 for content creators that want to enable layer addition or removal later as adding or removal of an enhancement layer to archived content is considered very desirable in many broadcast applications. With the above access unit structure, adding an enhancement layer to archived content is possible in the transport layer where a new PID can be signaled in the PMT carrying the enhancement layer with all its parameters (such as SPS extension and SEI with buffer parameters).

Access unit structure and associated HRD model in AVC scalability used the hypothetical ‘extraction’ model before decoding either base layer or base plus enhancement layers. Applications that use the MPEG-2 systems do not implement ‘extraction’ of access unit parts and instead ‘split up of scalable access unit’ to base layer and enhancement layers for transmission as separate streams within a multiplex. Decoders match this model in their implementations. Decoders collect the streams of interest to them and assemble these before decode as shown in the figure below.

This contribution suggests changing the HRD model from one-stream case where ‘extraction’ is required to multi-stream case where ‘re-assembly’ or combining of relevent streams is done before decode (as shown in figure below).

Base layer

Base dec

Enhancement layer

Combine layers

SHVC dec

# Conclusion

Extensions to VPS syntax to signal layer additions or removal to archived content, SHVC access unit structure and ordering and changes to HRD model are proposed for Scalable HEVC. These changes will greatly improve the functionality of scalable HEVC enabling greater deployment of SHVC in various broadband and broadcast networks.

# Patent rights declaration(s)

Arris may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).