| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, Austria, AT, July 29 – Aug 2, 2013 | Document: JCTVC-N0049 |
| --- | --- |

| Title: | Consideration of buffer management issues HEVC scalability |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Information and Proposal |  |  |  |  |
| Author(s) or Contact(s): | Sam Narasimhan, Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 1-858-404-3985 Sam.narasimhan@arrisi.com Ajay.luthra@arrisi.com |  |  |
|  | Kazushi Sato Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan |  | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |  |  |
|  | Ali Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA |  | 1-408-352-4725 ali.tabatabai@am.sony.com |  |  |
|  | Arris, Sony Corp., and Sony Electronics Inc. |  |  |  |  |

_____________________________

# Abstract

The buffer management in AVC based scalability (SVC and MVC) required an extension to system STD buffer model and introduced an additional layer of complexity to re-purposing and re-distribution equipment. The extensions required management of both the base layer buffer and buffer with base and enhancement layer/layers at the same time in both transmission and decoding equipment. This contribution suggests two options to reduce the buffer model complexity in HEVC based scalability.

# Introduction

## Buffer model considerations

The scalability model as introduced in SVC (and also used in MVC) uses AVC base layer and adds scalable enhancements to achieve temporal, SNR and spatial improvements. Typical SVC applications targetted use cases where bit rate savings were achieved over simulcast of ‘base layer and enhancement without scalability’. MVC provided view enhancements using a similar model.

Some of the application use cases required transmission of AVC base layer stream to base layer decoder while the enhancement layer was was transmitted seperately where the SVC/MVC decoder assembled the base and enhancement layers before decoding the enhanced data.

Base layer

AVC dec

Enhancement layer

Combine

SVC dec

The base layer video stream may use different profile and level than the [base + enhancement layer] (uaually this being higher level than base layer) and each required a different CPB size and bit rate for HRD buffer management. The video standard (and in turn systems) did not specify an independent buffer size and rate for each enhancement layer and the specification defined a scheme where the base layer and [base+enhancement layer] buffers had to be managed in parallel to make sure they did not overflow or underflow. This management was required at both the encoding end as well as decoders. The HRD model for SVC and MVC did not address such parallel CPB management of base layer and [base+enhancement layer]. The HRD model defined how to extract base layer video from the full SVC/MVC access unit and then manage the base layer.

The transport part of SVC/MVC had to support the STD management independently for base and [base+enhancement layer] as STD does not have the ability to extract base layer data from full SVC access units and this capability would have required the base layer decoders to support the larger buffer sizes that included the base and enhancement layer data. The STD extension in systems requires a virtual re-assembly buffer and the management of multiple buffers after re-assembly (see figure below). This has made the STD more complex for SVC and MVC transport and makes re-multiplexing very difficult,  complex and not implementable with legacy re-multiplexers. The defined compression of enhancement layer required variable bit rate and buffer allocation between base layer and enhancement layer to achieve best results. For example, if it was required to use a higher buffer size and rates for base layer, the equivalent was adjusted in enhancement layer and vice versa. However, the buffer size and rates for base layer or [base+enhancement] layer could never exceed the maximum limits for the level specified.

The following diagram shows an example of the system buffer model for HEVC layered coding and this is the same as that specified for SVC and MVC.

## Proposals to simplify buffer management issues in HEVC scalability

HEVC scalability includes additional use cases than just ‘simulcast’ and is expected to be more efficient in compression of enhancement layers.  In all these cases, it will be difficult to manage base layer and [base+enhancment layer] buffers in parallel to achieve conformant system buffer behaviour if HEVC scalability adopts a similar HRD buffer model as AVC scalability. This contribution proposes two schemes to simplify system buffer management where the parallel buffer management can be avoided - use of a new SEI and an extension to HRD parameters.

HEVC_scalability SEI: The proposed SEI (based on SVC scalability_info_SEI or MVC view_scalability SEI) signals the maximum bit rate, average bit rate and maximum CPB buffer size allocated for each of the layers in the video stream. This can also be mapped into the system layer using a descriptor and allows implementation of independent single buffer STD model in systems as well as HRD for base and enhancement layers.

HRD extensions: Currently, HEVC HRD parameters can be defined either as whole-sequence level or sub-stream level. There might be the cases where system buffer can be managed for each layer/view independently without having to re-assemble the lower layer/views, but current HRD specification does not support such scenario. This proposal uses the scalable HEVC extension in vps_extension() and sps_extension() to convey HRD parameters for a single-layer model or multi-layer model (like SVC or MVC) or both.

These proposals are expected to simplify the complexity of STD buffer management for HEVC and make is easy to implement HEVC in deployed AVC/MPEG-2 networks (as legacy re-multiplexers can be used for re-purposing HEVC content).  The buffers for base and enhancement layers can also be managed independently before re-assembly and this simplifies the STD model. Note that re-assembly is still required before decoding an enhanced HEVC video stream.

# Proposals

## HEVC scalability SEI

This proposal recommends new suffix SEI to HEVC to signal layer specific information such as maximum bit rate, average bit rate and maximum CPB buffer size. The following table provides an example of the SEI message syntax where the parameters can be signaled for a single layer (vps_max_layers_minus1=0) or multiple layers.

- HEVC Scalability information SEI message syntax

| HEVC_scalability_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| vps_max_layers_minus1 | 5 | ue(v) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| layer_id[ i ] | 5 | ue(v) |
| dependency_id[ i ] | 5 | u(3) |
| quality_id[ i ] | 5 | u(4) |
| temporal_id[ i ] | 5 | u(3) |
| profile_level_info_present_flag[ i ] | 5 | u(1) |
| bitrate_and_cpb_info_present_flag[ i ] | 5 | u(1) |
| frm_rate_info_present_flag[ i ] | 5 | u(1) |
| layer_dependency_info_present_flag[ i ] | 5 | u(1) |
| parameter_sets_info_present_flag[ i ] | 5 | u(1) |
| if( profile_level_info_present_flag[ i ] ) |  |  |
| layer_profile_level_idc[ i ] | 5 | u(24) |
| if( bitrate_and_cpb_info_present_flag[ i ] ) { |  |  |
| avg_bitrate[ i ] | 5 | u(16) |
| max_bitrate_layer[ i ] | 5 | u(16) |
| max_bitrate_layer_representation[ i ] | 5 | u(16) |
| max_cpb_size_layer[i] | 5 | u(16) |
| max_bitrate_calc_window[ i ] | 5 | u(16) |
| } |  |  |
| if( frm_rate_info_present_flag[ i ] ) { |  |  |
| constant_frm_rate_idc[ i ] | 5 | u(2) |
| avg_frm_rate[ i ] | 5 | u(16) |
| } |  |  |
| if( layer_dependency_info_present_flag[ i ] ) { |  |  |
| num_directly_dependent_layers[ i ] |  |  |
| for( j = 0; j < num_directly_dependent_layers[ i ]; j++ ) |  |  |
| directly_dependent_layer_id_delta_minus1[ i ][ j ] |  |  |
| } else |  |  |
| layer_dependency_info_src_layer_id_delta[ i ] |  |  |
| if( parameter_sets_info_present_flag[ i ] ) { |  |  |
| num_seq_parameter_set_minus1[ i ] |  |  |
| for( j = 0; j <= num_seq_parameter_set_minus1[ i ]; j++ ) |  |  |
| seq_parameter_set_id_delta[ i ][ j ] |  |  |
| num_subset_seq_parameter_set_minus1[ i ] |  |  |
| for( j = 0; j <= num_subset_seq_parameter_set_minus1[ i ]; j++ ) |  |  |
| subset_seq_parameter_set_id_delta[ i ][ j ] |  |  |
| num_pic_parameter_set_minus1[ i ] |  |  |
| for( j = 0; j <= num_pic_parameter_set_minus1[ i ]; j++ ) |  |  |
| pic_parameter_set_id_delta[ i ][ j ] |  |  |
| } else |  |  |
| parameter_sets_info_src_layer_id_delta[ i ] |  |  |
| } |  |  |

## HRD extensions

In Fig 2.2.1, the bitstream contains 2 layers or views. It is possible that whole bitstream be decoded with one decoder (scenario 1) or each layer can be decoded with separate decoders (scenario 2).

Fig 2.2.1

With 2-decoder model case, each layer is decoded as shown in Fig 2.2.2.

Fig 2.2.2

hrd_parameters() need to be defined differently with scenario 1 and 2: in scenario 1 parameters of both BL and EL should be included within one hrd_parameters() while in scenario 2 parameters of BL and EL should be included in separate hrd_parameters(). However in the current hrd_parameters() syntax specification is allowed to be specified for each layer set, which means that only scenario 1 is supported.

To solve this problem it is proposed to modify syntax so that scenario-2-type hrd_parameters() can be optionally transmitted in vps_extension() and sps_extension().

- vps_extension() syntax

| vps_extension () { | Descriptor |
| --- | --- |
| … |  |
| for( i=0; i <= vps_num_layer_sets_minus1; i++) |  |
| for( j=1; j <= vps_max_layer_id; j++) |  |
| if( layer_id_included_flag[ i ][ j ] ) { |  |
| layer_set_hrd_layer_info_present_flag[ i ][ j ] | u(1) |
| if( layer_set_hrd_layer_info_present_flag[ i ][ j ]) { |  |
| hrd_parameters( cprm_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| … |  |
| } |  |

- sps_extension syntax

| sps_extension () { | Descriptor |
| --- | --- |
| … |  |
| layer_set_hrd_layer_info_present_flag | u(1) |
| if( layer_set_hrd_layer_info_present_flag) { |  |
| hrd_parameters( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| … |  |
| } |  |

layer_set_hrd_layer_info_present_flag equals to 1 specifies that hrd_parameters() that is applied just to the current layer which is a part of layer set is additionally contained in the CVS, and equals to 0 specifies that hrd_parameters() that is applied just to the current layer which is a part of layer set is not contained.

# Conclusion

Two schemes are proposed for transmission of individual buffer parameters for HEVC scalable layers so that the system buffer management can be simplified. The first scheme proposes a new SEI to transmit the information while the second scheme proposes extensions to HRD parameters in vps_extension() and sps_extension(). It is recommended that both these signaling schemes be adopted into HEVC specification.

# Patent rights declaration(s)

Arris may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corp. and Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).