| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 27 Jul. – 2 Aug. 2013 | Document: JCTVC-N0051 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 Jul. – 2 Aug. 2013 | Document: JCT3V-E0038 |

| Title: | MV-HEVC/SHVC HLS: ViewId and view position index |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce 433 Hackensack Ave Hackensack, NJ, 07601, USA | Tel: Email: | +1 201 478 6145 jill@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

This contribution proposes reversing the VPS extension syntax related adoption of JCT3V-D0220, because of implications to the joint SHVC high-level syntax. JCT3V-D0220 changed the meaning of the ViewId variable, from an indication of the view coding dependency order to an indication of the relative view camera position, so that ViewId is no longer required to be monotonically increasing, and is not required to be equal to zero in the base layer.  It is proposed that ViewId indicate the video coding dependency order, and to add an additional view_position_idx[ i ] syntax element in the VPS extension to indicate the relative view camera positions, which may be used for reference picture list construction.

# Discussion

The JCT3V-D0220 contribution violated a fundamental design philosophy of the VPS extension’s dimension_id[ i ][ j ] syntax elements, which are mapped to ViewId, DependencyId, DepthMapFlag, etc.  In the original design, these scalability dimension values were all required to be equal to 0 in the base layer.  These scalability dimension variable values were also required to be monotonically increasing, and hence could be easily used for sub-bitstream extraction.

In the JCT3V-D0220 contribution, the value of ViewId corresponding to the base layer may be non-zero. However, when splitting_flag is equal to 1, the scalability dimension variable values are inferred from the bit fields of the nuh_layer_id syntax element.  In the base layer, nuh_layer_id is required to be equal to 0, and hence for the base layer all of ViewId, DependencyId, etc. would be required to be equal to zero when splitting_flag is set.  This splitting_flag feature can therefore not be used with the JCT3V-D0220 adoption.

It is proposed that the portion of the VPS extension syntax table related to the dimension_id[ i ][ j ] syntax elements revert to the design prior to the integration of JCT3V-D0220.  However, to accomplish the goal of JCT3V-D0220 to have relative camera view position information available in the VPS extension for use in reference picture list construction, it is proposed to add syntax elements for that purpose, view_position_idx[ i ].  The syntax element is proposed to be u(v) coded, as its upper limit value is known.

# Proposed Syntax and Semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 01; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag && i > 0) [Ed. (JB): syntax is not compatible with SHVC, or use of splitting_flag.] |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| for( i = 0; i < NumViews; i++ ) |  |
| view_position_idx[ i ] | u(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| vps_shvc_reserved_zero_flag | u(1) |
| } |  |

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th layer. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits. When dimension_id[ i ][ j ] is not present for j in the range of 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ i ][ j ] is inferred to be equal to ( ( layer_id_in_nuh[ i ] & ( (1 << dimBitOffset[ j + 1 ] ) − 1) ) >> dimBitOffset[ j ] ).

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer, the variable ViewId[ layer_id_in_nuh[ i ] ] specifying the view identifier of the i-th layer and the variable ViewScalExtLayerFlag specifying whether the i-th layer is a view scalability extension layer are derived as follows:

for ( i = 1; i  <=  vps_max_layers_minus1; i++) {

lId = layer_id_in_nuh[ i ]

for( smIdx= 0, j =0; smIdx < 16; smIdx ++ )

if( scalability_mask[ smIdx ] )

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

ViewId[ lId ] = ScalabilityId[ i ][ 0 ]

ViewScalExtLayerFlag[ lId ] = ( ViewId[ lId ]  !=  ViewId[ 0 ] )

}

NumViews = ViewId[ lId ] + 1

view_position_idx[ i ] specifies the view position index of the i-th view.  view_position_idx[ i ]. The length of the view_position_idx[ i ] syntax element is Ceil( Log2( NumViews ) ) bits.  The value of view_position_idx[ i ] shall be in the range of 0 to NumViews − 1, inclusive.

# Patent rights declaration(s)

Vidyo Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).