| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0052 |
| --- | --- |

| Title: | RCE2 subtest C.2: Extension of residual DPCM to lossy coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports the experimental results for Test C.2 of HEVC Range Extensions core experiment 2, as described in document JCTVC-M1122. The tested method extends residual DPCM to lossy coding over transform blocks for which transform is skipped. Lossy residual DPCM is only applied to intra coded blocks with horizontal or vertical intra prediction mode, without any explicit signalling. It is reported that in lossy configuration for screen content test conditions, the method shows BD-rate gains of up to 6.0%, 3.5% and 2.7% for all-Intra, random access and low delay configurations, respectively, for RGB 4:4:4 screen content sequences. For YUV 4:4:4 sequences, the respective gains are up to 4.6%, 2.8% and 1.3%.

# Introduction

Residual DPCM (RDPCM) for horizontal and vertical intra prediction for lossless coding was adopted at the Incheon meeting. Test C.2 from Range Extensions core experiment 2 (RCE2) [1] tested the extension of RDPCM to intra coded transform blocks with horizontal or vertical intra prediction mode, when the transform is skipped. This was first proposed in [2].

# Technical Description

For an N×N block, let ri,j, 0≤i≤N−1, 0≤j≤N−1 be the prediction residual after intra prediction. Assume that the transform is skipped and let Q(ri,j) denote the quantized ri,j. Lossy RDPCM is applied to the residual samples as follows:

ri,j=ri,j,i=0, 0≤j≤(N−1)ri,j−Q(r(i−1),j),1≤i≤(N−1), 0≤j≤(N−1)		(1)

Then, the modified residual sample ri,j is quantized to produce Q(ri,j). When the mode is vertical, is calculated as in the case of lossless RDPCM:

Q(ri,j)=k=0iQ(rk,j),0≤i≤(N−1), 0≤j≤(N−1).			(2)

The quantized modified residual samples Q(ri,j) are sent to the decoder. On the decoder side, the above calculations are repeated to produce Q(ri,j), 0≤i≤N−1, 0≤j≤N−1. The quantized residuals are added to the original prediction values to produce reconstructed sample values.

Sign data hiding is disabled for lossy RDPCM blocks because it modifies the quantized values.

# Results

The proposed method is implemented in HM10.1_Rext3.0 and simulated under the RCE2 common test conditions Error: Reference source not found. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs.The performance is compared to the anchor in terms of BD-rate savings.

|  |  | All Intra HE Main-tier |  |  |  |  |  | All Intra HE High-tier |  |  |  |  |  | All Intra HE Super-High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  | U |  | V |  | Y |  | U |  | V |  | Y | U |  | V |
| Class F |  | -2.3% |  | -2.8% |  | -2.9% |  | -2.4% |  | -2.9% |  | -3.0% |  | -2.4% | -2.9% |  | -3.0% |
| Class B |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% | 0.0% |  | 0.0% |
| SC RGB 444 |  | -5.9% |  | -5.8% |  | -5.8% |  | -6.0% |  | -6.0% |  | -5.8% |  | -5.5% | -5.4% |  | -5.1% |
| SC YUV 444 |  | -4.3% |  | -4.9% |  | -4.8% |  | -4.6% |  | -5.3% |  | -5.4% |  | -4.5% | -5.1% |  | -5.1% |
| RangeExt |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% | 0.0% |  | 0.0% |
| Enc Time[%] |  | 99% |  |  |  |  |  | 98% |  |  |  |  |  | 99% |  |  |  |
| Dec Time[%] |  | 98% |  |  |  |  |  | 97% |  |  |  |  |  | 96% |  |  |  |
|  |  |  | Random Access HE Main-tier |  |  |  |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |  |
|  |  |  | Y |  | U |  | V |  | Y |  | U |  | V |  |  |  |  |
|  | Class F |  | -1.4% |  | -1.7% |  | -1.9% |  | -1.2% |  | -1.5% |  | -1.7% |  |  |  |  |
|  | Class B |  | 0.0% |  | 0.0% |  | 0.1% |  | 0.0% |  | 0.0% |  | 0.1% |  |  |  |  |
|  | SC RGB 444 |  | -3.5% |  | -3.5% |  | -3.5% |  | -3.5% |  | -3.6% |  | -3.5% |  |  |  |  |
|  | SC YUV 444 |  | -2.7% |  | -3.2% |  | -3.2% |  | -2.8% |  | -3.4% |  | -3.5% |  |  |  |  |
|  | RangeExt |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |  |  |
|  | Enc Time[%] |  | 100% |  |  |  |  |  | 101% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] |  | 96% |  |  |  |  |  | 96% |  |  |  |  |  |  |  |  |
|  |  |  | Low delay B HE Main-tier |  |  |  |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |  |
|  |  |  | Y |  | U |  | V |  | Y |  | U |  | V |  |  |  |  |
|  | Class F |  | -0.8% |  | -1.0% |  | -1.2% |  | -0.5% |  | -0.9% |  | -0.8% |  |  |  |  |
|  | Class B |  | 0.0% |  | -0.1% |  | -0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |  |  |
|  | SC RGB 444 |  | -2.7% |  | -2.7% |  | -2.7% |  | -2.4% |  | -2.4% |  | -2.3% |  |  |  |  |
|  | SC YUV 444 |  | -1.3% |  | -1.5% |  | -1.6% |  | -1.3% |  | -1.6% |  | -1.8% |  |  |  |  |
|  | RangeExt |  | 0.0% |  | -0.1% |  | 0.0% |  | 0.0% |  | 0.0% |  | 0.0% |  |  |  |  |
|  | Enc Time[%] |  | 103% |  |  |  |  |  | 102% |  |  |  |  |  |  |  |  |
|  | Dec Time[%] |  | 99% |  |  |  |  |  | 99% |  |  |  |  |  |  |  |  |

Table 1: BD-rate performance of the lossy RDPCM

# Complexity

On the decoder side, the additional step of performing RDPCM as described in equation (2) can be performed in parallel for all the samples in the block. On the encoder side, all the samples in one row (for vertical prediction) or one column (for horizontal prediction) can be processed in parallel.

For an N×N block, on the decoder side, an average of (N−1)/2 additions per sample are needed to perform RDPCM if it is desired to process all the pixels in parallel. If it is necessary to only process all the samples in a row or column in parallel, only one addition per sample is needed. On the encoder side, one addition per sample is needed to perform RDPCM.

# Working Draft text

The modifications to the working draft text to incorporate lossy RDPCM are provided in a separate document accompanying this contribution.

# Conclusions

RDPCM is extended to lossy coding over transform-skipped blocks. Lossy RDPCM is only applied to intra coded blocks with horizontal or vertical intra prediction mode without any explicit signalling. This harmonizes the RDPCM design across transform-skip and transform-bypass blocks and provides good BD-rate improvement for screen content and class F sequences.

# References

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” JCTVC-M1122, Incheon, KR, April 2013.

- R. Joshi, J. Sole, and M. Karczewicz, “Residual DPCM for visually lossless coding,” JCTVC-M0351, Incheon, KR, April 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).