| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0053 |
| --- | --- |

| Title: | RCE2 subtest C.4: Replacement of DC mode for transform-skip and transform-bypass blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi, Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports the experimental results for Test C.4 of HEVC Range Extensions core experiment 2. The test replaces the DC intra prediction for transform-skip and transform-bypass blocks. The prediction for each sample is derived from the top, left and top-left samples. It is asserted that the method has the capability to decode a row (or column) of samples in parallel. It is reported that in lossy configuration for screen content test conditions, the method shows BD-rate gains of up to 4.5%, 3.1% and 2.1% for all-Intra, random access and low delay configurations, respectively, for RGB 4:4:4 screen content sequences. For YUV 4:4:4 sequences, the respective gains are up to 3.5%, 2.4% and 1.3%. For lossless configuration, BD-rates are:

|  | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -1.7% | -1.3% | -3.1% | -2.6% | -0.3% |
| RA | -1.0% | -0.2% | -2.2% | -1.7% | +0.1% |
| LB | -0.9% | -0.2% | -2.1% | -1.5% | +0.1% |

# Introduction

Test C.4 of HEVC Range Extensions core experiment 2 (RCE2) [1] tested the replacement of the DC intra prediction for transform-skip and transform-bypass blocks with a local neighborhood-based prediction for lossy and lossless coding. In the DC intra prediction mode, when the transform is skipped or bypassed, for each sample, a prediction based on the top, left and top-left samples is used. This was proposed as method 2 in [2].

# Technical Description

Consider a block of size N (rows) × N (cols). Let Pi,j, Ti,j, and ri,j,0≤i≤N−1, 0≤j≤N−1 be the reconstructed sample values, prediction values and prediction residuals, respectively, for the block. Assume that the transform is skipped and let Q(ri,j) denote the reconstructed residual ri,j. Then

Ti,j=Pi,j−1+Pi−1,j−Pi−1,j−1			(1)

and

Pi,j=Q(ri,j)+Ti,j or				(2)

Pi,j=n=0jQ(ri,j−n)+Pi−1,j+Pi,−1−Pi−1,−1.		(3)

For lossless coding, Q(ri,j)=ri,j and Pi,j are the original sample values. Sign data hiding is disabled for block coded using the proposed mode because it modifies the quantized values. The DC prediction filtering for the first row and column is also disabled.

# Results

The proposed methods are implemented in HM10.1_Rext3.0 and simulated under the RCE2 common test conditions Error: Reference source not found. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs.The performance is compared to the anchor in terms of BD-rates. Lossless as well as lossy results are presented.

Table 1: BD-rate performance of the proposed method under lossless configuration

Table 2: BD-rate performance of the proposed method under lossy configuration

# Complexity and Throughput

## Lossless coding

On the encoder side, all the predictions can be calculated in parallel. Hence all the samples in the block can be processed in parallel on the encoder side. On the decoder side, using Equation (3), the prediction calculation can be parallelized across all the samples in a row (or column).

For row i, (Pi,−1−Pi−1,−1) can be calculated once. Then, for the jth element in the row, (j+2) additions are needed to calculate sample Pi,j. Thus, for a row with N elements, the total number of additions needed is i=1N+1i. It should be noted that this includes the total number of additions needed to reconstruct all the samples in a row (or column).

## Lossy coding

On the decoder side, the complexity and throughput is exactly the same as in the lossless case. On the encoder side, the prediction of a sample depends on the reconstructed sample to the left. So to achieve the best BD-rate performance, some encoders may process the block sample-by-sample.

# Working Draft text

The modifications to the working draft text to incorporate the proposed method are provided in a separate document accompanying this contribution.

# Conclusions

A method for the replacement of the intra DC prediction mode for transform-skip and transform-bypass blocks is proposed. In the proposed method, instead of intra DC prediction, a prediction based on the top, left and top-left samples is used. It is shown that the proposed method has the capability to decode a row (or column) of samples in parallel. The method is applicable to both lossless and lossy coding configurations and provides good BD-rate improvement for screen content and class F sequences.

# References

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” JCTVC-M1122, Incheon, KR, April 2013.

- R. Joshi, J. Sole, and M. Karczewicz, “Modification to the DC intra prediction mode for lossless coding,” JCTVC-M0291, Incheon, KR, April 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).