| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0056 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0039 |
| --- | --- |

| Title: | On inter-layer reference picture output marking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Yamamoto, T. Tsukuba, and T. Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

JCT3V-E0039 and JCTVC-N0056 contain the same proposal commonly applicable to HLS of MV-HEVC and SHVC. This contribution proposes to set picture output flag for inter-layer reference picture. The proposed changes are asserted to provide clearer operation of picture buffer management since inter-layer reference picture can be handled in the bumping process without need of distinguishing inter-layer reference picture and other types of reference pictures.

# Introduction

In the latest draft text, how to manage the buffer of inter-layer reference picture is unclear. Inter-layer reference picture is generated and added to inter-layer reference picture set. Although reference picture mark is set, PicOutputFlag associated with the added inter-layer reference picture is never set.

Assignment of the correct PicOutputFlag to the inter-layer reference picture is useful in terms of complete picture buffer management and future extension of inter-layer reference picture usage.

In HEVC version 1, all picture buffer is well managed thanks to the following two properties; 1) all reference pictures have associated picture buffer, and 2) reference picture mark and output mark (PicOutputFlag) associated with each picture buffer are updated in the proper timing. It is desirable to keep these properties in SHVC and MV-HEVC for better picture buffer management operation.

In addition, assignment of the PicOutputFlag to the inter-layer reference picture makes it possible to handle it in the non-special bumping process. In other words, no need for distinguishing inter-layer reference picture from the other reference pictures in bumping process.

# Proposal

We propose to change the draft text so that PicOutputFlag is set to inter-layer reference picture. The following is the proposed text with changes compared to the latest draft text (JCTVC-M1008_v3) marked with cyan.

- G.8.1.2	Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer reference pictures RefPicSetInterLayer.

The variable currLayerId is set equal to nuh_layer_id of the current decoded picture

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
			nuh_layer_id equal to RefPicLayerId[ i ] ) {  
		an interlayer reference picture rsPic is derived by invoking the subclause G.8.1.4 with picX and   DirectRefLayerIdx[ currLayerId ][ RefPicLayerId[ i ] ] given as inputs
		RefPicSetInterLayer[ i ] = rsPic 
		The value of PicOutputFlag for rsPic is set equal to 0
		RefPicSetInterLayer[ i ] is marked as "used for long-term reference"
	} else
		RefPicSetInterLayer[ i ] = "no reference picture"
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer.

If the current picture is a RADL picture, there shall be no entry in the RefPicSetInterLayer that is a RASL picture.

NOTE –  An access unit may contain both RASL and RADL pictures.

# Discussion

The proposed change enables an inter-layer reference picture to be managed using ordinary HRD process. This is especially important when HRD is managed for each layer. Even when current and reference layer have the same spatial resolution, the actual picture buffer to store inter-layer reference picture is needed for decoding current layer. Since non-inter-layer reference pictures have associated picture buffer managed using reference and output marks, inter-layer reference picture should be handled in similar way. How to handle inter-layer reference picture in terms of picture buffer management also relates to profile and level constraints. For example, in profiles of HEVC version 1, NumPocTotalCurr, which is the number of reference pictures that may be used in the current picture, is restricted to be equal to 8.

# Conclusion

This contribution proposes to set picture output flag for inter-layer reference picture. It is recommended to include the proposed modification to the next draft text.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).