| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0057_r2 |
| --- | --- |
| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0040_r2 |

| Title: | MV-HEVC/SHVC HLS: On inter-layer picture selection in RPS and colPic |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Tadashi Uchiumi 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

# Abstract

In the current spec, any inter-view prediction pictures are included in RPS irrelevant to dependency type and the RPS includes motion only dependent pictures. Because motion only dependent pictures are not used as the reference samples, the ref_idx which used to indicate the picture of the reference sample is not efficient due to the non-used picture existence in the reference picture list.  Alternative colPic indication syntax for temporal motion vector derivation by layer_id has been introduced to support the motion only dependent picture which might be not included in the reference picture list, so there is inefficiency due to inclusion of motion only dependent picture in RPS. However this issue has not yet been addressed.

Similarly, any inter-view prediction pictures with motion dependency are included as candidates for the temporal picture indicated by the alternative colPic indication. That means the colPic can be selected by either the conventional syntax based on ref_idx or the alternative syntax based on layer_id.  Because of this redundancy, the layer_id base indication is inefficient. This proposal proposes that Part1) RPS doesn’t include motion only dependent picture and Part2) alternative colPic indication is only used in the case that the colPic is motion only dependent picture (i.e. not included in RPS).

Revision2(_r2) shows the change of Part 2 proposal separately as bellow[Part2]

if( !SamplePredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] && MotionPredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] )
		ActiveMotionPredRefLayerId[ j++ ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

# Introduction

RPS is used to distinguish reference picture for sample prediction by ref_idx and for motion prediction by col_ref_idx.  But when the reference picture for motion prediction dependency can be indicated by the alternative syntax based on layer_id, it is safe to say that RPS is mainly used for sample prediction. Motion only dependent picture is not used for sample prediction if it is included in RPS and the inclusion of non used picture in RPS can increase bits for ref_idx.  Thus we propose RPS doesn’t include motion only dependent picture to guarantee reference pictures referred by RPS / ref_idx have at least sample information.

Similarly, the current colPic indication method is inefficient. When RPS includes the motion only dependent picture in RPS, the motion only dependent picture can be selected by either ref_idx or layer_id. More specifically, if a picture is included in RPS so that it can be selected by ref_idx and the same picture can be indicated by the alternative colPic syntax by layer_idx, the syntax has redundunt.  Thus we propose that alternative syntax is only used in the case that the picture is motion only dependent picture (i.e. not included in RPS).

Table 1 Relation between reference and direct_dependency_type

|  |  | direct_dependency type |  |  |
| --- | --- | --- | --- | --- |
|  |  | sample + motion | sample only | motion only |
| Current | RPS | Y | Y |  |
|  | alternative colPic | Y (needed?) |  | Y |

# Proposal

We propose

- [Part1] RPS includes only pictures whose direct_dependency_type shows sample prediction dependency.

- [Part2] alternative colPic is selected from pictures whose direct_dependency_type shows motion prediction dependency only (exclude sample prediction dependency)

Table 2 Relation between reference and direct_dependency_type

|  |  | direct_dependency type |  |  |
| --- | --- | --- | --- | --- |
|  |  | sample + motion | sample only | motion only |
| Proposal | RPS | Y | Y |  |
|  | alternative colPic |  |  | Y |

#Specificaly we count ActiveSamplePredRefLayerId and ActiveMotionPredRefLayerId as follows and RPS includes only picture whose layer_id is included in ActiveSamplePredRefLayerId.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveSamplePredRefLayers > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 ) |  |
| alt_collocated_indication_flag | u(1) |
| if( alt_collocated_indication_flag ) |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |

G.7.4.7.1 General slice segment header semantics

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element inter_layer_pred_layer_idc[ i ] is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

When i is greater than 0, inter_layer_pred_layer_idc[ i ] shall be greater than inter_layer_pred_layer_idc[ i − 1 ].

The variables RefPicLayerId[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, NumActiveSamplePredRefLayers, and ActiveSamplePredRefLayerId[ k ] for each value of k in the range of 0 to NumActiveSamplePredRefLayers − 1, inclusive, NumActiveMotionPredRefLayers, and ActiveMotionPredRefLayerId[ j ] for each value of j in the range of 0 to NumActiveMotionPredRefLayers − 1, inclusive, are derived as follows:

for( i = 0, j = 0, k=0; i < NumActiveRefLayerPics; i++) {

RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

if( SamplePredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] )
		ActiveSamplePredRefLayerId[ k++ ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

else if( MotionPredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] )
		ActiveMotionPredRefLayerId[ j++ ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

// Part2

if( !SamplePredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] && MotionPredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] )
		ActiveMotionPredRefLayerId[ j++ ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]

}
NumActiveMotionPredRefLayers = j

NumActiveSamplePredRefLayers = k

inter_layer_sample_pred_only_flag equal to 1 specifies that inter prediction is not used in decoding of the current picture. inter_layer_sample_pred_only_flag equal to 0 specifies that inter prediction may be used in decoding of the current picture. When not present, the value of inter_layer_sample_pred_only_flag is inferred to be equal to 0.

The variable InterRefEnabledInRPLFlag is derived as follows:.

–	If NumSamplePredRefLayers[ nuh_layer_id ] is greater than 0 and NumActiveSamplePredRefLayers is greater than 0, InterRefEnabledInRPLFlag is set equal to !inter_layer_sample_pred_only_flag.

–	Otherwise, InterRefEnabledInRPLFlag is set equal to 1

G.8.7.4.7.2 Reference picture list modification semantic

The specifications in subclause Error: Reference source not found apply with following modifications.

–	Equation (7-43) specifying the derivation of NumPocTotalCurr is replaced by:

NumPocTotalCurr = 0
for( i = 0; i < NumNegativePics[ CurrRpsIdx ]; i++)
	if(UsedByCurrPicS0[ CurrRpsIdx ][ i ] )
		NumPocTotalCurr++
for( i = 0; i < NumPositivePics[ CurrRpsIdx ]; i++)	(G-1)
	if(UsedByCurrPicS1[ CurrRpsIdx ][ i ] )
		NumPocTotalCurr++
for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ )
	if( UsedByCurrPicLt[ i ] )
		NumPocTotalCurr++
NumPocTotalCurr += NumSamplePredRefLayerPics

G.8.1.1 Decoding process for inter-layer reference picture set

for( i = 0; i < NumActiveSamplePredRefLayers; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
			nuh_layer_id equal to ActiveSamplePredRefPicLayerId[ i ] ) {
		RefPicSetInterLayer[ i ] = picX
		RefPicSetInterLayer[ i ] is marked as "used for long-term reference"
	} else
		RefPicSetInterLayer[ i ] = "no reference picture"
}

G.8.5.1 Derivation process for temporal luma motion vector prediction

# This part is not changed. But counting method of ActiveMotionPredRefLayerId is changed by the proposal.

- If slice_temporal_mvp_enabled_flag is equal to 0, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the following ordered steps apply:

- Depending on the values of alt_collocated_indication_flag, collocated_ref_layer_idx, slice_type, collocated_from_l0_flag, and collocated_ref_idx, the variable colPic, specifying the collocated picture, is derived as follows:

- If alt_collocated_indication_flag is equal to 1, colPic is set equal to the picture in the current access unit with nuh_layer_id equal to ActiveMotionPredRefLayerId[ collocated_ref_layer_idx ].

# Conclusion

A dependency type usage related to inter-layer picture selection in RPS and alternative colPic is proposed in order to provide clear specification and to remove dependency. The change of direct_dependency_type representation is also proposed. It is recommended to adopt these techniques in the next SHVC and MV-HEVC WD and HM / HTM.

# Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).