| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0061 |
| --- | --- |

| Title: | Non-SCE3.3: Inter-layer interpolation-based SAO filtering for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov Vasily Rufitskiy Russian Research Division Krylatskaya str., 17, building 2 Moscow 121614, Russia | Tel: Email: | +79250082308 alexey.filippov@huawei.com vasily.rufitskiy@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This contribution proposes adaptive restriction of EO offsets for the inter-layer SAO filter that was initially proposed in JCTVC-L0234 [1] and JCTVC-M0114 [3]. The proposed modification of JCTVC-M0114 [3] is based on restricting EO offsets for pixels. The restrictions are calculated using the values of the same neighbouring pixels that are used for determining the edge index for the Edge Offset (EO). Coding gain measurement performed over all SHM2.0 configurations reveals 0.7%(Y), 0.6%(U), 0.6%(V) average BDR gain on mandatory configurations. For the LD-P SNR configuration, BDR gain can reach up to 2.7%(Y), 1.8%(U), 1.7%(V).

# Introduction

The proposed modification of the inter-layer SAO filter as compared to the previous SCE4.2.1 [2] and to the Canons proposal JCTVC-M0114 [3] is to adaptively restrict EO offsets using interpolation of neighbour pixels. Modified HF pass data-flow chart is shown in Figure 1.

Figure 1: Modified HF pass data-flow chart.

Context-adaptive EO offsets restriction is introduced by the proposed modification. A range of allowed values (RAV) for the the ith pixel can be calculated subject to the values of its neighbors as it is presented in Figure 2. In this case, the final ith pixel value cannot change more than the maximum offset value defined by RAV of the ith pixel.

Figure 2: Defining EO range for the case of five-sample EO pattern.

This proposal does not modify the table of EO offsets and does not introduce any syntax level changes.

The modification is only introduced at pixel level. Specifically, if the EO offset value defined exceeds RAV, the pixel offset is set to the extreme offset value of the RAV, which is a maximum for positive EO offsets or a minimum for negative EO offsets. If RAV of the pixel includes the specified EO offset, the pixel offset is set to the specified EO offset.

For the 5-samples patterns shown in Figure 2, the following formula is used to calculate the interpolated value for a given pixel c:

| Cmax=2n−1+1C1+C2−C3+C42n=C1+C2<<n−1+C1+C2−C3+C4>>n, n=3 | (1) |
| --- | --- |

Figure 2: Five-sample directional patterns.

The maximum offset value is derived from the interpolation value as Cmax−C. For the given EO offset Δ, the following procedures are proposed to determine the final value of the offset for a given pixel c. For EO categories 1 and 2:

| Δfinal=Δ, if Cmax>C&&C+Δ<=CmaxCmax−C, if Cmax>C&& &&C+Δ>Cmax | (2) |
| --- | --- |

For EO categories 3 and 4:

| Δfinal=Δ, if Cmax<C&&C+Δ>=CmaxC−Cmax, if Cmax<C&& &&C+Δ<Cmax | (3) |
| --- | --- |

An example of context-adaptive restrictions for EO offsets is shown in Figure 3

Figure 3. An example of context-adaptive restrictions for EO offsets

To reduce complexity of the rate-distortion optimization (RDO), it is proposed to modify statistic estimation procedure by introducing distortion look-up table for each EO category (Figure 4).

Figure 4. Structure of distortion look-up table

When estimating EO statistics, the same parameters are gathered as in the previous SCE4.2.1 [2], namely: the number of pixels belonging to the given category and distortion, i.e. the sum of differences between original and reconstructed pixels. While estimating the statistics, MaxOffset value for the pixels of EO categories 1..4 is calculated using formula (1): MaxOffset=Cmax−C. The clipped value clip3(1,M,abs(MaxOffset)) is used as an index value of the distortion look-up table to update “pixel count” and “Distortion” cells. M is the maximum EO offset value that in the current proposal is set to 7.

To calculate the distortion for the given offset k, the distortion calculation function is called for each index of the distortion table with the input offset parameter clipped to k. The sum of the resulting values gives the final distortion value for the given EO category.

# Results

This section presents the results of the proposed SAO method to adaptively restrict EO offsets of the IL-SAO filter by using the SHM2.0 testing conditions defined in the document [4]. In the following results, the SAO processing presented above is applied twice as for the HF-IL-SAO in the previous SCE4.2.1 [2].

In average for mandatory sequences, 0.7%(Y), 0.6%(U), 0.6%(V) BDR gains are reported with the proposed methods. Yet, for all SNR configurations the BDR gain reaches 1.8% average gain.

Table 1 BD-Rate savings for the proposed modifications compared to SHM2.0.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1,0% | -1,3% | -1,3% |  |  |  |  |  |  |
| Class B | -0,2% | -0,3% | -0,3% | -0,3% | -0,4% | -0,5% |  |  |  |
| Overall (Test vs Ref) | -0,5% | -0,6% | -0,6% | -0,3% | -0,4% | -0,5% |  |  |  |
| Overall (Test vs single layer) | 12,3% | 14,2% | 13,9% | 10,3% | 9,3% | 8,8% |  |  |  |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |  |  |  |
| EL only (Test vs Ref) | -0,9% | -1,0% | -1,0% | -0,1% | -0,4% | -0,4% |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1,0% | -0,7% | -0,7% |  |  |  | -2,5% | -1,8% | -2,0% |
| Class B | -0,4% | -0,3% | -0,2% | -0,3% | -0,4% | -0,3% | -1,1% | -0,8% | -0,6% |
| Overall (Test vs Ref) | -0,5% | -0,4% | -0,4% | -0,3% | -0,4% | -0,3% | -1,5% | -1,0% | -1,0% |
| Overall (Test vs single layer) | 18,6% | 32,8% | 31,5% | 15,8% | 28,4% | 28,7% | 12,7% | 30,8% | 32,8% |
| Overall (Ref vs single layer) | 19,2% | 33,3% | 32,0% | 16,2% | 28,8% | 29,1% | 14,4% | 32,1% | 34,1% |
| EL only (Test vs Ref) | -0,9% | -0,8% | -0,8% | -0,3% | -0,3% | -0,3% | -2,3% | -1,8% | -1,8% |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,9% | -0,7% | -0,6% |  |  |  | -2,2% | -1,5% | -1,9% |
| Class B | -0,4% | -0,3% | -0,2% | -0,4% | -0,4% | -0,4% | -0,9% | -1,0% | -0,8% |
| Overall (Test vs Ref) | -0,6% | -0,4% | -0,4% | -0,4% | -0,4% | -0,4% | -1,3% | -1,2% | -1,1% |
| Overall (Test vs single layer) | 27,8% | 38,4% | 39,3% | 24,3% | 32,5% | 35,4% | 22,7% | 33,2% | 38,0% |
| Overall (Ref vs single layer) | 28,5% | 39,0% | 39,7% | 24,8% | 33,0% | 35,9% | 24,3% | 34,7% | 39,5% |
| EL only (Test vs Ref) | -0,9% | -0,8% | -0,7% | -0,2% | -0,3% | -0,3% | -1,9% | -1,7% | -1,7% |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1,1% | -0,8% | -0,8% |  |  |  | -4,0% | -2,4% | -2,6% |
| Class B | -1,0% | -0,7% | -0,7% | -1,1% | -0,9% | -0,9% | -2,2% | -1,5% | -1,3% |
| Overall (Test vs Ref) | -1,0% | -0,7% | -0,7% | -1,1% | -0,9% | -0,9% | -2,7% | -1,8% | -1,7% |
| Overall (Test vs single layer) | 25,3% | 37,1% | 38,1% | 21,5% | 31,7% | 34,5% | 20,1% | 32,3% | 37,2% |
| Overall (Ref vs single layer) | 26,6% | 38,0% | 39,1% | 22,8% | 32,8% | 35,6% | 23,4% | 34,6% | 39,4% |
| EL only (Test vs Ref) | -1,5% | -1,2% | -1,3% | -1,1% | -0,9% | -0,9% | -3,8% | -2,8% | -2,7% |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

In table 2 preliminary values of encoding and decoding time increase are reported. The following limitations were applied during the evaluation: only first 32 frames of every test sequence were used; implementation was not profiled and optimized, the primary goal of this implementation was to ensure that solution could provide BD-rate gain.

Table 2 Preliminary evaluation of maximal encoding and decoding time increase

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enc Time[%] | 106,5% |  |  | 106,8% |  |  |  |  |  |  |
| Dec Time[%] | 126,6% |  |  | 134,2% |  |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |  |
| Enc Time[%] | 102,2% |  |  | 101,9% |  |  | 102,1% |  |  |  |
| Dec Time[%] | 132,6% |  |  | 127,9% |  |  | 142,3% |  |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |  |  |  |  |
| Enc Time[%] | 101,7% |  |  | 102,0% |  |  |  |  |  |  |
| Dec Time[%] | 136,4% |  |  | 141,4% |  |  |  |  |  |  |

# Conclusion

The average values of the BDR performance of the proposed solution are presented in Table 3.

Table 3 The average values of the BD-rate reductions for different configurations for the proposed modifications compared to SHM2.0.

| Configuration | BDR reduction for different color planes, % |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| The average value for the mandatory tests | 0.7 | 0.6 | 0.6 |
| The average value for the mandatory and optional tests | 0.9 | 0.8 | 0.7 |
| SNR configuration | 1.8 | 1.3 | 1.3 |

Complexity analysis results are very preliminary. The further reduction of computational complexity is possible for both the encoder and the decoder by optimizing the current software solution and introducing look-up tables.

According to these results, we recommend to include this modification of inter-layer SAO filter into an SHVC Core Experiment.

# References

- W. Pu, J. Chen, K. Rapaka, M. Karczewicz, “High Frequency Pass Inter Layer Sample Adaptive Offset Filter”, Joint Collaborative Team on Video Coding, JCTVC-L0234, January 2013.

- W. Pu, J. Chen, X. Li, M. Karczewicz, “SCE4: High Frequency Pass Inter Layer Sample Adaptive Offset Filter”, Joint Collaborative Team on Video Coding, JCTVC-M0265, April 2013.

- G. Laroche, P. Onno, J. Taquet, C. Gisquet and E. François, “Non-SCE4: On Interlayer SAO in SHVC”, Joint Collaborative Team on Video Coding, JCTVC-M0114, April 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-M1009, April 2013.

# Patent rights declaration(s)

Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).