| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0064r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0050r2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: on storage of motion fields |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The proposal consists of two parts:

- collocated_picture_constraint_flag in the SPS extension indicating, when equal to 1, that no TMVP is used for pictures within the same layer.

- An informative note describing when the storage of a motion field is required and when it becomes no longer needed.

Revision 1 proposes an SEI message that includes collocated_picture_constraint_flag.

Revision 2 proposes an extension of the SEI message including indications of other TMVP constraints too.

# Problem Statement

sps_temporal_mvp_enabled_flag indicates whether the TMVP mechanism may be in use (when the flag is equal to 1) or is not in use (when the flag is equal to 0) in the base layer/view (with nuh_layer_id equal to 0). When sps_temporal_mvp_enabled_flag is equal to 1, the slice_temporal_mvp_enabled_flag is present in the slice header and indicates if the TMVP mechanism is in use for the current picture.

It is envisioned that there will be SHVC and MV-HEVC implementations where the base layer decoding/coding is implemented with an existing HEVC v1 implementation without changes. Such an implementation of base layer decoding/coding would store motion fields only if sps_temporal_mvp_enabled_flag is equal to 1.

Base layer motion fields may be used for either or both of the following two purposes: temporal motion vector prediction between pictures of the base layer and inter-layer motion prediction. If the base layer motion fields are used only for inter-layer motion prediction, the memory used for base layer motion fields could be de-allocated or used for other purposes after decoding of the access unit has been finished (or, more accurately, decoding of all layers within the access unit that may use the abase layer as motion prediction reference has been finished). However, when sps_temporal_mvp_enabled_flag is used to control the storage of base layer motion fields, it cannot be used to indicate that base layer motion fields are used only for inter-layer motion prediction and not for temporal motion vector prediction within the base layer.

In JCTVC-M0065, it was proposed to add a flag in the SPS extension applicable for enhancement layers to indicate that the collocated picture is always an inter-layer reference picture rather than a temporal reference picture. The semantics of the proposed flag were proposed to be the following: collocated_picture_constraint_flag equal to 1 specifies that collocated picture used for inter-layer prediction are constrained in the CVS. When collocated_picture_constraint_flag is equal to 1, the collocated picture used for inter-layer prediction shall be the reference picture with nuh_layer_id not equal to that of current picture. When collocated_picture_constraint_flag is equal to 0, no constraint for the collocated picture used for inter-layer prediction is signalled by this flag. When not present, the collocated_picture_constraint_flag is inferred to be equal to 0.

It is asserted in this contribution that the approach proposed in JCTVC-M0065 had the following shortcomings:

- No mechanism has been proposed to indicate whether the base layer motion fields are needed for TMVP within the base layer or inter-layer motion prediction or both. The semantics of collocated_picture_constraint_flag for the base layer were not proposed in JCTVC-M0065. Consequently, even if the encoder used base layer motion fields only for inter-layer motion prediction, the decoder still has to store base layer motion fields beyond the decoding of the current access unit.

- If a first enhancement layer for which the collocated_picture_constraint_flag (as proposed in JCTVC-M0065) is set equal to 1 is used as a reference for inter-layer prediction for a second enhancement layer, the storage of the motion field of pictures in the first enhancement layer may be needed for inter-layer motion prediction. In other words, the collocated_picture_constraint_flag (as proposed in JCTVC-M0065) is not sufficient to determine if the motion field of the enhancement layer picture needs to be stored or not.

The proposal enables decoders to conclude:

- whether or not the motion field of the currently decoded picture has to be stored (to serve as reference for TMVP for subsequent pictures in decoding order), and

- when a motion field of a picture is no longer needed (as reference for TMVP).

Consequently, the proposal enables decoders to be implemented with smaller memory consumption than that what would be required without the proposal.

# Proposal (revision 2)

## Overview

The proposal consists of two parts:

- temporal motion vector prediction constraints SEI message capable of indicating:

- how the associated layer use temporal motion vector prediction

  - no temporal motion vector prediction

  - temporal motion vector prediction from reference layer or the same layer, and signalling in slice level

  - temporal motion vector prediction from only reference layer

  - temporal motion vector prediction from only the same layer

- whether motion fields of associated layer need to be stored for inter prediction or inter-layer prediction or both inter and inter-layer prediction

- An informative note describing when the storage of a motion field is required and when it becomes no longer needed.

## Proposed specification text

| temporal_motion_vector_prediction_constraints( payloadSize ) { | Descriptor |
| --- | --- |
| no_intra_layer_col_pic_flag | u(1) |
| not_used_as_interlayer_col_pic_flag | u(1) |
| does_not_use_interlayer_col_pic_flag | u(1) |
| } |  |

The temporal motion vector prediction constraints SEI message indicates constraints on collocated pictures of the pictures to which the message applies.

When the message is not included in a scalable nesting SEI message, it applies to pictures with nuh_layer_id equal to that of the SEI NAL unit containing the message. When the message is included in a scalable nesting SEI message, it applies to the pictures with nuh_layer_id equal to all values of nestingLayerIdList[ i ] derived through the semantics of the containing scalable nesting SEI message. Let associatedLayerId be any nuh_layer_id value to which the temporal motion vector prediction constraints SEI message applies.

The temporal motion vector prediction constraints SEI message is a prefix SEI message and may be present for a picture that activates an SPS or a layer SPS for nuh_layer_id equal to associatedLayerId and and shall not be present for any other any other picture.

Let a set of pictures assocatedPicSet be the pictures with nuh_layer_id equal to associatedLayerId from the picture associated with the SEI message until the picture that activates next SPS or layer SPS with nuh_layer_id equal to associatedLayerId, exclusive, or until the end of the CVS, exclusive, whichever is earlier in decoding order. The temporal motion vector prediction constraints applies to associatedPicSet.

Let a set of access units associatedAUSet be the set of access units containing the pictures of associatedPicSet.

no_intra_layer_col_pic_flag equal to 1 indicates the following:

–	If associatedLayerId is equal to 0, slice_temporal_mvp_enabled_flag is not present or is equal to 0 in each picture of associatedPicSet.

–	Otherwise, each collocated picture, if any, of the pictures in associatedPicSet has nuh_layer_id different from associatedLayerId.

When no_intra_layer_col_pic_flag is equal to 0, the collocated pictures of the pictures in associatedPicSet may or may not have the same nuh_layer_id value as that of the pictures in associatedPicSet.

not_used_as_interlayer_col_pic_flag equal to 1 indicates that no picture in associatedAUSet uses any picture in associatedPicSet as a collocated picture for temporal motion vector prediction. not_used_as_interlayer_col_pic_flag equal to 0 indicates that there may or may not be pictures in associatedAUSet that use pictures in associatedPicSet as a collocated picture.

does_not_use_interlayer_col_pic_flag equal to 1 indicates that no picture in associatedPicSet uses a collocated picture with nuh_layer_id not equal to associatedLayerId for temporal motion vector prediction. does_not_use_interlayer_col_pic_flag equal to 0 indicates that there may or may not be in associatedPicSet that use a collocated picture with nuh_layer_id not equal to associatedLayerId for temporal motion vector prediction.

Let NoIntraLayerColPicFlag[ associatedLayerId ] be equal to no_intra_layer_col_pic_flag. Let NotUsedAsILColPicFlag[ associatedLayerId ] be equal to not_used_as_interlayer_col_pic_flag.

NOTE – The motion vectors of the current picture with nuh_layer_id equal to layerId have to be stored when they may be used for temporal motion vector prediction of other pictures in the same layer or when they may be used for inter-layer motion prediction. In other words, the motion vectors of the current picture have to be stored when NoIntraLayerColPicFlag[ layerId ] is equal to 1 and NotUsedAsILColPicFlag[ layerId ] is equal to 1.

The motion vectors of a picture with nuh_layer_id equal to layerId need no longer be stored when either the picture is marked as “unused for reference” or when the picture is not used for temporal motion vector prediction of other pictures in the same layer and all pictures in the same access unit that may use the picture as a reference for inter-layer motion prediction have been decoded. In other words, the motion vectors of a picture need no longer be stored when at least one of the following is true:

- The picture is marked as “unused for reference”.

- NoIntraLayerColPicFlag[ layerId ] is equal to 1 for the picture and the access unit containing the picture has been decoded.

- NoIntraLayerColPicFlag[ layerId ] is equal to 1 for the picture and there is no nuh_layer_id value nuhLayerIdA among those values of TargetDecLayerIdList that remain to be decoded from the access unit containing the picture such that MotionPredRefLayerId[ nuhLayerIdA ][ mIdx ] for any mIdx in the range of 0 to NumMotionPredRefLayers[ nuhLayerIdA ] – 1, inclusive, is equal to layerId.

# Proposal (revision 1)

## Overview

The proposal consists of two parts:

- temporal motion vector prediction constraints SEI messge capable of indicating whether no TMVP is used for pictures within the same layer.

- An informative note describing when the storage of a motion field is required and when it becomes no longer needed.

## Proposed specification text

| temporal_motion_vector_prediction_constraints( payloadSize ) { | Descriptor |
| --- | --- |
| collocated_picture_constraint_flag | u(1) |
| } |  |

The temporal motion vector prediction constraints SEI message indicates constraints on collocated pictures of the pictures to which the message applies.

When the message is not included in a scalable nesting SEI message, it applies to pictures with nuh_layer_id equal to that of the SEI NAL unit containing the message. When the message is included in a scalable nesting SEI message, it applies to the pictures with nuh_layer_id equal to all values of nestingLayerIdList[ i ] derived through the semantics of the containing scalable nesting SEI message. Let associatedLayerId be any nuh_layer_id value to which the temporal motion vector prediction constraints SEI message applies.

The temporal motion vector prediction constraints SEI message is a prefix SEI message and may be present for a picture that activates an SPS or a layer SPS for nuh_layer_id equal to associatedLayerId and and shall not be present for any other any other picture.

Let a set of pictures assocatedPicSet be the pictures with nuh_layer_id equal to associatedLayerId from the picture associated with the SEI message until the picture that activates next SPS or layer SPS with nuh_layer_id equal to associatedLayerId, exclusive, or until the end of the CVS, exclusive, whichever is earlier in decoding order. The temporal motion vector prediction constraints applies to associatedPicSet.

collocated_picture_constraint_flag equal to 1 indicates the following:

–	If associatedLayerId is equal to 0, slice_temporal_mvp_enabled_flag is not present or is equal to 0 in each picture of associatedPicSet.

–	Otherwise, any collocated picture of the pictures of associatedPicSet has nuh_layer_id different from associatedLayerId.

When collocated_picture_constraint_flag is equal to 0, no constraint for the collocated picture of the pictures in associatedPicSet is indicated.

Let CollocatedPictureConstraintFlag[ associatedLayerId ] be equal to collocated_picture_constraint_flag.

NOTE – The motion vectors of the current picture with nuh_layer_id equal to layerId have to be stored when they may be used for temporal motion vector prediction of other pictures in the same layer or when they may be used for inter-layer motion prediction. In other words, the motion vectors of the current picture have to be stored when at least one of the following is true:

- sps_temporal_mvp_enabled_flag is equal to 1 and CollocatedPictureConstraintFlag[ layerId ] is equal 0.

- CollocatedPictureConstraintFlag[ layerId ] is equal to 1 and there is a nuh_layer_id value nuhLayerIdA such that MotionPredRefLayerId[ nuhLayerIdA ][ mIdx ] for any mIdx in the range of 0 to NumMotionPredRefLayers[ nuhLayerIdA ] – 1, inclusive, is equal to the layerId and nuhLayerIdA is among TargetDecLayerIdList.

The motion vectors of a picture with nuh_layer_id equal to layerId need no longer be stored when either the picture is marked as “unused for reference” or when the picture is not used for temporal motion vector prediction of other pictures in the same layer and all pictures in the same access unit that may use the picture as a reference for inter-layer motion prediction have been decoded. In other words, the motion vectors of a picture need no longer be stored when at least one of the following is true:

- The picture is marked as “unused for reference”.

- CollocatedPictureConstraintFlag[ layerId ] is equal to 1 for the picture and the access unit containing the picture has been decoded.

- CollocatedPictureConstraintFlag[ layerId ] is equal to 1 for the picture and there is no nuh_layer_id value nuhLayerIdA among those values of TargetDecLayerIdList that remain to be decoded from the access unit containing the picture such that MotionPredRefLayerId[ nuhLayerIdA ][ mIdx ] for any mIdx in the range of 0 to NumMotionPredRefLayers[ nuhLayerIdA ] – 1, inclusive, is equal to layerId.

# Proposal (revision 0)

## Overview

The proposal consists of two parts:

- collocated_picture_constraint_flag in the SPS extension indicating, when equal to 1, that no TMVP is used for pictures within the same layer.

- An informative note describing when the storage of a motion field is required and when it becomes no longer needed.

## Proposed specification text

| sps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| collocated_picture_constraint_flag | u(1) |
| … |  |
| } |  |

collocated_picture_constraint_flag equal to 1 indicates the following:

–	In coded slice segment NAL units with nuh_layer_id equal to 0 for which this SPS is the active SPS, slice_temporal_mvp_enabled_flag is not present or is equal to 0.

–	In coded slice segment NAL units with nuh_layer_id nuhLayerIdA greater 0 for which this SPS is the active layer SPS, the collocated picture, if any, has nuh_layer_id nuhLayerIdB that is not equal to nuhLayerIdA.

When collocated_picture_constraint_flag is equal to 0, no constraint for the collocated picture is indicated. When not present, collocated_picture_constraint_flag is inferred to be equal to 0.

NOTE – The motion vectors of the current picture have to be stored when they may be used for temporal motion vector prediction of other pictures in the same layer or when they may be used for inter-layer motion prediction. In other words, the motion vectors of the current picture have to be stored when at least one of the following is true:

- sps_temporal_mvp_enabled_flag is equal to 1 and collocated_picture_constraint_flag is equal 0.

- collocated_picture_constraint_flag is equal to 1 and there is a nuh_layer_id value nuhLayerIdA such that MotionPredRefLayerId[ nuhLayerIdA ][ mIdx ] for any mIdx in the range of 0 to NumMotionPredRefLayers[ nuhLayerIdA ] – 1, inclusive, is equal to the nuh_layer_id value of the current picture and nuhLayerIdA is among TargetDecLayerIdList.

The motion vectors of a picture need no longer be stored when either the picture is marked as “unused for reference” or when the picture is not used for temporal motion vector prediction of other pictures in the same layer and all pictures in the same access unit that may use the picture as a reference for inter-layer motion prediction have been decoded. In other words, the motion vectors of a picture need no longer be stored when at least one of the following is true:

- The picture is marked as “unused for reference”.

- collocated_picture_constraint_flag is equal to 1 for the picture and the access unit containing the picture has been decoded.

- collocated_picture_constraint_flag is equal to 1 for the picture and there is no nuh_layer_id value nuhLayerIdA among those values of TargetDecLayerIdList that remain to be decoded from the access unit containing the picture such that MotionPredRefLayerId[ nuhLayerIdA ][ mIdx ] for any mIdx in the range of 0 to NumMotionPredRefLayers[ nuhLayerIdA ] – 1, inclusive, is equal to the nuh_layer_id value of the picture.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).