| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0072 |
| --- | --- |

| Title: | RCE2-related: Variants of simplified sample-based weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Andreas Hutter Siemens Corporate Technology Research and Technology Center Imaging and Computer Vision Munich, Germany Eugen Wige, André Kaup Universität Erlangen-Nürnberg Multimedia Communications and Signal Processing Erlangen, Germany | Tel: Email: | +49 89 636 54642 p.amon@siemens.com |
| Source: | Siemens |  |  |

_____________________________

# Abstract

This contribution presents two variants of the intra prediction scheme presented in JCTVC-N0071 and tested in RCE2 Test C.3. The first variant replaces the weighted prediction process by the selection of the predictor where the templates of current and candidate pixels have the highest similarity, i.e., the lowest SAD. This is signalled by the PLANAR mode. The second variant combines weighted prediction and selection of the best predictor, where the weighted prediction is signalled by the PLANAR mode and the best predictor is signalled by the DC mode. In comparison to the unmodified HEVC Test Model HM10.1_RExt3.0 configured for lossless coding by disabling/bypassing transformation, quantization, and in-loop filters, the proposed method reportedly provides for All Intra Main settings the following results: For variant 1, savings of 2.2% for Class F, 0.3% for Class B, 5.3% for SC RGB 444, 4.1% for SC YUV 444, and 0.0% for RangeExt are reported. For variant 2, savings of 2.8% for Class F, 2.3% for Class B, 5.4% for SC RGB 444, 4.4% for SC YUV 444, and 1.1% for RangeExt are reportedly achieved.

# Introduction

JCTVC-N0071 [1] presents an intra-prediction scheme designed for lossless coding. The method comprises a pixel-wise prediction based on original samples. It is realized as a separate intra prediction mode, which replaces the PLANAR mode. In order to perform the prediction, a two-sample template around the pixel that is to be predicted is compared to the respective templates of a four-pixel neighbourhood. For each reference template, the sum of absolute differences (SAD) is determined. A table look-up of the SAD value gives the respective weighting factor for each neighbourhood pixel. The predictor for the current pixel is calculated as the weighted average of the neighbourhood pixels. This contribution presents two variants of this method, which are described in the following.

# Proposed methods

## Variant 1: Selection of best predictor

The first variant replaces the weighted prediction process in JCTVC-N0071 [1] by the selection of the predictor, where the templates of current and candidate pixels have the highest similarity, i.e., the lowest SAD. This simplifies the prediction process to

pi=popt

This also means that the calculation of the individual weights is avoided. This prediction scheme is signalled by the PLANAR mode in the case of lossless coding.

## Variant 2: Combination of weighted prediction and best predictor

The second variant combines weighted prediction as of JCTVC-N0071 [1] and selection of the best predictor as of variant 1, where the weighted prediction is signalled by the PLANAR mode and the best predictor is signalled by the DC mode in case of lossless coding.

# Simulation results

The proposed method was implemented into HEVC Test Model HM10.1_RExt3.0. The common test conditions for RCE2 as defined in [2] were used. The simulation platform is a homogeneous cluster (consisting of Intel(R) Core(TM)2 Quad CPU Q9400 @ 2.66GHz CPUs).

## Simulation results for variant 1

Table 1, Table 2, and Table 3 show the simulation results for variant 1.

Table 1. Simulation results for variant 1 for All Intra Main configuration settings

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -2.2% |
| Class B | 2.2 | 2.3 | -0.3% |
| SC RGB 444 | 10.1 | 10.7 | -5.3% |
| SC YUV 444 | 11.4 | 12.0 | -4.1% |
| RangeExt | 2.4 | 2.4 | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 103% |  |  |

Table 2. Simulation results for variant 1 for Random Access Main configuration settings

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 31.7 | 32.2 | -1.2% |
| Class B | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 100.4 | 107.0 | -4.0% |
| SC YUV 444 | 128.9 | 135.3 | -3.3% |
| RangeExt | 2.5 | 2.5 | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 3. Simulation results for variant 1 for Low delay B Main configuration settings

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 49.8 | 50.4 | -1.0% |
| Class B | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 381.6 | 412.1 | -3.8% |
| SC YUV 444 | 325.6 | 346.8 | -3.0% |
| RangeExt | 2.5 | 2.5 | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

## Simulation results for variant 2

Table 4, Table 5, and Table 6 show the simulation results for variant 2.

Table 4. Simulation results for variant 2 for All Intra Main configuration settings

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -2.8% |
| Class B | 2.2 | 2.3 | -2.3% |
| SC RGB 444 | 10.1 | 10.7 | -5.4% |
| SC YUV 444 | 11.4 | 12.0 | -4.4% |
| RangeExt | 2.4 | 2.4 | -1.1% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 106% |  |  |

Table 5. Simulation results for variant 2 for Random Access Main configuration settings

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 31.7 | 32.3 | -1.4% |
| Class B | 2.6 | 2.6 | -0.5% |
| SC RGB 444 | 100.4 | 107.0 | -3.9% |
| SC YUV 444 | 128.9 | 135.3 | -3.4% |
| RangeExt | 2.5 | 2.5 | -0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 6. Simulation results for variant 2 for Low delay B Main configuration settings

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 49.8 | 50.5 | -1.2% |
| Class B | 2.6 | 2.6 | -0.4% |
| SC RGB 444 | 381.6 | 412.5 | -3.8% |
| SC YUV 444 | 325.6 | 346.6 | -3.1% |
| RangeExt | 2.5 | 2.5 | -0.3% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# Conclusion

Lossless coding is need in several professional applications, e.g., in the medical domain. This contribution presented an intra prediction scheme designed for lossless coding, which shows significant gain over the current HEVC design. Therefore, we propose to include the proposed technique in the Proposed Draft Amendment (PDAM) for Range Extensions of HEVC.

# Acknowledgment

The research leading to these results has received funding from the European Union’s Seventh Framework Programme ([FP7/2007-2013]) under grant agreement n◦ 288502 (CONCERTO).

# References

- Peter Amon, Andreas Hutter, Eugen Wige, André Kaup, “RCE2: Simplified sample-based weighted prediction (Test C.3)”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, document JCTVC-N0071/m29675, Vienna, Austria, July/August 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, document JCTVC-M1122/m29534, Incheon, Korea, April 2013.

# Patent rights declaration(s)

Siemens AG and Universität Erlangen-Nürnberg may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).