| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 02 Aug. 2013 | Document: JCTVC-N0073 |
| --- | --- |

| Title: | RCE2: Description and report for Test D.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Prop |  |  |
| Author(s) or Contact(s): | Eugen Wige, André Kaup Universität Erlangen-Nürnberg, Germany Peter Amon, Andreas Hutter Siemens Corporate Technology, Germany Rajan Joshi, Joel Sole, Marta Karczewicz Qualcomm Inc., CA, USA | Email: Email: Email: | wige@LNT.de kaup@LNT.de p.amon@siemens.com andreas.hutter@siemens.com rajanj@qti.qualcomm.com joels@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Siemens, Qualcomm |  |  |

_____________________________

# Abstract

In this contribution, a method for intra coding is proposed for HEVC lossless configuration. The method is a combination of local-neighbourhood-based intra prediction techniques from Test C.3 and Test C.4. In Test C.3, the PLANAR prediction is replaced by a sample-based weighted prediction scheme. In Test C.4, the regular DC prediction of the HEVC standard is replaced by a local neighborhood-based prediction. It is reported that in lossless configuration, the performance of the combination is:

|  | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -3.4% | -2.9% | -6.2% | -4.8% | -1.4% |
| RA | -1.8% | -0.7% | -4.5% | -3.4% | -0.4% |
| LB | -1.5% | -0.6% | -4.4% | -3.3% | -0.3% |

# Introduction

For exploration of efficient tools for transform-skip blocks and lossless coding, the Range Extension Core Experiment 2 (RCE2) was established [3]. In these experiments, primarily screen content coding for applications such as remote desktop and wireless display is targeted. However lossless compression is also highly relevant for professional applications such as medical imaging, automotive vision, and digital cinema, where long-term archiving of the produced visual content is needed. In the Range Extension development, sample-based prediction methods provide considerable gain in compression performance. Therefore, SAP-HV from [4] has already been adopted into the current release of the reference software for Range Extension development. SAP-HV replaces the horizontal and vertical block-based prediction mode of HEVC by a sample-based horizontal and vertical prediction method. Further promising sample-wise prediction tools are now considered in RCE2, i.e., a simplified sample-base weighted prediction in Test C.3 [6] and a local neighborhood-based prediction method in Test C.4 [5].

# Algorithmic description

The algorithm in Test C.3 performs a sample-based weighted prediction, which replaces the regular PLANAR prediction of the HEVC standard. The algorithm from [2] has been simplified to avoid divisions and long look-up tables. Also the template has been reduced to a two-pixel-template. For prediction of the current sample, the similarity of the four direct neighbouring pixels to the current pixel is determined and based on the estimated similarity a weighted averaging of the neighbouring pixels yields the predicted value. A detailed explanation of the simplified SWP algorithm can be found in [6].

The method in Test C.4. performs a local neighborhood-based prediction which replaces the regular DC mode of the HEVC standard. A three sample neighborhood (top, left and top-left) is used to derive the prediction for the current sample. Even though the prediction for each sample may be different, the method has the additional attribute that all the pixels in a row or a column of a block may be decoded in parallel.

# Simulations

The proposed methods were implemented into HEVC range extension software HM10.1_RExt3.0 modified for lossless coding. Test sequences and common test conditions as defined in [3] were used for the tests. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs.

The summaries of the simulation results are illustrated in the following tables. Compression ratios, relative bit-rate savings, and encoder as well as decoder runtime are shown in these tables for the simulated configurations. Complete simulation results are attached in a separate Excel sheet.

Table 1 – Summary of RCE2 Test D.3 for All Intra Main configuration settings

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 5.2 | 5.5 | -3.4% |
| Class B | 2.2 | 2.3 | -2.9% |
| SC RGB 444 | 10.1 | 10.8 | -6.2% |
| SC YUV 444 | 11.4 | 12.1 | -4.8% |
| RangeExt | 2.4 | 2.4 | -1.4% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2 - Summary of RCE2 Test D.3 for Random Access Main configuration settings

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 31.7 | 32.4 | -1.8% |
| Class B | 2.6 | 2.6 | -0.7% |
| SC RGB 444 | 100.4 | 107.6 | -4.5% |
| SC YUV 444 | 128.9 | 135.4 | -3.4% |
| RangeExt | 2.5 | 2.5 | -0.4% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 99% |  |  |

Table 3 - Summary of RCE2 Test D.3 for Low delay B Main configuration settings

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 49.8 | 50.5 | -1.5% |
| Class B | 2.6 | 2.6 | -0.6% |
| SC RGB 444 | 381.6 | 415.2 | -4.4% |
| SC YUV 444 | 325.6 | 347.7 | -3.3% |
| RangeExt | 2.5 | 2.5 | -0.3% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 91% |  |  |

# Complexity analysis and WD text changes

Complexity analysis of the proposed tools is provided separately in the corresponding documents, i.e., local neighborhood-based prediction method is analyzed in [5] whereas the simplified SWP predictor is analyzed in [6]. The WD text changes are attached in a separate document.

# Conclusion

Efficient lossless compression is desired in many different applications such as medical imaging, automotive vision and digital cinema. The proposed combination of two prediction techniques provides significant bitrate reductions over the current HM10.1_RExt3.0 reference software for lossless configuration, which bypasses transform, quantization, and in-loop filtering. Therefore, we propose to adopt the proposed sample-based prediction methods into the test model and working draft of HEVC range extension.

# References

- R. Joshi, J. Sole, M. Karczewicz, “Non-RCE2: Modification to the DC intra prediction mode for lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M0291, 13th JCT-VC Meeting, Incheon, KR, Apr. 2013

- P. Amon, A. Hutter, E. Wige, A. Kaup, “RCE2: Sample-based weighted intra prediction for lossless coding”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M0052, 13th JCT-VC Meeting, Incheon, KR, Apr. 2013

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M1122, 13th JCT-VC Meeting, Incheon, KR, Apr. 2013.

- M. Zhou, M. Budagavi, “RCE2: Experimental results on Test 3 and Test 4”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-M0056, 13th JCT-VC Meeting, Incheon, KR, Apr. 2013

- R. Joshi, J. Sole, M. Karczewicz, “RCE2 subtest C.4: Replacement of DC mode for transform-skip and transform-bypass blocks”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-N0053, 14th JCT-VC Meeting, Vienna, AT, July 2013

- P. Amon, A. Hutter, E. Wige, A. Kaup, “RCE2: Simplified sample-based weighted prediction (Test C.3)”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-N0071, 14th JCT-VC Meeting, Vienna, AT, July 2013

# Patent rights declaration(s)

Siemens and Universität Erlangen-Nürnberg may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).