| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0074 |
| --- | --- |

| Title: | RCE2: Experimental results for Test C.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mark Block D, Centre House 56 Wood Lane, W12 7SB London, UK | Email: | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document reports the experimental results for the Test C.1: residual DPCM for inter coded blocks. Residual DPCM applies a sample-by-sample prediction along two directions: horizontal and vertical. For each coded block, the Sum of Absolute Difference (SAD) on the predicted residuals is computed for the following three modes: no residuals DPCM, residuals DPCM along the horizontal direction and residuals DPCM along the vertical direction. The mode with the lowest SAD is selected as the best and signalled to the decoder. For lossless encoding, inter residual DPCM is tested at two different levels: Coding Unit (CU) and Transform Unit (TU), leading to two different implementations denoted as CU-based and TU-based. The CU-based implementation applies the residual DPCM over the samples contained in a CU and signals the prediction direction on a per CU basis. The TU-based implementation applies residual DPCM over all the samples contained in a TU and therefore signals the prediction direction on a per TU basis implementations of inter residual DPCM. Moreover, for lossy coding, inter residual DPCM is applied only on those transform units where the transform skip flag is equal to one. For the test conditions of RCE2 the following results are reported:

- For lossless and CU-based implementation, average bit-rate savings of up to 3.4% and 5.5% are obtained in random access and low delay, respectively.

- For lossless and TU-based implementation, average bit-rate savings of up to 3.4% and 5.7% are obtained in random access and low delay, respectively.

- For lossy coding, average BD-rate of up to 1.3% and 3.3% are obtained in random access and low delay, respectively.

# Introduction

Lossless coding is enabled in HEVC via the cu_transquant_bypass_flag syntax element signalled at coding unit level. When this flag is equal to one, transform, quantization and in-loop filtering are bypassed and the residuals are directly entropy encoded. To better exploit the spatial redundancy among residuals, some sample-by-sample prediction methods have been proposed in documents JCTVC-M0056 and JCTVC-M0079. At the Incheon meeting Sample Adaptive Prediction (SAP) along the Horizontal and Vertical (SAP H/V) direction was adopted in the context of the RExt activities. SAP H/V is conceptually identical to the Residual DPCM (RDPCM) proposed in JCTVC-M0079. The main idea of both SAP H/V and RDPCM is to exploit the spatial redundancy among samples inside intra coded blocks. The sample spatial redundancy can be also present in inter coded block, namely when high frequency details (e.g. sharp edges) are poorly predicted by motion compensation. Inter RDPCM applies residual DPCM along the horizontal and vertical directions and computes the Sum of Absolute Difference (SAD) on the predicted samples. The obtained SAD values are compared with the SAD computed over the residuals when no RDPCM is applied. The mode with the lowest SAD is selected and signalled to the decoder. The RDPCM described for lossless coding can be also extended to inter coded blocks in lossy coding. In this case the blocks where RDPCM is applied are the ones where transform is skipped. This document describes the inter RDPCM method applied to both lossless and lossy coding and reports the experimental results according to the RCE2 test conditions. The document also analyses the computational complexity associated to the method and provides the changes required by inter RDPCM to the current working draft text for RExt.

# Residual DPCM for inter coded blocks

This section describes the inter RDPCM method tested in the RCE2. The method for the lossless coding case is introduced first followed by the lossy coding case.

## Residual DPCM for inter coded blocks and lossless coding

Over the motion compensated residuals RDPCM is applied separately along the horizontal and vertical directions, leading to r’hor and r’ver, which at the encoder side are defined as follows:

r'hor(i,j)=r(i,j)j=0 and 0≤i≤M−1r(i,j)−r(i,j−1)0<j≤N−1 and 0≤i≤M−1

r'ver(i,j)=r(i,j)i=0 and 0≤j≤N−1r(i,j)−r(i−1,j)0<i≤M−1 and 0≤j≤N−1,

where M and N denote the number of rows and columns for a coded block, respectively. After RDPCM is performed the SAD is computed over r’hor, r’ver and compared to the SAD associated to the motion compensated residuals (i.e. when no RDPCM is applied). There are therefore three modes available for RDPCM over inter coded blocks: RDPCM-off, RDPCM-horizontal and RDPCM-vertical. The option with the minimum SAD is selected as the best and communicated to the decoder. The execution flow for the selection of the best inter RDPCM mode is depicted in . The signalling of the selected RDPCM mode is done using two binary flags: one to signal whether RDPCM should be applied over the residuals and, if this is the case, the second to signal which RDPCM direction is selected. Each binary flag has one context per channel (i.e. luma and chroma) and therefore there are a total of 4 contexts required to signal the RDPCM mode. In this document the block size where inter RDPCM is applied refers to either the coding unit or transform unit. When inter RDPCM is applied over coding units the selection of the best mode and residuals prediction is done during the CU partitioning (i.e. inside the “encodeResAndCalcRdInterCU” function in the HM source code). Conversely, for block size equal to the TU, the best mode selection is performed during the RQT partitioning (i.e. inside the “xEstimateResidualQT” function in the HM source code).

At the decoder side, if the inter RDPCM mode for a coded block is horizontal the motion compensated residual at sample location (i,j), r(i,j) is obtained as follows:

r(i,j)=k=0jr'hor(i,k)

Conversely, if the inter RDPCM mode is vertical, r is obtained as follows:

r(i,j)=k=0ir'ver(k,j)

As mentioned above, the block size where inter RDPCM is applied can be either coding unit or transform unit.

## Residual DPCM for inter coded blocks and lossy coding

When lossy coding is used, inter RDPCM is applied at TU level and only for those TUs where the transform is skipped and associated normalisation is applied (i.e. 4×4 transform units). As in the lossless case, RDPCM is applied over a N×N (N = 4) block separately along the horizontal and vertical directions leading to the residuals rhor and rver. These residuals are obtained by applying RDPCM using as predictor the adjacent (horizontally or vertically) residual reconstructed after inverse quantisation and denoted as r(i,j). The obtained residual is then quantised to obtain rhor or rver depending on which prediction direction is selected. More specifically, rhor and rver are therefore defined as follows:

rhor(i,j)=Q(r(i,j))j=0 and 0≤i≤N−1Q(r(i,j)−r(i,j−1))0<j≤N−1 and 0≤i≤N−1

rver(i,j)=Q(r(i,j))i=0 and 0≤j≤N−1Q(r(i,j)−r(i−1,j))0<i≤N−1 and 0≤j≤N−1,

where Q denotes the forward quantisation operation. Over both the residuals rhor and rver the SAD is computed and compared to the SAD associated to the residuals where no RDPCM is applied. The mode with the minimum SAD, i.e. RDPCM-off, RDPCM-horizontal or RDPCM-vertical, is selected as the best and transmitted to the decoder. As for the lossless case, also here two flags are used to signal the RDPCM mode and one context per channel is used for both flags. The processing required by inter RDPCM is performed after the up scaling operated by transform skip. In this way the rounding errors introduced by the down scaling in inverse transform skip are avoided and more accurate sample predictors can be used for RDPCM. In the current implementation, when RDPCM-horizontal or RDPCM-vertical is selected as the best mode, the Rate Distortion Optimised Quantisation (RDOQ) is not performed as the sample processing order used in RDOQ is different from the order for both horizontal and vertical. This different order would lead to some drift inside the block which may decrease the rate distortion performance. Over those TUs where RDPCM-off is selected as best mode, RDOQ is applied as currently implemented in the HM software. At the decoder side, the reconstructed residuals after inverse quantisation are added up according to the selected RDPCM mode to obtain the reconstructed residuals r(i,j):

r(i,j)=k=0jQ−1(rhor(i,k))

r(i,j)=k=0iQ−1(rver(k,j)),

where Q-1 denotes the inverse quantisation operation. Finally the shift down for inverse transform skip is performed to obtain the residuals which will be added back to the motion compensated predictor.

# Test results

The inter RDPCM has been implemented in the HM10.1-rext-3.0 software and tested over the test material and coding conditions specified for RCE2 (document JCTVC-M1122_r1). This section reports the results for both the lossless and lossy case.

## Results for lossless inter RDPCM CU-based implementation

| Configuration Class | Random Access (RA) | Low Delay (LD) |
| --- | --- | --- |
|  | Bit-rate savings [%] | Bit-rate savings [%] |
| Class F | -3.4% | -5.5% |
| Class B | -0.4% | -0.5% |
| SC RGB 444 | -2.7% | -5.1% |
| SC YUV 444 | -1.7% | -3.4% |
| Range Ext. | -0.2% | -0.2% |
| Enc. Time [%] | 104% | 103% |
| Dec. Time [%] | 98% | 97% |

## Results for lossless inter RDPCM TU-based implementation

| Configuration Class | Random Access (RA) | Low Delay (LD) |
| --- | --- | --- |
|  | Bit-rate savings [%] | Bit-rate savings [%] |
| Class F | -3.4% | -5.5% |
| Class B | -0.4% | -0.5% |
| SC RGB 444 | -3.3% | -5.7% |
| SC YUV 444 | -2.0% | -4.2% |
| Range Ext. | -0.2% | -0.3% |
| Enc. Time [%] | 104% | 102% |
| Dec. Time [%] | 98% | 95% |

## Results for lossy inter RDPCM

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.1% | -1.0% | -1.1% | -1.3% | -1.2% | -1.2% |
| SC YUV 444 | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | 0.0% |
| SC RGB 444 | -3.3% | -3.1% | -3.2% | -3.2% | -3.2% | -3.2% |
| SC YUV 444 | -1.1% | -1.2% | -1.3% | -1.2% | -1.3% | -1.4% |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

# Complexity analysis

The complexity related to the tested inter residual DPCM is now analysed both in terms of throughput and number of operations needed for each pixel.

## Throughput analysis

In the lossless coding case, when the prediction mode is horizontal, the reconstruction of one pixel p[x][y] is done according to the following:

p[x][y]=p[x][−1]+k=0yr'hor[x][k].

Or according to the following formula when the prediction mode is vertical:

p[x][y]=p[−1][y]+k=0xr'ver[k][y].

In both prediction modes, each pixel does not depend on the adjacent pixel availability therefore at both encoder and decoder the inter residual DPCM can be fully parallelised at CU or TU level depending on the selected implementation.

For the lossy case, the same arguments pointed out for the lossless case are valid provided that inverse quantisation is performed before.

## Number of operations analysis

For a block of size N×N, over each row or column the number of additions required to reconstruct the samples are equal to sample position index (i.e. zero additions for the sample at position 0, one addition for sample at position 1 and so on). On average the number of additions required to reconstruct the residues is equal to (N - 1) / 2.

# Text changes

See attached documents.

# Conclusions

The results for inter RDPCM tested in RCE2 on both lossless and lossy coding are presented. CU and TU-based implementations are provided for lossless coding which bring average bitrate reductions of up to 5.7% in LD and TU-based implementation. The inter RDPCM tool proves therefore to be effective over screen content videos on both lossless and lossy coding. It is therefore suggested its adoption in the draft text for HEVC-RExt.

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).