| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0074 |
| --- | --- |

| Title: | Draft text changes for Test C.1 in RCE2 (lossy inter RDPCM) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mark Block D, Centre House 56 Wood Lane, W12 7SB London, UK | Email: | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document reports the changes required by lossy inter residual DPCM tested in RCE2 (Test C.1). The proposed text changes are based on the document JCTVC-L1005 (file JCTVC-M1005_v1.doc). The required additions and modifications are highlighted in yellow. It is proposed to modify Section 7.3.8.11 “Residual coding syntax” and Section 7.4.9.11 “Residual coding semantics” to signal the inter residual DPCM mode selected. Section 8.6.2 “Scaling and transformation process” is modified to account for the processing required by inter residual DPCM while Section 8.6.6 “Inter residual transform-bypass decoding process” is added to specify this processing. Finally, it is proposed to modify Table 94, Table 932 and Table 937.

# Changes to residual coding syntax and semantics

#### 7.3.8.11		Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize = = 2 ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) { |  |
| inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| inter_rdpcm_dir_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| lastScanPos = 16 |  |
| lastSubBlock = ( 1 << ( log2TrafoSize − 2 ) ) * ( 1 << ( log2TrafoSize − 2 ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = 16 |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 1 ] |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| for( n = ( i = = lastSubBlock ) ? lastScanPos − 1 : 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| } |  |
| firstSigScanPos = 16 |  |
| lastSigScanPos = −1 |  |
| numGreater1Flag = 0 |  |
| lastGreater1ScanPos = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if( numGreater1Flag < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numGreater1Flag++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && lastGreater1ScanPos = = −1 ) |  |
| lastGreater1ScanPos = n |  |
| } |  |
| if( lastSigScanPos = = −1 ) |  |
| lastSigScanPos = n |  |
| firstSigScanPos = n |  |
| } |  |
| } |  |
| signHidden = ( lastSigScanPos − firstSigScanPos > 3 && !cu_transquant_bypass_flag ) |  |
| if( transform_skip_flag[ x0 ][ y0 ][ cIdx ] && CuPredMode[ x0 ][ y0 ] == MODE_INTER && inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| signHidden = 0 |  |
| if( lastGreater1ScanPos != −1 ) |  |
| coeff_abs_level_greater2_flag[ lastGreater1ScanPos ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] && ( !sign_data_hiding_enabled_flag ¦ ¦ !signHidden ¦ ¦ ( n != firstSigScanPos ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbsLevel = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = lastGreater1ScanPos) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_enabled_flag && signHidden ) { |  |
| sumAbsLevel += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( ( n = = firstSigScanPos ) && ( ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| numSigCoeff++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.9.10		Residual coding semantics

...

inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] indicates whether the inter residuals prediction process specified in clause 8.6.6 is applied to the associated transform block or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that the process indicated in clause 8.6.6 is applied to the current transform block. inter_rdpcm_flag [ x0 ][ y0 ][ cIdx ] equal to 0 specifies that the process indicated in clause 8.6.6 is not applied to the current transform block. When inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

inter_rdpcm_dir_flag[ x0 ][ y0 ][ cIdx ] indicates the direction where the inter residuals prediction process specified in clause 8.6.6 is applied. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. inter_rdpcm_dir_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that the direction where the process specified in clause 8.6.6 is vertical. inter_rdpcm_dir_flag[ x0 ][ y0 ][ cIdx ] equal to 1 specifies that the direction where the process specified in clause 8.6.6 is horizontal.

...

# Changes to scaling and transform process at transform unit level

8.6.2	Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Qp′Y			(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr			(8-3)

The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nTbS)x(nTbS) array r is set equal to the (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTER, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- If inter_rdpcm_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the inter residual transform-bypass decoding process as specified in subclause 8.6.6 is invoked with the size of the transform block nTbS, the inter_rdpcm_dir_flag[ xTbY ][ yTbY ][ cIdx ], and the (nTbS)x(nTbS) array d as the inputs, and the output is the (nTbS)x(nTbS) array of residual samples r.

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( d[ x ][ y ]  <<  7 )	(8-4)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

- The variable bdShift is derived as follows:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-5)

- The residual sample values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-6)

# Changes required by the inter residual DPCM processing

8.6.6	Inter residual transform-bypass decoding process

This process is invoked when transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, CuPredMode[ xCb ][ yCb ] is not equal to MODE_INTRA, and the flag inter_rdpcm_flag[ xCb ][ yCb ][ cIdx ] is equal to 1 for the considered value of the cIdx array index. The process for the Cb and Cr components is applied in the same way as for the luma component.

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

	a variable inter_rdpcm_dir_flag specifying the direction where the inter residuals prediction process should is applied,

	an (nTbS)x(nTbS) array d with elements d[ x ][ y ] which is either an array relating to a residual transform-bypass block of the luma component or an array relating to a residual transform-bypass block of the Cb and Cr component.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ] containing the results of the inter residual transform-bypass decoding process.

Let f be a temporary (nTbS)x(nTbS) array with elements f[ x ][ y ], which are derived by:

f[ x ][ y ] = d[ x ][ y ]   with x=0.. nTbS -1 and y=0.. nTbS -1

Depending on the inter_rdpcm_dir_flag, the following apply:

If inter_rdpcm_dir_flag is equal to 0 (horizontal direction) the array d is modified as follows:

d[ x ][ y ] = Clip3-32768,32767,k=0yf[x][k] with x=0..nCurrSw -1 and y=0..nCurrSh -1

Otherwise (inter_rdpcm_dir_flag is equal to 1, vertical direction) the array d is modified as follows:

d[ x ][ y ] = Clip3-32768,32767,k=0xf[k][y] with x=0..nCurrSw -1 and y=0..nCurrSh -1

Finally the elements r[ x ][ y ] are derived as follows:

r[ x ][ y ] = ( d[ x ][ y ]  <<  7 ) with x=0..nCurrSw -1 and y=0..nCurrSh -1

# Changes required to the initialization process for context variables

The following changes to Table 9-1 are required for the introduction of the inter_rdpcm_flag and inter_rdpcm_dir_flag binary flags. Moreover, one initialization table (Table 9X) is added.

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| sao( ) | sao_merge_left_flag sao_merge_up_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sao_type_idx_luma sao_type_idx_chroma | Error: Reference source not found | 0 | 1 | 2 |
| coding_quadtree( ) | split_cu_flag[ ][ ] | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |
| prediction_unit( ) | merge_flag[ ][ ] | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx[ ][ ] | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc[ ][ ] | Error: Reference source not found |  | 0..4 | 5..9 |
|  | ref_idx_l0[ ][ ], ref_idx_l1[ ][ ] | Error: Reference source not found |  | 0..1 | 2..3 |
|  | mvp_l0_flag[ ][ ], mvp_l1_flag[ ][ ] | Error: Reference source not found |  | 0 | 1 |
| transform_tree( ) | split_transform_flag[ ][ ][ ] | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | cbf_luma[ ][ ][ ] | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb[ ][ ][ ], cbf_cr[ ][ ][ ] | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | Error: Reference source not found |  | 0 | 2 |
|  | abs_mvd_greater1_flag[ ] | Error: Reference source not found |  | 1 | 3 |
| transform_unit( ) | cu_qp_delta_abs | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
| residual_coding( ) | transform_skip_flag[ ][ ][ 0 ] | Table 9- | 0 | 1 | 2 |
|  | transform_skip_flag[ ][ ][ 1 ] transform_skip_flag[ ][ ][ 2 ] | Table 9- | 3 | 4 | 5 |
|  | inter_rdpcm_flag[ ][ ][ 0 ] | Table 9X |  | 0 | 1 |
|  | inter_rdpcm_flag[ ][ ][ 1 ] inter_rdpcm_flag[ ][ ][ 2 ] | Table 9X |  | 2 | 3 |
|  | inter_rdpcm_dir_flag[ ][ ][ 0 ] | Table 9X |  | 0 | 1 |
|  | inter_rdpcm_dir_flag[ ][ ][ 1 ] inter_rdpcm_dir_flag[ ][ ][ 2 ] | Table 9X |  | 2 | 3 |
|  | last_sig_coeff_x_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..53 |
|  | last_sig_coeff_y_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..53 |
|  | coded_sub_block_flag[ ][ ] | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | sig_coeff_flag[ ][ ] | Error: Reference source not found | 0..41 | 42..83 | 84..125 |
|  | coeff_abs_level_greater1_flag[ ] | Error: Reference source not found | 0..23 | 24..47 | 48..71 |
|  | coeff_abs_level_greater2_flag[ ] | Error: Reference source not found | 0..5 | 6..11 | 12..17 |

Table 9-X – Values of initValue for ctxIdx of inter_rdpcm_flag and inter_rdpcm_dir_flag

| Initialization variable | ctxIdx of inter_rdpcm_flag and inter_rdpcm_dir_flag |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| initValue | 139 | 139 | 139 | 139 |

# Changes required to the binarization process

Table 9-2 is changed to define the binarization process for the inter_rdpcm_flag and inter_rdpcm_dir_flag flags.

| Table 9-2 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| slice_segment_data( ) | end_of_slice_segment_flag | FL | cMax = 1 |
|  | end_of_sub_stream_one_bit | FL | cMax = 1 |
| sao( ) | sao_merge_left_flag | FL | cMax = 1 |
|  | sao_merge_up_flag | FL | cMax = 1 |
|  | sao_type_idx_luma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_type_idx_chroma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_offset_abs[ ][ ][ ][ ] | TR | cMax = ( 1 << ( Min( bitDepth, 10 ) − 5 ) ) − 1, cRiceParam = 0 |
|  | sao_offset_sign[ ][ ][ ][ ] | FL | cMax = 1 |
|  | sao_band_position[ ][ ][ ] | FL | cMax = 31 |
|  | sao_eo_class_luma | FL | cMax = 3 |
|  | sao_eo_class_chroma | FL | cMax = 3 |
| coding_quadtree( ) | split_cu_flag[ ][ ] | FL | cMax = 1 |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | - |
|  | rqt_root_cbf | FL | cMax = 1 |
| prediction_unit( ) | merge_flag[ ][ ] | FL | cMax = 1 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
|  | inter_pred_idc[ x0 ][ y0 ] | Error: Reference source not found | nPbW, nPbH |
|  | ref_idx_l0[ ][ ] | TR | cMax = num_ref_idx_l0_active_minus1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = num_ref_idx_l1_active_minus1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
| transform_tree( ) | split_transform_flag[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_luma[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cb[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cr[ ][ ][ ] | FL | cMax = 1 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
| transform_unit( ) | cu_qp_delta_abs | Error: Reference source not found | - |
|  | cu_qp_delta_sign_flag | FL | cMax = 1 |
| residual_coding( ) | transform_skip_flag[ ][ ][ ] | FL | cMax = 1 |
|  | inter_rdpcm_flag[ ][ ][ ] | FL | cMax = 1 |
|  | inter_rdpcm_dir_flag[ ][ ][ ] | FL | cMax = 1 |
|  | last_sig_coeff_x_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) − 1 ) |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) − 1 ) |
|  | coded_sub_block_flag[ ][ ] | FL | cMax = 1 |
|  | sig_coeff_flag[ ][ ] | FL | cMax = 1 |
|  | coeff_abs_level_greater1_flag[ ] | FL | cMax = 1 |
|  | coeff_abs_level_greater2_flag[ ] | FL | cMax = 1 |
|  | coeff_abs_level_remaining[ ] | Error: Reference source not found | current sub-block scan index i, baseLevel |
|  | coeff_sign_flag[ ] | FL | cMax = 1 |

# Changes required to the derivation of context index variables and bypass

The following rows are added to Table 9-3 for the inter_rdpcm_flag and inter_rdpcm_dir_flag flags.

| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_slice_segment_flag | terminate | na | na | na | na | na |
| end_of_sub_stream_one_bit | terminate | na | na | na | na | na |
| sao_merge_left_flag | 0 | na | na | na | na | na |
| sao_merge_up_flag | 0 | na | na | na | na | na |
| sao_type_idx_luma | 0 | bypass | na | na | na | na |
| sao_type_idx_chroma | 0 | bypass | na | na | na | na |
| sao_offset_abs[ ][ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_offset_sign[ ][ ][ ][ ] | bypass | na | na | na | na | na |
| sao_band_position[ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_eo_class_luma | bypass | bypass | bypass | na | na | na |
| sao_eo_class_chroma | bypass | bypass | bypass | na | na | na |
| split_cu_flag[ ][ ] | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| cu_transquant_bypass_flag | 0 | na | na | na | na | na |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |
| part_mode log2CbSize > MinCbLog2SizeY | 0 | 1 | 3 | bypass | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| prev_intra_luma_pred_flag[ ][ ] | 0 | na | na | na | na | na |
| mpm_idx[ ][ ] | bypass | bypass | na | na | na | na |
| rem_intra_luma_pred_mode[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| intra_chroma_pred_mode[ ][ ] | 0 | bypass | bypass | na | na | na |
| rqt_root_cbf | 0 | na | na | na | na | na |
| merge_flag[ ][ ] | 0 | na | na | na | na | na |
| merge_idx[ ][ ] | 0 | bypass | bypass | bypass | na | na |
| inter_pred_idc[ x0 ][ y0 ] | ( nPbW + nPbH ) != 12 ? CtDepth[ x0 ][ y0 ] : 4 | 4 | na | na | na | na |
| ref_idx_l0[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| ref_idx_l1[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| mvp_l0_flag[ ][ ] | 0 | na | na | na | na | na |
| mvp_l1_flag[ ][ ] | 0 | na | na | na | na | na |
| split_transform_flag[ ][ ][ ] | 5 − log2TrafoSize | na | na | na | na | na |
| cbf_cb[ ][ ][ ] | trafoDepth | na | na | na | na | na |
| cbf_cr[ ][ ][ ] | trafoDepth | na | na | na | na | na |
| cbf_luma[ ][ ][ ] | trafoDepth = = 0 ? 1 : 0 | na | na | na | na | na |
| abs_mvd_greater0_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_minus2[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |
| cu_qp_delta_abs | 0 | 1 | 1 | 1 | 1 | bypass |
| cu_qp_delta_sign_flag | bypass | na | na | na | na | na |
| transform_skip_flag[ ][ ][ ] | 0 | na | na | na | na | na |
| last_sig_coeff_x_prefix | 0..17 (subclause Error: Reference source not found) |  |  |  |  |  |
| last_sig_coeff_y_prefix | 0..17 (subclause Error: Reference source not found) |  |  |  |  |  |
| last_sig_coeff_x_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_y_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| coded_sub_block_flag[ ][ ] | 0..3 (subclause Error: Reference source not found) | na | na | na | na | na |
| sig_coeff_flag[ ][ ] | 0..41 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_greater1_flag[ ] | 0..23 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_greater2_flag[ ] | 0..5 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_remaining[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| coeff_sign_flag[ ] | bypass | na | na | na | na | na |
| inter_rdpcm_flag[ ][ ][ ] | 0 | na | na | na | na | na |
| inter_rdpcm_dir_flag[ ][ ][ ] | 0 | na | na | na | na | na |