| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0075 |
| --- | --- |

| Title: | Non-RCE2: Complexity reduction for inter residual DPCM in lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mark Block D, Centre House 56 Wood Lane, W12 7SB London, UK | Email: | matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

The residual DPCM on inter coded residuals has been proposed in document JCTVC-M0442 for lossless coding. The main idea is to apply residual DPCM on top of the motion compensated residuals and along the horizontal and vertical directions. For each coding block (either CU or TU) three different modes are tested: no residual DPCM, residual DPCM along the horizontal and residual DPCM along the vertical direction. The mode which brings the lowest sum of absolute difference is selected and communicated to the decoder. If residual DPCM is applied, the reconstruction of the samples at the decoder can be done by adding the received data according to the selected direction. In order to limit the number of operations per sample, namely at the decoder side, it is proposed to divide the rows or columns where residual DPCM is applied into chunks with length equal to 8. In this contribution the proposed chunking of the residuals is implemented in two implementations for inter residuals DPCM. The first implementation (denoted as CU-based) applies the residual DPCM over the samples contained in a CU and signals the prediction direction on a per CU basis. Conversely, the second implementation (denoted as TU-based) applies residual DPCM over all the samples contained in a TU and therefore signals the prediction direction on a per TU basis implementations of inter residual DPCM. Over the RCE2 common test conditions, compression efficiency gains of up to 4.6% are reported for the CU-based implementation while compression efficiency gains of up to 5.0% are reported for the TU-based implementation.

# Introduction

Inter Residual DPCM (RDPCM) for lossless coding has been proposed in document JCTVC-M0442 to exploit the redundancy which may still exist on motion compensated blocks containing high frequency details. Inter RDPCM is applied along the horizontal and vertical directions and also the no RDPCM case is allowed for those motion compensated residuals which are uncorrelated along the two aforementioned directions. Therefore, for each coding block (i.e. CU or TU) there are three possible modes: no RDPCM, RDPCM horizontal and RDPCM vertical. The mode which leads to the lowest Sum of Absolute Differences (SAD) is selected as the best and communicated to the decoder. At the decoder side, the residual samples are reconstructed by adding up the entropy decoded data according to the selected directions. The samples in large blocks lying at the rightmost columns or bottom rows require a high number of operations per sample to get the reconstructed value. In order to limit the number of operations per sample, it is proposed to group rows or columns of residuals into chunks with length equal to 8. In this way the residual sample prediction process is stopped every 8 samples and the average number of operations per sample is reduced. In the following the main idea for the proposed residual chunking is presented followed by the experiment results and complexity analysis. Moreover, also the text changes are provided in a separate document.

# Chunking of inter residuals

In order to limit the number of operations per sample at the decoder side, the residuals are subdivided in chunks of a predetermined length L in the direction of prediction, so that no residual to be reconstructed requires additions of more than L values. After the prediction of (L - 1) consecutive samples in a row or column, the DPCM prediction chain is halted: the Lth sample is not predicted from the previous samples but instead is directly signalled in the bitstream. The Lth sample is the start of the second chunk of residual samples. This subdivision of the residual block is introduced to limit the number of operations required to fully reconstruct a sample at the decoder side. In this proposal, the chunk length L is set to 8. With the proposed residual chunking the RDPCM process at the encoder is now defined as:

r'hor(i,j)=r(i,j)if i=0,L,2⋅L,...r(i,j)−r(i,j−1)otherwise,

r'ver(i,j)=r(i,j)if j=0,L,2⋅L,...r(i,j)−r(i−1,j)otherwise.

At the decoder side, the reconstruction is done according to the following:

rhor(i,j)=k=jL⋅Ljr'hor(i,k)   ,

rver(i,j)=k=iL⋅lir'ver(k,j).

where  denotes the floor operator which returns the integer less than or equal to its argument. An example of how the proposed residual DPCM chunking is applied is represented in the following figure where residual DPCM is applied along the vertical direction.

# Test results

The proposed residual chunking been implemented in the HM10.1-rext-3.0 software and tested over the test material and coding conditions specified for RCE2 (document JCTVC-M1122_r1). The chunking is applied to two implementations for inter RDPCM: CU-based and TU-based.

## Results for inter RDPCM chunking and CU-based implementation

| Configuration Class | Random Access (RA) |  | Low Delay (LD) |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate savings [%] |  | Bit-rate savings [%] |  |
|  | Chunking off | Chunking on | Chunking off | Chunking on |
| Class F | -3.4% | -2.9% | -5.5% | -4.6% |
| Class B | -0.4% | -0.4% | -0.5% | -0.5% |
| SC RGB 444 | -2.7% | -2.6% | -5.1% | -4.2% |
| SC YUV 444 | -1.7% | -1.5% | -3.4% | -3.1% |
| Range Ext. | -0.2% | -0.2% | -0.2% | -0.2% |
| Enc. Time [%] | 104% | 103% | 103% | 103% |
| Dec. Time [%] | 98% | 96% | 97% | 96% |

## Results for inter RDPCM chunking and TU-based implementation

| Configuration Class | Random Access (RA) |  | Low Delay (LD) |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate savings [%] |  | Bit-rate savings [%] |  |
|  | Chunking off | Chunking on | Chunking off | Chunking on |
| Class F | -3.4% | -3.0% | -5.5% | -4.7% |
| Class B | -0.4% | -0.4% | -0.5% | -0.5% |
| SC RGB 444 | -3.3% | -3.0% | -5.7% | -5.0% |
| SC YUV 444 | -2.0% | -1.9% | -4.2% | -3.8% |
| Range Ext. | -0.2% | -0.2% | -0.3% | -0.3% |
| Enc. Time [%] | 104% | 105% | 102% | 103% |
| Dec. Time [%] | 98% | 96% | 95% | 95% |

# Complexity analysis

The complexity related to the inter residual DPCM with chunking is now analysed in terms of number of operations needed for each pixel.

Accordingly, for a block with size N×N, over each row or column the number of additions required to reconstruct the samples are equal to sample position index (i.e. zero additions for the sample at position 0, one addition for sample at position 1 and so on). Given the chunk length equal to 8 in this proposal, the average the number of additions required to reconstruct the residues is equal to (L - 1) / 2, i.e. 3.5 operations per sample.

# Text changes

See attached documents.

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).