| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0079_r2 |
| --- | --- |

| Title: | Non-RCE2: Simplified sample based intra prediction for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianqing Zhu Wang Zheng Kimihiko Kazui | Tel: Email: | +86-21-63350606 ext. 5810 cheng@cn.fujitsu.com zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

Sample based intra prediction such as adopted SAP is verified to be an effective and low complexity scheme for lossless coding. In this contribution, simplified sample based intra prediction is proposed to reduce complexity of encoding and decoding for lossless coding. Only 13 out of 35 possible intra prediction modes are used for lossless coding. DC and planar modes are unchanged. The other 11 angular modes use the nearest reference sample’s value in a specific direction as prediction value rather than using a filter. The influence on bit-rate for the All-Intra main configuration and lossless coding is reported to be 0.7% gain and 0.2% loss for non screen content sequences of Class F and Class B respectively, 2.9% and 2.8% gain for screen content sequences in RGB and YUV formats respectively, and 2.1% gain for range extension sequences. Encoding and decoding time is reduced.

# Introduction

Sample based intra prediction such as adopted SAP[1] is verified to be an effective and low complexity scheme for lossless coding Compared with residual DPCM[2], Sample based intra prediction has less computation because residual DPCM has 1 more prediction process to get residuals. Either adopted SAP or residual DPCM is only applied in horizontal and vertical intra prediction. For those angles other than horizontal and vertical modes, filtering of reference samples is applied for intra prediction in current draft standard [3] so that the complexity is more than DPCM. In this contribution, a simple sample based intra prediction is proposed for multiple angles for lossless coding. It will reduce encoding and decoding complexity greatly in 2 aspects. Firstly, no filtering is applied to reference samples. Secondly, fewer modes are used.

# Algorithm

Encoding diagram of intra prediction for lossless is as Figure 1.

Figure 1 Diagram of encoding process of intra prediction for lossless

Prediction process for Mode 0 (DC) and mode 1(planar) is kept unchanged. For angular modes, prediction value is the nearest reference sample in each angle as shown in figure 2.

Figure 2 Prediction for angular modes

Let S[x][y] be original samples and PU size be NxN, x=-1..N+1, y=-1..N+1. Reference sample substitution for those locations “not available for intra prediction” is same as the draft standard [3].

Reference sample mapping is shown as Figure3.

Prediction block, predSample[x][y], is derived as follows, x=0..N-1, y=0..N-1.

Angle1:

predSample[x][y]=S[x−1][y+1],x=1..N−1,y=N−2S[x−1][y],x=1..N−1,y=N−1S[x−1][y+2]otherwise

Angle2:

predSample[x][y]=S[x−1][y],x=1..N−1,y==(N−1)S[x−1][y+1],Otherwise

Angle3:

predSample[x][y]=S[x−1][y+1],x=0S[x−1][y],x=1..N−1,y=N−1S[x−2][y+1]otherwise

Angle 4:

predSample[x][y]=S[x-1][y];

Figure 3 reference sample mapping

Angle 5:

predSample[x][y]=S[x−1][y−1],x==1S[x−2][y−1],Otherwise

Angle 6:

predSample[x][y]=S[x-1][y-1];

Angle 7:

predSample[x][y]=S[x−1][y−1],y==1S[x−1][y−2],Otherwise

Angle 8:

predSample[x][y]=S[x][y-1];

Angle 9:

predSample[x][y]=S[x+1][y−1],y=0S[x][y−1],x=N−1,y=1..N−1S[x+1][y−2]otherwise

Angle 10:

predSample[x][y]=S[x][y−1],x==N−1,y=1..N−1S[x+1][y−1],Otherwise

Angle 11:

predSample[x][y]=S[x+1][y−1],x=N−2,y=1..N−1S[x][y−1],x=N−1,y=1..N−1S[x+2][y−1]otherwise

# Experimental results

In this section, coding results for the sample based intra prediction are presented according to the test conditions used in RCE2 [4]. The anchor is HM 10.1-RExt-3.0 with the patch provided in [2] for lossless coding. Table 1 presents the coding results.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.7% |
| Class B | 2.2 | 2.2 | 0.2% |
| SC RGB 444 | 10.1 | 10.4 | -2.9% |
| SC YUV 444 | 11.1 | 11.6 | -2.8% |
| RangeExt | 2.3 | 2.3 | -2.1% |
| Enc Time[%] | 86% |  |  |
| Dec Time[%] | 99% |  |  |

Table 1. Simulation results for Simplified sample based intra prediction in lossless coding mode

KDDI’s contribution JCTVC-N0367 verifies the result. It is reported that the compression ratios of some test sequences in anchor encoding are different. Encoding time and decoding time are similar.

Conclusion

In this contribution, a simple sample based intra prediction is proposed to reduce complexity of encoding and decoding for lossless coding. The influence on bit-rate for the All-Intra main configuration and lossless coding is reported to be 0.7% gain and 0.2% loss for non screen content sequences of Class F and Class B respectively, 2.9% and 2.8% gain for screen content sequences in RGB and YUV formats respectively, and 2.1% gain for range extension sequences.

# Suggested text on the draft standard

It is suggested that HEVC Range Extensions Draft[1] is modified as follows. The part marked in yellow is new added.

8.4.1 Derivation process for luma intra prediction mode

Input to this process is a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

In this process, the luma intra prediction mode IntraPredModeY[ xPb ][ yPb ] is derived.

Table 8-1 specifies the value for the intra prediction mode and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..34 | INTRA_ANGULAR2..INTRA_ANGULAR34 |

IntraPredModeY[ xPb ][ yPb ] labelled 0..34 represents directions of predictions as illustrated in Figure 8-1 and Figure 8-2.

Figure 8-1 – Intra prediction mode directions if cu_transquant_bypass_flag is equal to 0 (informative)

Figure 8-2 – Intra prediction mode directions if cu_transquant_bypass_flag is equal to 1 (informative)

IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xPb − 1, yPb ) and ( xPb, yPb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the location ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

- Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

- Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..2 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is less than 2 (i.e. equal to INTRA_PLANAR or INTRA_DC), candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = INTRA_PLANAR	(8-1)

candModeList[ 1 ] = INTRA_DC		(8-2)

- If cu_transquant_bypass_flag is equal to 0

candModeList[ 2 ] = INTRA_ANGULAR26	(8-3)

-Otherwise,

candModeList[ 2 ] = INTRA_ANGULAR9                                             (8-18)

  - Otherwise, candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-19)

- If cu_transquant_bypass_flag is equal to 0

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 29 ) % 32 )	(8-20)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 32 )	(8-21)

-Otherwise,

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 8 ) % 11 )	(8-22)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 11 )	(8-23)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

  - candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-24)

candModeList[ 1 ] = candIntraPredModeB	(8-25)

  - If neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_PLANAR, candModeList[ 2 ] is set equal to INTRA_PLANAR,

  - Otherwise, if neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_DC, candModeList[ 2 ] is set equal to INTRA_DC,

  - Otherwise,

- If cu_transquant_bypass_flag is equal to 0

candModeList[ 2 ] is set equal to INTRA_ANGULAR26.

-Otherwise,

candModeList[ 2 ] is set equal to INTRA_ANGULAR9.

- IntraPredModeY[ xPb ][ yPb ] is derived by applying the following procedure:

- If prev_intra_luma_pred_flag[ xPb ][ yPb ] is equal to 1, the IntraPredModeY[ xPb ][ yPb ] is set equal to  candModeList[ mpm_idx ].

- Otherwise, IntraPredModeY[ xPb ][ yPb ] is derived by applying the following ordered steps:

- The array candModeList[ x ], x = 0..2 is modified as the following ordered steps:

  - When candModeList[ 0 ] is greater than candModeList[ 1 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 1 ] ) = Swap( candModeList[ 0 ], candModeList[ 1 ] )	(8-4)

  - When candModeList[ 0 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 2 ] ) = Swap( candModeList[ 0 ], candModeList[ 2 ] )	(8-5)

  - When candModeList[ 1 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 1 ], candModeList[ 2 ] ) = Swap( candModeList[ 1 ], candModeList[ 2 ] )	(8-6)

- IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

  - IntraPredModeY[ xPb ][ yPb ] is set equal to rem_intra_luma_pred_mode[ xPb ][ yPb ].

  - For i equal to 0 to 2, inclusive, when IntraPredModeY[ xPb ][ yPb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xPb ][ yPb ] is incremented by one.

8.4.3 Derivation process for chroma intra prediction mode

This process is only invoked when ChromaArrayType is not equal to 0.

Input to this process is a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

The variable modeIdx is derived using intra_chroma_pred_mode[ xCb ][ yCb ] and IntraPredModeY[ xCb ][ yCb ] as specified in Table 8-2 and 8-3.

The chroma intra prediction mode IntraPredModeC is derived as follows:

–	If ChromaArrayType is equal to 2, IntraPredModeC is set using modeIdx as specified in XXX.

–	Otherwise, IntraPredModeC is set equal to modeIdx.

Table 8-2 – Specification of modeIdx if if cu_transquant_bypass_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb ][ yCb ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 26 | 10 | 1 | X ( 0 <= X <= 34 ) |
| 0 | 34 | 0 | 0 | 0 | 0 |
| 1 | 26 | 34 | 26 | 26 | 26 |
| 2 | 10 | 10 | 34 | 10 | 10 |
| 3 | 1 | 1 | 1 | 34 | 1 |
| 4 | 0 | 26 | 10 | 1 | X |

Table 8-3 – Specification of modeIdx if cu_transquant_bypass_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | IntraPredModeY[ xCb ][ yCb ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 9 | 5 | 1 | X ( 0 <= X <= 12 ) |
| 0 | 12 | 0 | 0 | 0 | 0 |
| 1 | 9 | 12 | 9 | 9 | 9 |
| 2 | 5 | 5 | 12 | 5 | 5 |
| 3 | 1 | 1 | 1 | 12 | 1 |
| 4 | 0 | 9 | 5 | 1 | X |

Table 8-4 – Specification of intraPredModeC when ChromaArrayType is equal to 2 and cu_transquant_bypass_flag is equal to 0

| modeIdx | X <= 2 |  | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredModeC | X |  | 2 | 2 | 2 | 3 | 5 | 7 | 8 | 10 | 12 | 13 | 15 | 17 | 18 | 19 | 20 |
| modeIdx | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| IntraPredModeC | 21 | 22 | 23 | 23 | 24 | 24 | 25 | 25 | 26 | 27 | 27 | 28 | 28 | 29 | 29 | 30 | 31 |

Table 8-5 – Specification of intraPredModeC when ChromaArrayType is equal to 2 and cu_transquant_bypass_flag is equal to 1

| modeIdx | X <= 2 |  | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IntraPredModeC | X |  | 2 | 3 | 5 | 7 | 8 | 8 | 9 | 10 | 10 | 11 |  |  |  |  |  |

8.4.4.2.1 General intra sample prediction

Inputs to this process are:

–	a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The nTbS * 4 + 1 neighbouring samples p[ x ][ y ] that are constructed samples prior to the deblocking filter process, with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1, are derived as follows:

–	The neighbouring location ( xNbCmp, yNbCmp ) is specified by:

( xNbCmp, yNbCmp ) = ( xTbCmp + x, yTbCmp + y )	(8-7)

- The current luma location ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) are derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTbCmp, yTbCmp ) : ( xTbCmp * SubWidthC, yTbCmp * SubHeightC )	(8-8)

( xNbY, yNbY ) = ( cIdx  = =  0 ) ? ( xNbCmp, yNbCmp ) : ( xNbCmp * SubWidthC, yNbCmp * SubHeightC )	(8-9)

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) as inputs, and the output is assigned to availableN.

–	Each sample p[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, the sample p[ x ][ y ] is marked as "not available for intra prediction":

  - The variable availableN is equal to FALSE.

  - CuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA and constrained_intra_pred_flag is equal to 1.

- Otherwise, the sample p[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xNbCmp, yNbCmp ) is assigned to p[ x ][ y ].

When at least one sample p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process for intra sample prediction in subclause Error: Reference source not found is invoked with the samples p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1, nTbS, and cIdx as inputs, and the modified samples p[ x ][ y ] with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1 as output.

Depending on the value of predModeIntra, the following ordered steps apply:

- When cIdx is equal to 0 or ChromaArrayType is equal to 3, the filtering process of neighbouring samples specified in subclause Error: Reference source not found is invoked with the sample array p and the transform block size nTbS as inputs, and the output is reassigned to the sample array p.

- The intra sample prediction process according to predModeIntra applies as follows:

  - If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p and the transform block size nTbS as inputs, and the output is the predicted sample array predSamples.

  - Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode specified in subclause Error: Reference source not found is invoked with the sample array p, the transform block size nTbS, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

  - Otherwise, if cu_transquant_bypass_flag is equal to 0 , the corresponding intra prediction mode specified in subclause .1 is invoked with the intra prediction mode predModeIntra, the sample array p, the transform block size nTbS, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

  - Otherwise, the corresponding intra prediction mode specified in subclause .2 is invoked with the intra prediction mode predModeIntra, the sample array p, the transform block size nTbS, and the colour component index cIdx as inputs, and the output is the predicted sample array predSamples.

8.4.2.6 Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

8.4.2.6.1 Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34 if cu_transquant_bypass_flag is equal to 0

Inputs to this process are:

–	the intra prediction mode predModeIntra,

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Figure 8-2 illustrates the total 33 intra angles and Table 8-3 specifies the mapping table between predModeIntra and the angle parameter intraPredAngle.

Figure 8-2 – Intra prediction angle definition (informative)

Table 8-3 – Specification of intraPredAngle

| predModeIntra | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| intraPredAngle | - | 32 | 26 | 21 | 17 | 13 | 9 | 5 | 2 | 0 | −2 | −5 | −9 | −13 | −17 | −21 | −26 |
| predModeIntra | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| intraPredAngle | −32 | −26 | −21 | −17 | −13 | −9 | −5 | −2 | 0 | 2 | 5 | 9 | 13 | 17 | 21 | 26 | 32 |

Table 8-4 further specifies the mapping table between predModeIntra and the inverse angle parameter invAngle.

Table 8-4 – Specification of invAngle

| predModeIntra | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| invAngle | −4096 | −1638 | −910 | −630 | −482 | −390 | −315 | −256 |
| predModeIntra | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |
| invAngle | −315 | −390 | −482 | −630 | −910 | −1638 | −4096 | - |

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

–	If predModeIntra is equal or greater than 18, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nTbS	(8-10)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ],
	with x = −1..( nTbS * intraPredAngle )  >>  5	(8-11)

- Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nTbS + 1..2 * nTbS	(8-12)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

  - The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>  5		(8-13)

iFact = ( ( y + 1 ) * intraPredAngle ) & 31		(8-14)

  - Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-15)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-16)

  - When predModeIntra is equal to 26 (vertical), cIdx is equal to 0, nTbS is less than 32 and cu_transquant_bypass_flag is equal to 0, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-17)

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nTbS	(8-18)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],
	with x = −1..( nTbS * intraPredAngle )  >>  5	(8-19)

- Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nTbS + 1..2 * nTbS	(8-20)

- The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8-21)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8-22)

- Depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-23)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-24)

- When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0, nTbS is less than 32 and cu_transquant_bypass_flag is equal to 0, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8-25)

8.4.2.6.2 Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR12 if cu_transquant_bypass_flag is equal to 1

Inputs to this process are:

–	a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	the intra prediction mode predModeIntra,

–	the reconstructed samples resSamples[ x ][ y ],

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as following.

S[x][y] = resSamples[ xTbCmp+ x ][  yTbCmp +y ];

Depending on predModeIntra, one of the prediction modes specified in subclause 8.4.2.6.2.1 to  8.4.2.6.2.11 is invoked with Prediction samples array S, the transform block size nTbS as inputs, and the output is the predicted sample array predSamples. .

8.4.2.6.2.1 specification of INTRA_ANGULAR2 mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x−1][y+1],x=1..N−1,y=N−2S[x−1][y],x=1..N−1,y=N−1S[x−1][y+2]otherwise

8.4.2.6.2.2 specification of INTRA_ANGULAR3 mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x−1][y],x=1..N−1,y==(N−1)S[x−1][y+1],Otherwise

8.4.2.6.2.3 specification of INTRA_ANGULAR4 mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x−1][y+1],x=0S[x−1][y],x=1..N−1,y=N−1S[x−2][y+1]otherwise

8.4.2.6.2.4 specification of INTRA_ANGULAR5mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x-1][y];

8.4.2.6.2.5 specification of INTRA_ANGULAR6 mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x−1][y−1],x==1S[x−2][y−1],Otherwise

8.4.2.6.2.6 specification of INTRA_ANGULAR7 mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x-1][y-1];

8.4.2.6.2.7 specification of INTRA_ANGULAR8 mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x−1][y−1],y==1S[x−1][y−2],Otherwise

8.4.2.6.2.8 specification of INTRA_ANGULAR9 mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x][y-1];

8.4.2.6.2.9 specification of INTRA_ANGULAR10 mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x+1][y−1],y=0S[x][y−1],x=N−1,y=1..N−1S[x+1][y−2]otherwise

8.4.2.6.2.10 specification of INTRA_ANGULAR11 mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x][y−1],x==N−1,y=1..N−1S[x+1][y−1],Otherwise

8.4.2.6.2.11 specification of INTRA_ANGULAR12 mode

Inputs to this process are:

–	a variable nTbS specifying the transform block size,

–	Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

Variable N is assigned to be nTbS.

Prediction samples array S[x][y], x=-1.. N +1, y=-1.. N +1is derived as:

predSample[x][y]=S[x+1][y−1],x=N−2,y=1..N−1S[x][y−1],x=N−1,y=1..N−1S[x+2][y−1]otherwise

# References

- M. Zhou, “AHG7: Sample-based angular intra prediction for HEVC range extension,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0176, Jan. 2013.

- S. Lee, I.-K. Kim, and C. Kim, “AHG7: Residual DPCM for HEVC lossless coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0117, Jan. 2013.

- D.Flynn, J.Sole, and T.Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 3,” JCTVC-M1005, June 2013, Incheon Korea.

- R.Joshi, P.Amon, R.Cohen, S.Lee and M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” JCTVC-M1122, June 2013, Incheon Korea.

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).