| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0080_r2 |
| --- | --- |

| Title: | Non-RCE2: Skip of neighbouring samples filtering in intra prediction for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianqing Zhu Fujitsu R&D Center Co., Ltd. Kimihiko Kazui FUJITSU LABORATORIES LTD. | Tel: Email: | +86-21-63350606 ext. 5809 zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu R&D Center Co., Ltd. FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

In HEVC Range Extensions Draft [1], neighboring samples filtering is performed for intra prediction. In this contribution, skip of neighboring samples’ filtering is proposed to reduce complexity of encoding and decoding in case of disabling transform and quantization processes. The influence on bit-rate for the All-Intra main configuration is reported to be 0.0% for non screen content sequences, and 0.2% and 0.4% gain for screen content sequences in RGB and YUV format respectively.

# Introduction

In HEVC Range Extensions Draft [1], neighboring samples filtering is performed for intra prediction. Generally, it helps the accuracy of intra prediction. But for lossless coding, it may have trivial benefit or even negative effect on coding efficiency. So this contribution did experiment by switch off neighboring samples filtering for lossless coding. Reported result verifies the hypothesis. Meanwhile, encoding and decoding complexity is reduced without neighboring samples filtering process.

Note: Similar proposal [3] for AVC was presented at the 42th VCEG meeting.

# Experimental results

In this section, coding results for no neighbouring samples filtering in intra prediction are presented according to the test conditions used in RCE2 [2]. The anchor is HM 10.1-RExt-3.0 with the patch provided in [2]. Table 1 presents the coding results.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 5.2 | 5.2 | -0.01% |
| Class B | 2.2 | 2.2 | 0.02% |
| SC RGB 444 | 10.1 | 10.1 | -0.20% |
| SC YUV 444 | 11.1 | 11.2 | -0.36% |
| RangeExt | 2.3 | 2.3 | 0.02% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 1. Simulation results in lossless coding mode

Table 2 shows the results of the combination with JCTVC-N0079.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.7% |
| Class B | 2.2 | 2.2 | 0.2% |
| SC RGB 444 | 10.1 | 10.4 | -2.9% |
| SC YUV 444 | 11.1 | 11.6 | -2.8% |
| RangeExt | 2.3 | 2.3 | -2.1% |
| Enc Time[%] | 85.66% |  |  |
| Dec Time[%] | 98.56% |  |  |

Table 2. Simulation results of the combination with JCTVC-N0079

- Conclusion

In this contribution, skip of neighboring samples’ filtering is proposed to reduce complexity of encoding and decoding in case of disabling transform and quantization processes. The influence on bit-rate for the All-Intra main configuration is reported to be 0.0% for non screen content sequences, and 0.2% and 0.4% gain for screen content sequences in RGB and YUV format respectively.

# Suggested text on the draft standard

It is suggested that HEVC Range Extensions Draft[1] is modified as follows. The part marked in yellow is new added.

8.4.4.2.3	Filtering process of neighbouring samples

Inputs to this process are:

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size.

Outputs of this process are the filtered samples pF[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1.

The variable filterFlag is derived as follows:

–	If one or more of the following conditions are true, filterFlag is set equal to 0:

–	cu_transquant_bypass_flag is equal to 1.

–	predModeIntra is equal to INTRA_DC.

–	nTbS is equal 4.

–	Otherwise, the following applies:

….

# References

- D.Flynn, J.Sole, and T.Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 3,” JCTVC-M1005, June 2013, Incheon Korea.

- R.Joshi, P.Amon, R.Cohen, S.Lee and M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” JCTVC-M1122, June 2013, Incheon Korea.

- L.Song, Z.Luo and X.Yang, "On reference sample filtering and transform-bypass lossless intra coding", VCEG-AO11, October 2010, Guangzhou China.

# Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).