| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0081_r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0054_r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On inter-layer prediction related syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA Ohji Nakagami, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Electronics Inc. Sony Corporation |  |  |

_____________________________

# Abstract

This contribution suggests cleanups and improvement regarding the inter-layer prediction related syntax.

First, it is proposed to modify the signalling position of the syntax under P or B-slice condition in slice header to reduce redundancy. Also, it is proposed to add a condition to signal inter_layer_pred_layer_idc[ i ] for the same reason.

Second, inter_layer_ref_pic_set is proposed to arrange the syntax. The motivation is efficiently to signal the inter-layer reference information like short_term_ref_pic_set.

Finally, inter-layer sample only prediction and TMVP interaction aspect is pointed. It is proposed to check that inter_layer_sample_pred_only_flag is false before signaling collocated_ref related syntax.

In the revision1 document, an internal variable approach is alternatively added to the third aspect. InterLayerSamplePredOnlyFlag, which is derived from the number of inter reference pictures, is proposed to use for the condition to send TMVP syntax instead of checking inter_layer_sample_pred_only_flag.

# Proposal

## Point1: redundancy removal of the inter-layer prediction related syntax signalling

### slice_type check

According to the current draft, inter-layer reference is possible only in P-slice or B-slice. It is not possible in I-slice. Thus, it is proposed to move the inter-layer prediction related syntax position under P-slice or B-slice condition.

### reference layer number check

Currently, the number of reference layer is signalled in VPS and in slice header. The former is for sequence-level dependency and the latter is for picture-level. If the number of reference layers is same between NumDirectRefLayers[ nuh_layer_id ] (sequence-level) and NumActiveRefLayerPics (picture-level), it is not necessary to specify which layer to refer. Therefore, it is proposed to add a condition to avoid the redundant signalling.

### Proposed text

The proposed clean-up is shown. The change part is yellow highlighted.

(Note: blur highlighted shows the inter-later reference related syntax as the original draft)

- G.7.3.6.1	General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumDirectRefLayers[ nuh_layer_id ] != NumActiveRefLayerPics ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| num_ref_idx_active_override_flag | u(1) |
| … |  |
| } |  |

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element inter_layer_pred_layer_idc[ i ] is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.  When not present, the value of inter_layer_pred_layer_idc[ i ] is inferred to be equal to i.

## Point2: introduction of inter-layer reference picture set for more efficient inter-layer information signaling

### Motivation

For further cleanup, inter_layer_ref_pic_set is newly introduced. It arranges the inter-layer reference related syntax. Like the short_term_ref_pic_set, it is signalled in SPS and/or slice header. When an inter_layer_ref_pic_set is signalled in the active SPS, the following pictures can refer the inter_layer_ref_pic_set by signalling the index in their slice header. It reduces the redundancy when the inter-layer reference information is not negligible (e.g. multi-view case in MV/3D-HEVC). It is also allowed to signal new parameter set in each slice header for the flexibility.

### Proposed text

The proposed text is shown. The aspects regarding point1 are included. The change part is yellow highlighted, and the detailed change is emphasized in red.

G. 7.3.2.2 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 1 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| num_inter_layer_ref_pic_sets | ue(v) |
| for( i = 0; i < num_inter_layer_ref_pic_sets; i++) |  |
| inter_layer_ref_pic_set( i ) |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

- G. 7.3.x.x Inter layer reference picture set syntax

| inter_layer_ref_pic_set( stRpsIdx ) { | Descriptor |
| --- | --- |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i <= num_inter_layer_ref_pics_minus1; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |

G. 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| inter_layer_ref_pic_set_sps_flag | u(1) |
| if( ! inter_layer_ref_pic_set_sps_flag ) |  |
| inter_layer_ref_pic_set( num_inter_layer_ref_pic_sets ) |  |
| else if(num_inter_layer_ref_pic_sets > 1 ) |  |
| inter_layer_ref_pic_set_idx | u(v) |
| } |  |
| if( NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| } |  |
| … |  |
| } |  |

- G. 7.4.3.2.1 Sequence parameter set RBSP semantics

num_inter_layer_ref_pic_sets specifies the number of inter_layer_ref_pic_set( ) syntax structures included in the SPS. The value of num_inter_layer_ref_pic_sets shall be in the range of 0 to 64, inclusive.

NOTE 1 – A decoder should allocate memory for a total number of num_inter_layer_ref_pic_sets + 1 inter_layer_ref_pic_set( ) syntax structures since there may be an inter_layer_ref_pic_set( ) syntax structure directly signalled in the slice headers of a current picture. An inter_layer_ref_pic_set( ) syntax structure directly signalled in the slice headers of a current picture has an index equal to num_inter_layer_ref_pic_sets.

G. 7.4.x.x.x Inter layer reference picture set syntax

An inter_layer_ref_pic_set( stRpsIdx ) syntax structure may be present in an SPS or in a slice header. Depending on whether the syntax structure is included in a slice header or an SPS, the following applies:

–	If present in a slice header, the inter_layer_ref_pic_set( stRpsIdx ) syntax structure specifies the inter-layer RPS of the current picture (the picture containing the slice), and the following applies:

–	The content of the inter_layer_ref_pic_set( stRpsIdx ) syntax structure shall be the same in all slice headers of the current picture.

–	The value of stRpsIdx shall be equal to the syntax element num_ inter_layer_ref_pic_sets in the active SPS.

–	The inter-layer RPS of the current picture is also referred to as the num_inter_layer_ref_pic_sets-th candidate inter-layer RPS in the semantics specified in the remainder of this subclause.

–	Otherwise (present in an SPS), the inter_layer_ref_pic_set( stRpsIdx ) syntax structure specifies a candidate inter_layer RPS, and the term "the current picture" in the semantics specified in the remainder of this subclause refers to each picture that has inter_layer_ref_pic_set_idx equal to stRpsIdx in a CVS that has the SPS as the active SPS.

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictures that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive. When not present, the value of num_inter_layer_ref_pics_minus1 is inferred to be equal to 0.

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0  | |  !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

All slices of a coded picture shall have the same value of NumActiveRefLayerPics.

NOTE – If NumActiveRefLayerPics is equal to NumDirectRefLayers[ nuh_layer_id ] , it is not necessary to send inter_layer_ref_pic_set ( stRpsIdx ) syntax structure.

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element inter_layer_pred_layer_idc[ i ] is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.  When not present, the value of inter_layer_pred_layer_idc[ i ] is inferred to be equal to i.

When i is greater than 0, inter_layer_pred_layer_idc[ i ] shall be greater than inter_layer_pred_layer_idc[ i − 1 ].

The variables RefPicLayerId[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, NumActiveMotionPredRefLayers, and ActiveMotionPredRefLayerId[ j ] for each value of j in the range of 0 to NumActiveMotionPredRefLayers  − 1, inclusive, are derived as follows:

for( i = 0, j = 0; i < NumActiveRefLayerPics; i++)
	RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]
	if( MotionPredEnabledFlag[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ] )
		ActiveMotionPredRefLayerId[ j++ ] = RefLayerId[ nuh_layer_id ][ inter_layer_pred_layer_idc[ i ] ]
}
NumActiveMotionPredRefLayers = j

All slices of a picture shall have the same value of inter_layer_pred_layer_idc[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, either of the following two conditions shall be true:

–	The value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] is greater than TemporalId.

–	The values of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] and TemporalId are both equal to 0 and the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is an IRAP picture.

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, the value of SamplePredEnabledFlag[ nuh_layer_id ] [ RefPicLayerId[ i ] ] or MotionPredEnabledFlag[ nuh_layer_id ] [ RefPicLayerId[ i ] ] shall be equal to 1.

- G. 7.4.7.1 Slice segment header semantics

inter_layer_ref_pic_set_sps_flag equal to 1 specifies that the inter-layer RPS of the current picture is derived based on one of the inter_layer_ref_pic_set( ) syntax structures in the active SPS that is identified by the syntax element inter_layer_ref_pic_set_idx in the slice header. inter_layer_ref_pic_set_sps_flag equal to 0 specifies that the inter-layer RPS of the current picture is derived based on the inter_layer_ref_pic_set( ) syntax structure that is directly included in the slice headers of the current picture. When num_inter_layer_ref_pic_sets is equal to 0, the value of inter_layer_ref_pic_set_sps_flag shall be equal to 0.

inter_layer_ref_pic_set_idx specifies the index, into the list of the inter_layer_ref_pic_set( ) syntax structures included in the active SPS, of the inter_layer_ref_pic_set( ) syntax structure that is used for derivation of the inter-layer RPS of the current picture. The syntax element inter_layer_ref_pic_set_idx is represented by Ceil( Log2( num_inter_layer_ref_pic_sets ) ) bits. When not present, the value of inter_layer_ref_pic_set_idx is inferred to be equal to 0. The value of inter_layer_ref_pic_set_idx shall be in the range of 0 to num_inter_layer_ref_pic_sets − 1, inclusive.

The variable CurrRpsIdx is derived as follows:

–	If inter_layer_ref_pic_set_sps_flag is equal to 1, CurrRpsIdx is set equal to inter_layer_ref_pic_set_idx.

–	Otherwise, CurrRpsIdx is set equal to num_inter_layer_ref_pic_sets.

## Point3: inter-layer sample only prediction and TMVP interaction

### Motivation

According to the current draft, when inter_layer_sample_pred_only_flag is TRUE, no inter prediction is used. CUs can either be Intra mode or Inter mode with zero MV, since inter-layer sample prediction is done through reference list consisting of only up-sampled referenced layer picture. ”zero MV” is agreed in current reference software for inter-layer sample prediction. In MV coding of HEVC, TMVP is placed after spatial neighboring MVs in both merge list and AMVP list. When inter_layer_sample_pred_only_flag is TRUE, all available spatial neighboring MVs are zeros but TMVP (up-scaled from reference layer) may have non-zero MVs as shown in Figure 1. In current draft, a CU (with zero MV) could select TMVP as the predicator at the cost of larger index number and possible nonzero MVD. We believe that for this CU (with zero MV), selecting spatial candidates with smaller index and zero MVD will be more efficient and always doable in coding MVs for this scenario, because all available spatial neighboring MVs are zeros. So TMVP is not necessarily helpful in this scenario. In addition, TMVP in this scenario will need MV up-scaling from reference layer at the price of computational complexity and memory access.

For the sake of simple processing and efficiency in MV coding, it is proposed to skip TMVP related syntax and related processing when inter_layer_sample_pred_only_flag is TRUE.

Figure1: inter-layer sample only prediction and TMVP interaction

### Proposed text

The proposed clean-up applies to Slice segment header. It can be combined with either Section 1.1 or Section1.2. The change part is yellow highlighted.

- G.7.3.6.1	General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ....... |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ....... |  |
| if( slice_temporal_mvp_enabled_flag && ! inter_layer_sample_pred_only_flag) { |  |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 ) |  |
| alt_collocated_indication_flag | u(1) |
| if( alt_collocated_indication_flag ) |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| ....... |  |
| } |  |

### Proposed text with an intermediate variable InterLayerSamplePredOnlyFlag

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ....... |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| ....... |  |
| if( slice_temporal_mvp_enabled_flag && ! InterLayerSamplePredOnlyFlag) { |  |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 ) |  |
| alt_collocated_indication_flag | u(1) |
| if( alt_collocated_indication_flag ) |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| ....... |  |
| } |  |

An internal variable InterLayerSamplePredOnlyFlag is derived as follows.

If   NumPocStCurrBefore is equal to 0, NumPocStCurrAfter is equal to 0 and NumPocLtCurr is equal to 0, InterLayerSamplePredOnlyFlag is set to 1; Otherwise, InterLayerSamplePredOnlyFlag is set to 0.

# Patent rights declaration(s)

Sony Corporation and Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).