| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0082_r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0055_r1 |
| --- | --- |

| Title: | On initialization process of reference picture lists for HEVC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution is a follow-up of JCTVC-M0081/JCT3V-D0058.

It is proposed that inter-layer reference picture (ILRP) is inserted between RefPicSetStCurrBefore and RefPicSetStCurrAfter in creating the temporal list. The motivation is to reduce reference picture list modification (RPLM) signalling for common coding structures.

The proposal is tested under the recommended condition described in JCTVC-M0461. It is reported that the RPLM bit for SHVC(RA), 3D(PIP) and 3D(IBP) are reduced to 51.1, 60.7 and 86.5 [%], respectively.

It is recommended to modify the list0 initialization process as proposed.

Thanks to the cross-checker (JCTVC-N0361 and JCT3V-E0293), the number of bit for LD-B case is corrected in Table 3 and 4 with the change log. The proportion is kept since the proposal doesn’t affect the process for LD-B case.

# Introduction

In the current MV-HEVC draft [7] and SHVC working draft [6], initialization process for reference picture lists is specified in G.8.3.4. An inter-view reference picture is always appended at the end of temporal reference picture list (RefPicListTempX).

As discussed in [1], RPLM is necessary even for common coding structures in the enhancement layer.

In the proposal, an inter-layer reference picture is inserted between RefPicSetStCurrBefore and RefPicSetStCurrAfter in creating RefPicListTemp0 as the following order.

RefPicSetStCurrBefore, RefPicSetIvCurr, RefPicSetStCurrAfter and RefPicSetLtCurr.

In order to confirm the impact, the number of RPL-related bit is counted by RPL_COUNTER.patch which was distributed in JCT-VC/3V reflector. In Table 1, the syntax element counted by the patch is listed. Since the motivation of this contribution is to reduce the number of RPLM bits, RPLM-related syntax is emphasized in Bold.

Table 1 Syntax element that RPL_COUNTER.patch count

| Signaling position | Syntax element name |
| --- | --- |
| VPS | (no syntax element) |
| SPS | (no syntax element) |
| PPS | num_ref_idx_l0_default_active_minus1 num_ref_idx_l1_default_active_minus1 lists_modification_present_flag |
| Slice segment header ref_pic_lists_modification( ) | num_ref_idx_active_override_flag num_ref_idx_l0_active_minus1 num_ref_idx_l1_active_minus1 ref_pic_list_modification_flag_l0 list_entry_l0[ i ] ref_pic_list_modification_flag_l1 list_entry_l1[ i ] |

Tabe2 shows a bit-count result as an example of the modified decoder. The input bitstream is one of CTC generated by the original SHM-2.0 encoder. The condition is spatial x2, RA and Kimono (240) frames. The number of RPL-related bit is reported for each layer and syntax position. Also, “total” is reported as a sum of all layers.

As can be seen, the number of RPL-related bit for Layer1 (=7+5442) is much bigger than Layer0 (=7+445) due to the management of inter-layer reference picture.

In Table 2, the number of RPLM bit is separately shown in Bold. It is a part of the RPL-related bit in slice header. The RPLM bit is approximately 70 % (=3832/5792*100) of the RPL-related bit in this case.

Table 2 Example result of RPL_COUNTER.patch
(from SHVC CTC, Spatial x2, RA, 240 frames)

| Layer# | Position | Bits |
| --- | --- | --- |
| Layer0 | RPL-related bits in VPS | 0 |
|  | RPL-related bits in SPS | 0 |
|  | RPL-related bits in PPS | 7 |
|  | RPL-related bits in slice header | 445 |
|  | (RPLM bits in slice header) | (0) |
| Layer1 | RPL-related bits in VPS | 0 |
|  | RPL-related bits in SPS | 0 |
|  | RPL-related bits in PPS | 7 |
|  | RPL-related bits in slice header | 5442 |
|  | (RPLM bits in slice heade)r | (3832) |
| Total | RPL-related bits in VPS | 0 |
|  | RPL-related bits in SPS | 0 |
|  | RPL-related bits in PPS | 14 |
|  | RPL-related bits in slice header | 5792 |
|  | (RPLM bits in slice header) | (3832) |

# Experimental results

## Basic test condition

The condition described in JCTVC-M0461 [3] is studied.

### SHVC

SHM-2.0 is used. RA, LD-P and LD-B configurations are tested. Scalability type is 2X spatial. Sequence Kimono, 240 frames, is used for bit count. Base layer QP 26, enhancement layer QP 26.

### MV-HEVC / 3D-HEVC

HTM-6.2 is used. Undo Dancer, 250 frames, is used for bit count, coded at texture QP 25, depth QP 34.

Also, HTM-DEV-0.1 and HTM-DEV-1.0 are used to clarify the difference regarding RPL-related syntax from HTM-6.2.

## Results

The number of RPL-related bits is summarized in Table 3. The total number, which is sum of all layers, is shown.

Table 4 shows the number of RPLM bits, which is a part of the bit in Table 3. The number is consistent with the theoretical number reported in [1]. The number of RPLM bit by HTM-DEV-0.1/1.0 is consistent with the one by HTM-6.2.

Table 3 the number of bits counted by RPL_COUNTER.patch (all layers)

| Scalability | Software ver. | Coding structure | Current [bit] | Proposed [bit] | Proportion [%] |
| --- | --- | --- | --- | --- | --- |
| SHVC Spatialx2 | SHM-2.0 | RA* | 5,792 | 3,920 | 67.7 |
| SHVC Spatialx2 | SHM-2.0 | LD-P | 1,912 | 1,912 | 100 |
| SHVC Spatialx2 | SHM-2.0 | LD-B | 3,344 | 3,344 | 100 |
| MV/3D-HEVC | HTM-6.2 | PIP (3t) | 10,029 | 8,091 | 80.7 |
| MV/3D-HEVC | HTM-6.2 | PIP (3t+3d) | 20,198 | 16,322 | 80.8 |
| MV/3D-HEVC | HTM-6.2 | IBP (3t) | 12,304 | 11,335 | 92.1 |
| MV/3D-HEVC | HTM-6.2 | IBP (3t+3d) | 24,748 | 22,810 | 92.2 |
| MV/3D-HEVC | HTM-DEV-0.1 | PIP (3t) | 6,159 | - | - |
| MV/3D-HEVC | HTM-DEV-1.0 | PIP (3t+3d) | 12,185 | - | - |

Table 4 the number of RPLM bits (all layers)

| Scalability | Software | Coding structure | Current [bit] | Proposed [bit] | Proportion [%] |
| --- | --- | --- | --- | --- | --- |
| SHVC Spatialx2 | SHM-2.0 | RA* | 3,832 | 1,960 | 51.1 |
| SHVC Spatialx2 | SHM-2.0 | LD-P | 239 | 239 | 100 |
| SHVC Spatialx2 | SHM-2.0 | LD-B | 478 | 478 | 100 |
| MV/3D-HEVC | HTM-6.2 | PIP (3t) | 4,930 | 2,992 | 60.7 |
| MV/3D-HEVC | HTM-6.2 | PIP (3t+3d) | 9,860 | 5,984 | 60.7 |
| MV/3D-HEVC | HTM-6.2 | IBP (3t) | 7,175 | 6,206 | 86.5 |
| MV/3D-HEVC | HTM-6.2 | IBP (3t+3d) | 14,350 | 12,412 | 86.5 |
| MV/3D-HEVC | HTM-DEV-0.1 | PIP (3t) | 4,930 | - | - |
| MV/3D-HEVC | HTM-DEV-1.0 | PIP (3t+3d) | 9,700 | - | - |

* The coding structure is slightly different from CTC. The exact coding structure and its result are shown in the Appendix. It is reported that the bitrate difference is 0.0[%].

# Conclusion

As the results show, the proposal reduces the number of RPL-related bits to 67.7, 80.7 and 92.1 [%] for SHVC, 3D(PIP), 3D(IBP) case, respectively. Focusing on RPLM bits, the number is 51.1, 60.7 and 86.5 [%]. It is asserted that it is especially effective for multi-slice, or long sequence. Therefore, it is recommended to modify the process as proposed.

# Proposed text

Below is the proposed text on top of JCTVC-M1008_v3. The change part is yellow highlighted.

- G.8.3.4 Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause 8.5.3.3.2. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture lists RefPicList0 and, for B slices, RefPicList1 are derived as follows:

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 ) {
	if( InterRefEnabledInRPLFlag ) {
		for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
	}
	for( i = 0; i < NumActiveRefLayerPics; rIdx++, i++ )
		RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ]
	if( InterRefEnabledInRPLFlag ) {
		for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(Error: Reference source not found-1)
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	}
}

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++)		(Error: Reference source not found-2)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 ) {
	if( InterRefEnabledInRPLFlag ) {
		for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(Error: Reference source not found-3)
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	}
	for( i = 0; i< NumActiveRefLayerPics; rIdx++, i++ )
		RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(Error: Reference source not found-4)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

# Appendix

## Coding performance for SHVC RA condition

Table 5 shows the differences for the list0 process between the current and the proposed in red. The bitstream for Kimono, Spatial 2x and RA condition is attached in the contribution as an example.

### UsedByCurr signaling in layer 1 SPS

In the proposal, RPLM signaling is not necessary when UsedByCurr flag is correctly set in RPS. To prove that, the bitstream is generated by setting UsedByCurr flag equal to 0 for the unnecessary temporal reference picture at POC 6 and 7 in layer1 SPS.

For that purpose, SHM-2.0 encoder is modified to generate two SPSs for base and enhancement layer, respectively. The enhancement layer SPS has the modified RPS parameters, where the diferrence is UsedByCurrflag value for POC 6 and 7. The base layer SPS is not changed.

### Coding structure in POC 4, 2 and 1

As described in JCTVC-M0081/JCT3V-D0058, the coding structure is different at POC 1, 2 and 4 of List0 for RA case in the proposal. The coding result is shown in Table 6. It is reported that the impact is negligible. The complete result is attached in the contribution by the excel file.

Table 5 the order of reference pictures in List0 under SHM RA test condition

|  |  | Current |  |  | Proposed |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| POC | #ref_pics_active | L0Temp | RPLM | L0Final | L0Temp | RPLM | L0Final |
| 8 | 5 | -8 -10 -12 -16 IL | - | -8 -10 -12 -16 IL | -8 -10 -12 -16 IL | - | -8 -10 -12 -16 IL |
| 4 | 3 | -4 (-6) 4 IL | ○ | -4 (-6 or 4) IL | -4 (-6) IL 4 | - | -4 -6 IL or -4 IL 4 |
| 2 | 3 | -2 (-4) 2 6 IL | ○ | -2 (-4 or 2) IL | -2 (–4) IL 2 6 | - | -2 -4 IL or -2 IL 2 |
| 1 | 3 | -1 1 3 7 IL | ○ | -1 1 IL | -1 IL 1 3 7 | - | -1 IL 1 |
| 3 | 3 | -1 -3 1 5 IL | ○ | -1 -3 IL | -1 -3 IL 1 5 | - | -1 -3 IL |
| 6 | 3 | -2 -4 -6 2 IL | ○ | -2 -4 IL | -2 -4 -6 IL 2 | △ | -2 -4 IL |
| 5 | 3 | -1 -5 1 3 IL | ○ | -1 -5 IL | -1 -5 IL 1 3 | - | -1 -5 IL |
| 7 | 3 | -1 -3 -7 1 IL | ○ | -1 -3 IL | -1 -3 -7 IL 1 | △ | -1 -3 IL |

Table 6 Coding result of the proposed reference structure for RA condition

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |  |  |  | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall (Test vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.2% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 94.0% |  |  | 94.3% |  |  | 92.2% |  |  |
| Dec Time[%] | 99.6% |  |  | 91.8% |  |  | 85.4% |  |  |
| Enc Mem[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

## Study on IBP-case in MV/3D-HEVC

Table 7 the order of reference pictures in List0 with JCT-3V IBP setting (view2 as B-pic)

|  |  | Current |  |  | Proposed |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| POC | #ref_pics_active | L0Temp | RPLM | L0Final | L0Temp | RPLM | L0Final |
| 8 | 4 | -8 -10 -12 -16 IL1 IL0 | ○ | -8 IL0 -10 -12 | -8 -10 -12 -16 IL1 IL0 | ○ | -8 IL0 -10 -12 |
| 4 | 4 | -4 (-6) 4 IL1 IL0 | ○ | -4 IL0 -6 | -4 (-6) IL1 IL0 4 | ○ | -4 IL0 (-6 or 4) |
| 2 | 3 | -2 (-4) 2 6 IL1 IL0 | ○ | -2 IL0 2 | -2 (–4) IL1 IL0 2 6 | ○ | -2 IL0 (-4 or 2) |
| 1 | 3 | -1 1 3 7 IL1 IL0 | ○ | -1 IL0 1 | -1 IL1 IL0 1 3 7 | ○ | -1 IL0 1 |
| 3 | 3 | -1 -3 1 5 IL1 IL0 | ○ | -1 -3 IL0 | -1 -3 IL1 IL0 1 5 | ○ | -1 -3 IL0 |
| 6 | 3 | -2 -4 -6 2 IL1 IL0 | ○ | -2 -4 IL0 | -2 -4 -6 IL1 IL0 2 | ○ | -2 -4 IL0 |
| 5 | 3 | -1 -5 1 3 IL1 IL0 | ○ | -1 -5 IL0 | -1 -5 IL1 IL0 1 3 | ○ | -1 -5 IL0 |
| 7 | 3 | -1 -3 -7 1 IL1 IL0 | ○ | -1 -3 IL0 | -1 -3 -7 IL1 IL0 1 | ○ | -1 -3 IL0 |

Table 8 the order of reference pictures in List1 under JCT-3V IBP setting (view2 as B-pic)

|  |  | Current and Proposed (option1) |  |  | Proposed (option2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| POC | #ref_pics_active | L1Temp | RPLM | L1Final | L1Temp | RPLM | L1Final |
| 8 | 4 | -8 -10 -12 -16 IL0 IL1 | ○ | -8 IL0 IL1 or -8 IL1 -10 -12 | IL0 IL1 -8 -10 -12 -16 | ○ | -8 IL0 IL1 or -8 IL1 -10 -12 |
| 4 | 4 | 4 -4 (-6) IL0 IL1 | ○ | 4 IL1 -4 | 4 IL0 IL1 -4 (-6) | ○ | 4 IL1 -4 |
| 2 | 3 | 2 6 -2 (-4) IL0 IL1 | ○ | 2 IL1 6 | 2 6 IL0 IL1 -2 (-4) | ○ | 2 IL1 6 |
| 1 | 3 | 1 3 7 -1 IL0 IL1 | ○ | 1 IL1 3 | 1 3 7 IL0 IL1 -1 | ○ | 1 IL1 3 |
| 3 | 3 | 1 5 -1 -3 IL0 IL1 | ○ | 1 5 IL1 | 1 5 IL0 IL1 -1 -3 | ○ | 1 5 IL1 |
| 6 | 3 | 2 -2 -4 -6 IL0 IL1 | ○ | 2 -2 IL1 | 2 IL0 IL1 -2 -4 -6 | ○ | 2 -2 IL1 |
| 5 | 3 | 1 3 -1 -5 IL0 IL1 | ○ | 1 3 IL1 | 1 3 IL0 IL1 -1 -5 | ○ | 1 3 IL1 |
| 7 | 3 | 1 -1 -3 -7 IL0 IL1 | ○ | 1 -1 IL1 | 1 IL0 IL1 -1 -3 -7 | ○ | 1 -1 IL1 |

# Reference

- JCTVC-M0081/JCT3V-D0057: On initialization process of reference picture lists for HEVC extensions, Apr. 2013

- JCT3V-C0079: On initialization process for reference picture lists, Jan. 2013

- JCTVC-M0461: Report of BoG on guidelines for evaluation of reference picture list modification proposals, Apr. 2013

- JCTVC-M1009: Common SHM test conditions and software reference configurations, Apr. 2013

- JCT3V-D1100: Common Test Conditions of 3DV Core Experiments, Apr. 2013

- JCTVC-M1008: SHVC Working Draft 2, Apr. 2013

- JCT3V-D1004: MV-HEVC Draft Text 4

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).